3GPP TSG-RAN WG4#109	R4-2320247

Chicago, US, October 9th – 13th, 2023

Agenda item:	7.1.1.1

Source: 	Qualcomm France

Title: 	MSD for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A

Document for:	Approval

# Introduction

Analysis and proposals on CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A MSD are provided in this contribution.

# Considerations

The following was agreed in WF [1]:

<Way forward>:

The following test points are used for evaluation in next meeting:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | TBD | FDD | IMD9 |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | TBD | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | TBD | FDD | IMD7 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | TBD | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | TBD | FDD | IMD11 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | TBD | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

NOTE: Only one MSD testpoint for each NR CA combination is specified based on the evaluation

We used the conventional 3GPP assumptions for the TX for the PA calibration.

- 20 MHz 100RB DFT-s-OFDM

- 30dBc ACLR

- 4dB Post-PA loss

- Image -28dB

- LO -28dBc

- CIM3 -60dBc

- CIM5 -70dBc

The parameters and MSD results are shown below.

Table below includes all MSD test points, as well as MSD evaluations for n5 PCC/SCC.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 28.1 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | 21.7 |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 8.3 | FDD | IMD9 |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 28.9 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | 15.4 |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 15.3 | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 28.1 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | 21.7 |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 7.9 | FDD | IMD7 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 27.8 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | 22.6 |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 22.6 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 28.1 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | 21.7 |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 6.7 | FDD | IMD11 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | 29.1 | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | 13.6 |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 10.4 | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

Based on table above, the MSD is clearly largest with direct hit for these cases. The MSD for n5 PCC/SCC is a bit smaller than specified for 10+10MHz CA_n5B.

Proposal 1: Use the following lowest order full hit MSD text points for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B_n29A:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 15.3 | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 22.6 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 10.4 | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

Potential testing challenge due to heavy desense observed in n5 PCC/SCC was discussed in RAN4#108bis. After further considerations, it should not be a big issue eventually. Notes X and Y should be clear enough so that RAN5 is able to address the aspect appropriately in RAN5.

Proposal 2: No need to send an LS to RAN5 due to n5 PCC/SCC MSD

# Conclusion

MSD analysis was conducted for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A

Proposal 1: Use the following lowest order full hit MSD text points for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B_n29A:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 15.3 | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 22.6 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 10.4 | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

Proposal 2: No need to send an LS to RAN5 due to n5 PCC/SCC MSD

# Reference

[1] R4-2317656, WF on cross-band MSD test point for FDD bands with intra-band contiguous UL CA