3GPP TSG-RAN WG4 Meeting #109 	R4-2320283

Chicago, USA, November 13-17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures

Agenda item:	5.2.3

Document for:	Approval

Introduction

RAN4 has been discussing the requirements related to SCell activation using tci-ActivatedConfig for a couple of meetings.

The issue at hand is that tci-ActivatedConfig has not been accounted fully into the RAN4 requirements. Exactly how to capture such requirements and when the tci-ActivatedConfig is applicable was left open.

In this paper we discuss the applicability issue of tci-ActivatedConfig and in our CR [1] we have a proposal how to capture the requirements.

This paper is an updated version of [3] submitted in RAN4#108 meeting. An alternative option discussed in that meeting has been included in this contribution.

Discussion

tci-ActivatedConfig

tci-ActivatedConfig is introduced in Rel-17 by RAN2. It may be configured together with ServingCellConfig [2]:

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

The detailed parameter description [2]:

where

Where:

More specifically, looking at the parts that would be of interest for defining the UE requirements:

tci-ActivatedConfig

- If configured for an SCell, or

- if configured for the PSCell when the SCG is being activated upon the reception of the containing message,

  - the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell.

- If configured for the PSCell when the SCG is indicated as deactivated in the containing message:

- the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD.

When this field is absent for the PSCell and the SCG is being deactivated:

- the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tci-ActivatedConfig applicability

As we see from the RAN2 description, the tci-ActvatedConfig  can be configured for:

- an SCell, or

- a PSCell when the SCG is activated upon configuration.

From the RAN2 description it is apparent that the tci-ActvatedConfig can be configured for an SCell unconditionally while for a PSCell tci-ActvatedConfig can be configured only when the PSCell is activated upon configuration. Based on the current RAN2 specification we conclude following:

tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig ActvatedConfig can only be configured for a PSCell which is activated when configured.

Hence, from the current RAN2 wording, tci-ActivatedConfig can be configured for an SCell no matter if the SCell is activated not activated when configured. The tci-ActivatedConfig can be configured for a deactivated SCell as well as for an SCell being configured as activated upon configuration (direct activated).

tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

In [4] we have provided a CR capturing the UE requirements accounting tci-ActivatedConfig.

However, in RAN4#108 meeting views were diverse on how the RAN2 specification was to be interpreted and the proposed changes in the CR was not agreed. One proposal was to ask RAN2 to clarify the relevant part of the specification quoted above and clarify RAN4 specification accordingly – if needed.

In [5] we have provided a draft LS TP as follows:

RAN4 has been discussing the requirements related to PSCell and SCell activation when the PSCell or SCell configuration includes tci-ActivatedConfig.

RAN4 would like to ask RAN2 if the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration (hence, the SCell is configured in deactivated state)?

Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

If proposal 1 is not agreeable, send LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

Conclusion

In this paper we discussed the applicability issue of tci-ActivatdConfig and observe:

- tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig can only be configured for a PSCell which is activated when configured.

tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

We propose either of:

- Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

If proposal 1 is not agreeable, send LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

In CR [4] we have a proposal how to capture the requirements and we have in [5] provided a Draft LS.

- References

- R4-2313357, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- 38.331.

- R4-2313356, LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures, Nokia, Nokia Shanghai Bell.

- R4-2320284, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- R4-2320286, Draft LS on TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell