3GPP TSG-RAN WG4 Meeting # 109	R4-2320295

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On network A requirements

Agenda item:	5.25.2.3

Document for:	Discussion

Introduction

In the last RAN4#108bis meeting RAN4 made progress regarding the Network-A requirements when UE is configured with MUSIM gaps.

However, two aspects were left open [10] which related to whether to discuss MUSIM gaps and NTN, and whether there would be any impact from having MUSIM gaps configured on the Measurement for Propagation Delay Compensation.

We shortly discuss these aspects and provide TP and CR for the second issue.

Discussion

On network A requirements

MUSIM gap impact on NTN

Status of the outcome from RAN4#108bis is:

- Proposals

  - P1: Descope MUSIM gaps impact on NTN requirements in R18 (MTK)

In short, we believe RAN4 has enough work and in order to finalize this WI in a timely manner accounting also the quality of the requirements, RAN4 should not consider MUSIM gaps impact on NTN requirements in R18.

Support option P1. RAN4 does not consider MUSIM gaps impact on NTN requirements in R18.

MUSIM gap impact on Measurement requirement for Propagation Delay Compensation

Status of the outcome from RAN4#108bis is:

- Proposals

  - P1: While UE is performing measurements for propagation delay compensation, the UE will drop any overlapping MUSIM gaps (Nokia)

Measurements for propagation delay compensation are based on a request and a limited number of samples, we suggest that for propagation delay compensation measurements, these measurements will not be impacted by any MUSIM gaps. UE is already allowed scaling due to non-MUSIM gaps and this would be unchanged.

Hence, we suggest that while UE is performing measurements for propagation delay compensation, the UE will drop any overlapping MUSIM gaps.

While UE is performing measurements for propagation delay compensation, the UE will drop any overlapping MUSIM gaps.

We suggest capturing this by introducing a new section 9.12.8 - for example as follows:

### 9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps

For a UE configured with MUSIM gaps, the measurement requirement defined for measurement for Propagation Delay Compensation, as defined in this section 9.12, applies.

We have provided CR in [8]

Conclusion

In this paper, we address two aspects left open concerning whether to discuss MUSIM gaps and NTN and whether there would be any impact from having MUSIM gaps configured on the Measurement for Propagation Delay Compensation. We propose:

- Support option P1. RAN4 does not consider MUSIM gaps impact on NTN requirements in R18.

- While UE is performing measurements for propagation delay compensation, the UE will drop any overlapping MUSIM gaps.

A CR has been provided in [9]

- References

- R2-2309278, LS to RAN4 on MUSIM gap priorities, RAN2

- R4-2314364, WF on R18 MUSIM, vivo

- R4-2310165, WF on MUSIM RRM requirements, vivo

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo.

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo.

- R4-2306357, WF on NR Dual Tx/Rx Multi-SIM, vivo

- R4-2316674, DraftCR on Measurement for Propagation Delay Compensation, Nokia, Nokia Shanghai Bell

- R4-2320297, DraftCR on Measurement for Propagation Delay Compensation, Nokia, Nokia Shanghai Bell

- R4-2317425, WF on R18 MUSIM, vivo