3GPP TSG-RAN WG4 Meeting #109	R4-2320313

Chicago, USA, 13th November – 17th November 2023

Source:	Ericsson, Bell Mobility

Title:	TP for 38.899 adding PC2 non-contiguous UL to CA_n5-n78

Agenda item:			4.20.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.899 adding PC2 non-contiguous UL to CA_n5-n78.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n5-n78

### 5.x.1	Configurations

To enable CA_n78(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n78(2A) | n788, 9 CA_n5A-n78A8 CA_n78(2A) 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n78(2A) UL for PC2.

#### 5.x.3.1	Power class 2 case a, b

CA_n78(2A) UL for CA_n5A-n78(2A) DL is already defined for PC3. PC2 is also defined for CA_n5A-n78(2A) DL but not yet for CA_n78(2A) UL.

PC2 test points for the now added CA_n78(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n28-n78.

Table 5.x.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 RBSTART=25 | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 RBSTART=25 | 3780 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.x.3.2      Power class 2 for single uplink n78

This section is omitted since PC2 single uplink for CA_n5A-n78(2A) DL is already specified.

---End of changes---

# Reference

[1] 		R4-2311065, Revised WID for HPUE_NR_CADC_SUL_R18 RAN4#108