3GPP TSG-WG4 Meeting #109	 R4-2320326

# Chicago, USA, Nov 13 - Nov 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-1: clause 6.5J,7.1J,7.2J |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The draft R4-2317744 has been endorsed in last RAN4 meeting,it is to introduce the ATG UE RF requirement including output RF spectrum emission, receiver general and diversity; One more clarification note is added to address the concerns on SEM for low power case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the output RF spectrum emission for ATG, receiver general and diversity requirement for ATG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF requirement for ATG CPE is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5J, 7.1J, 7.2J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

6.5J	 Output RF spectrum emissions for ATG

6.5J.1	Occupied bandwidth for ATG

The requirements for occupied bandwidth in clause 6.5.1 apply.

6.5J.2	Out of band emission for ATG

6.5J.2.1	General

This clause contains requirements for out of band emissions for ATG UE, the requirement defined in general part of clause 6.5.2.1 should apply.

6.5J.2.2	Spectrum emission mask

If rated maximum output power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in  clause 6.5.2.2 apply; if[ rated maximum output power] of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (rated maximum output power minus 31) dB.

NOTE: This scaling factor is only applicable to ATG airborne UE. If the transmission power is lower than rated maximum output power at maximum modulation order, then scaling factor should comply with the actual transmission power.

6.5J.2.3	Adjacent channel leakage ratio

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5J.2.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value 30dBc.

Table 6.5J.2.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

6.5J.3	Spurious emissions for ATG

The requirements for spurious emission in general part of clause 6.5.3.0 and clause 6.5.3.1 apply.

7.1J	 General for ATG

Unless otherwise stated, the receiver characteristics are specified at the antenna connector(s) of the ATG UE with one or multiple omni-direction antenna(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array. The definition about transceiver array boundary (TAB) is specified in clause 4.3.2 of TS 38.104 [X].

For the ATG UE with multiple omni-direction antennas, the receiver RF requirements are defined on top of each antenna connector.

For the ATG UE with the phase antenna array, the receiver RF requirements are defined on top of each TAB connector.

7.2J  Diversity characteristics for ATG

The ATG UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands. ATG UE is required optionally to be equipped with four Rx antenna ports.

<End of Change>