3GPP TSG-RAN WG4 Meeting#109                              R4-2320331

Chicago, USA, Nov 13 – Nov 17, 2023

Agenda item:	8.26.3

Source: 	ZTE Corporation

Title: 	Further discussion on SAN RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In the previous meetings, we have some discussion on NTN SAN RF requirement definition and reached some progress. In this contribution, we shared some further views on remaining issues left for NTN SAN in Ka-band.

Discussion

## 2.1. OOBB requirement

In the previous RAN4 meeting, there were some discussion on OOBB requirement, however there are no much progress for it. From our understanding, OOBB requirement within FR1 should be also applicable for Ka-band SAN since this interfering signal is agonistic to the victim system. However for frequency range between 12.75GHz to 2nd harmonic of upper frequency, this should be subject to the potential interference signal from other aggressor, at least we could see the following potential aggressor signals

- Ku-band satellite UL transmission.

- Frequency and timing reference signal between satellite nodes;

- Wireless positioning between satellite.

Figure 1. OOBB requirement for NTN SAN in Ka-band.

Figure 2. The spectrum chart for 12-30GHz

Proposal 1: for frequency range between 30MHz and 12.75GHz, to reuse the FR1 OOBB requirement for Ka-band SAN;

Proposal 2: for frequency range between 12.75GHz and 2nd harmonic of upper frequency, the following service should be taken into account.

- Ku-band satellite UL transmission.

- Frequency and timing reference signal between satellite nodes;

- Wireless positioning between satellite.

## 2.2. ICS requirement

For SAN ICS requirement in Ka-band, similar as FR1 NTN and TN BS ICS requirement, we need to figure out the IoT levels in this system due to the co-channel interference from other cells or beams.

Interfering signal power level = -174dBm/Hz+10*log10(BW)+NF+ICS-G,
Where:

-	BW is interfering signal bandwidth in Hz, e.g. 33PRB for 50MHz SCS:60 kHz;

-	G is dependent on the mmWave receiver antenna gain and other factors for OTA REFSENS requirement

-	ICS is agreed as 14dBc for all BS type, the specific reasons can be found in the following Table.

Based on the following initial simulation results for IoT level in GEO, LEO1200km and LEO600KM as shown in Figure 3~5, it could be found that IoT level around 21dBc is relatively higher compared with FR1 with FRF equal to 1 which should be the worst case assumption, however if FRF is equal to 2 with dual polarization considered, then the expected IoT level would be close to 0dB.

Figure 3. IoT level for GEO scenario [FRF=1]

Figure 4. IoT level for LEO1200KM scenario [FRF=1]

Figure 5. IoT level for LEO600KM scenario [FRF=1]

Observation 1: for FRF=1 without consideration of polarization isolation, then IoT level could reach up to around 21dBc.

## 2.3. ACLR requirement for SAN

| ## Scenario | ## ACIR requirement [Case 6 for SAN ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |
| ## LEO-1200KM | ## 0 | ## 0 |
| ## LEO-600KM | ## 0 | ## 0 |

| ## Scenario | ## ACIR requirement [Case 7 for SAN ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |
| ## LEO-1200KM | ## 0 | ## 0 |
| ## LEO-600KM | ## 0 | ## 0 |

Observation 2: Regarding ACLR requirement for SAN transmitter, 0dB ACIR requirement is enough.

## 2.4. ACS requirement for SAN

| ## Scenario | ## ACIR requirement [Case 4 for SAN ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 8 | ## 14 |
| ## LEO-1200KM | ## 8 | ## 18 |
| ## LEO-600KM | ## 10 | ## 18 |

| ## Scenario | ## ACIR requirement [Case 2 for SAN ACS] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |
| ## LEO-1200KM | ## 0 | ## 0 |
| ## LEO-600KM | ## 0 | ## 0 |

Observation 3: Regarding ACS requirement for SAN receiver, 14dB ACIR requirement for GEO and 18dBc for LEO is enough.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for NTN in Ka-band.andproposals are made as following:

Proposal 1: for frequency range between 30MHz and 12.75GHz, to reuse the FR1 OOBB requirement for Ka-band SAN;

Proposal 2: for frequency range between 12.75GHz and 2nd harmonic of upper frequency, the following service should be taken into account.

- Ku-band satellite UL transmission.

- Frequency and timing reference signal between satellite nodes;

- Wireless positioning between satellite.

Observation 1: for FRF=1 without consideration of polarization isolation, then IoT level could reach up to around 21dBc.

Observation 2: Regarding ACLR requirement for SAN transmitter, 0dB ACIR requirement is enough.

Observation 3: Regarding ACS requirement for SAN receiver, 14dB ACIR requirement for GEO and 18dBc for LEO is enough.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2313996,WF for SAN RF requirements, Ericsson.

- R4-2302877, WF fro SAN RF requirement, Ericsson

- R4-2309770, WF on above 10 GHz SAN RF requirements, CATT, RAN4#107

- Home Page | National Telecommunications and Information Administration (ntia.gov)