3GPP TSG-RAN WG4 Meeting#109                              R4-2320333

Chicago, USA, Nov 13 – Nov 17, 2023

Agenda item:	8.26.5.1

Source: 	ZTE Corporation, Thales, Samsung, Ericsson, Huawei

Title: 	Joint contribution for NTN VSAT RF requirements in Ka-band

Document for:	Information

Introduction

This document is to capture the outcome of offline discussion minutes for NTN VSAT RF requirements in Ka-band at Oct 30 and Oct 31.

The participant includes the following companies: ZTE, Thales, Samsung, Ericsson, Huawei, Qualcomm.

Discussion

## 2.1. Recommended WF

- Issue 1-1: For NTN UE power class/types definition:

Recommended WF:

| UE/Power class | UE type | Type description |
| --- | --- | --- |
| [A] [fixed VSAT] | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronical steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronical steering antenna. |
| [B][Mobile VSAT] | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronical steering antenna. |
| Note: Assuming that UE has single beam towards one single satellite at a given time. |  |  |

- Issue 1-2: Mapping between NF and NTN UE power class/types

- Recommended WF:

Fixed VSAT:

Electronic steering

Class 1: 2.5 dB NF  (LEO and GEO)

Class 2: 6 dB NF  (LEO only)

Mechanical steering

Class 1: 2.5 dB NF  (LEO and GEO)

Mobile VSAT:

electronic steering

Class 1: 2.5 dB NF  (LEO and GEO)

Class 2: 6 dB NF  (LEO only)

mechanical steering

Class 1: 2.5 dB NF  (LEO and GEO)

NOTE: Mobile VSAT in the LEO is not precluded in the future release and we need to further discuss with operators.

- Issue 1-3: the necessity of introduction of NTN UE capable of both electronic steering and mechanical steering and how reflect in the above UE types.

Recommend WF from RF perspective:

For the hybrid beam steering capable NTN UE, one clarification note is added in 38.133, hybrid steering capability UE, the requirement could follow either electronic or mechanical beam steering capability.

- Issue 1-4: how to define the on-axis EIRP limit

- Recommended WF:

- maximum EIRP limit is 76.2dBm.

- The maximum TRP limit for NTN UE follow the ITU recommendation.

- Issue 1-5: transmit power assumption for each UE types.

- Recommended WF:

- for each UE type, single value for minimum EIRP limit for certain elevation angle and FFS for the exact value;

- If UE is capable of communicating with both GSO and LEO, to follow the minimum EIRP for worse case.

- Issue 1-6: freq error requirement for NTN VSAT

- Recommended WF:

- Reuse the existing FR1 NTN UE freq error requirement [+/-0.1ppm with 1ms];

- Issue 1-7: EVM requirement for NTN VSAT

Agreement:

- Reuse the existing RF requirement with Modulation limited up to 16QAM and optional to support 64QAM

- Issue 1-12 Phase continuity requirement for DMRS bundling.

- Recommended WF:

- NOT applicable for FR2 NTN VSAT

- Issue 1-13 OBW

Recommended WF:

- Option 1: to follow the existing requirement of TS 38.101-2

- Issue 2-1B FRC and FDD pattern in Annex.

- Recommend WF:

- Reuse the existing DL FRC defined in TS38.101-2 with update TDD pattern configuration for FDD.  FFS for UL FRC/ PRB configuration.

- Issue 2-2: Maximum input power for NTN VSAT

- Recommend WF:

- Depend on the simulation inputs/link budget [elevation angle as 90 degree] and only consider the receiver power without considering receiver array gain/aperture gain.

- If the VSAT is communicating with both GSO and LEO, the requirement will be decided by the worse case;

- Issue 2-5: Receiver spurious emission requirement for NTN VSAT.

- Recommend WF:

- Option 1: not needed since this is FDD band and OTA conformance testing.

## 2.2. Open for further discussions

- Issue 1-7A: the necessity of introduction of PI/2 BPSK.

- To introduce the PI/2 BPSK for NTN UE.

- Optional for NTN UE;

- Issue 1-8: the necessity of carrier leakage requirement for NTN VSAT

- FFS

- Issue 1-9 the necessity of in-band emission for NTN VSAT

- FFS

- Issue 1-10 the necessity of EVM equalizer spectrum flatness requirement for NTN VSAT

- FFS

- Issue 1-11 the necessity of EVM spectral flatness for Pi/2 BPSK modulation for NTN VSAT

- FFS

- Issue 1-14 SEM

Recommended WF:

- Option 1: To follow ITU-R SM.1541-6, Annex 5 OoB domain emission limits for space services (earth and space stations)

- Option 2: other options are not precluded.

- Issue 1-14 ACLR requirement

- Option 1: this depends on the coexistence study.

- Issue 1-15 Transmitter spurious emission requirement

- FFS

- Issue 2-1 Power level for Min EIS

- Option 1:  to use the following formula with Rx Gain as [TBD]

EIS=-174+10*log10(BW)+NF+SINR+IM-G

- Where G is including antenna gain and [polarization gain]

- For Rx gain without polarization gain:

- to use the assumption in the coexistence study [39.7dBi] for NF=2.5dBi

- [17dBi] for VSAT connecting with LEO only with NF 6dB;

- Issue 2-1A the necessity of introduction of 3dB polarization gain

- Option 1:  Yes

- Option 2:  No

- Option 3:  other options.

- Issue 2-3: ACS requirement for NTN VSAT

- Option 1: depend on the simulation inputs

- Option 2: other options are not precluded.

- Issue 2-4: in-band blocking for NTN VSAT

- Option 1: depend on the simulation inputs/link budget and less than maximum input power;

- Option 2: other options are not precluded.