3GPP TSG-RAN WG4 Meeting#109                              R4-2320344

Chicago, US, November 13 – 17,2023

Agenda item:	8.28.2.2

Source: 	ZTE Corporation

Title: 	Discussion on RF requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In the previous RAN4 meetings, we reached good consensus on RF requirement for NCR-MT, however there are still some open issues left for further discussions. In this contribution, we  would like to share some further views on it.

Discussion

## 2.1. RF requirements for simultaneous reception between NCR-Fwd and NCR-MT

In the previous RAN4 meeting, we reached the consensus for the emission requirement definition in case of simultaneous transmission, however for the simultaneous reception between NCR-MT and NCR-Fwd, it is still open for further discussion. During the last RAN4 meeting, it was proposed to have test individual reception and not simultaneous and with separate requirement for NCR-MT and NCR-Fwd. However the exact requirement for the simultaneous reception between NCR-MT and NCR-Fwd is still left for further discussions.

Simultaneous reception - Propose to test individual reception and not simultaneous - [Separate requirement for NCR-MT and NCR-Fwd] - Consider the simultaneous reception with input intermodulation requirement only.

As we discussed in the previous RAN4 meetings, RF requirement between NCR-MT and NCR-Fwd don’t have much commonality and it is difficult to test them together or merge them into single requirement. The only commonality between two sets of RF requirement from our understanding might be input intermodulation requirement.

Figure 1. RF requirement for NCR-MT and NCR-Fwd.

For the simultaneous reception of NCR-MT and NCR-Fwd in NCR input intermodulation requirement perspective, since intermodulation requirement might fall into the wanted signal of NCR-MT especially for FR1 scenario. In addition, it seems that CW signal power level for repeater input intermodulation requirement is also a bit higher than -46dBm CW defined for FR1 UE receiver intermodulation requirement. From these perspective, certain reference degradation on NCR-MT wanted signal should be considered instead of without any performance degradation.

For FR2, there are no transmitter intermodulation and receiver intermodulation requirement defined yet, however considering the potential intermodulation signal from repeater fwd link fall into NCR-MT carrier bandwidth similar as FR1 scenario, we also propose to have certain REFSENS degradation to be allowed.

Proposal 1:For the simultaneous reception of NCR-MT and NCR-Fwd in NCR input intermodulation requirement, 6dB REFSENS degradation could be allowed once the intermodulation signal fall into NCR-MT bandwidth;

Figure 7. the illustration of Input IMD requirement with NCR-MT Rx requirements

## 2.2. OTA requirement for FR1 NCR-MT

| General for all Rx requirements | R4-2220255@RAN4#105 meeting - Agreement: - Proposal 1: For NCR-MT Rx direction requirements, it’s suggested to specify following OTA Rx direction declarations. These declarations apply for both input signal and interference signal. - A declared receiver target redirection range. This range is not needed if NCR don’t support redirection. - [five] declared sensitivity RoAoA comprising the conformance testing directions. If don’t support redirection, only one RoAoA is suggested - Receiver target reference direction - Min EIS may only need for MT part Proposal 3: for the RF requirement of NCR-MT receiver, to define the refense requirement for NCR-MT receiver. |
| --- | --- |

In the previous RAN4 meeting, for FR1 local area NCR-MT, we agreed to reuse the legacy FR1 UE RF requirement, however for legacy FR1 UE RF requirement, we only have the conducted requirement without any radiated requirement for it. Based on the agreement reached in the previous RAN4 meeting, the legacy RoAoA concept (OTA REFSENS RoAoA, minSENS RoAoA) could be still reused for NCR-MT. To follow the legacy FR1 BS receiver OTA requirement principle, we propose to define the FR1 LA NCR-MT radiated receiver requirement as following:

| RF requirement | Corresponding conducted Rx requirement for FR1 LA NCR-MT | Radiated Rx requirement for FR1 LA NCR-MT | FR1 BS OTA Receiver requirement |
| --- | --- | --- | --- |
| OTA reference sensitivity | TS 38.101-1, subclause 7.3 | ΔOTAREFSENS | ΔOTAREFSENS |
| OTA maximum input level | TS 38.101-2 , subclause 7.4 | ΔOTAREFSENS | For dynamic range requirement, - ΔOTAREFSENS |
| OTA adjacent channel selectivity | TS 38.101-1, subclause 7.5 | ΔminSENS | ΔminSENS |
| OTA blocking characteristics | TS 38.101-1, subclause 7.6 | For IBB and NBB requirement.: - ΔOTAREFSENS - ΔminSENS For OOBB requirement: ΔminSENS | For IBB and NBB requirement.: - ΔOTAREFSENS - ΔminSENS For OOBB requirement: - ΔminSENS |
| OTA spurious response | TS 38.101-1, subclause 7.7 | - ΔOTAREFSENS ΔminSENS |  |
| OTA intermodulation characteristics | TS 38.101-1 [xx], subclause 7.8 | - ΔOTAREFSENS - ΔminSENS | - ΔOTAREFSENS - ΔminSENS |
| OTA spurious emissions | TS 38.101-1, subclause 7.9 | Scaled with 6ddB relaxation on top of basic receiver spurious emission requirement. |  |

Conclusions

In this contribution, we want to share further views on NCR-MT RF requirements and proposals are made as following:

Proposal 1:For the simultaneous reception of NCR-MT and NCR-Fwd in NCR input intermodulation requirement, 6dB REFSENS degradation could be allowed once the intermodulation signal fall into NCR-MT bandwidth;

Proposal 2: for FR1 LA NCR-MT receiver radiated requirement, to consider the following ways to define the requirement on top of conducted receiver requirement

| RF requirement | Corresponding conducted Rx requirement for FR1 LA NCR-MT | Radiated Rx requirement for FR1 LA NCR-MT |
| --- | --- | --- |
| OTA reference sensitivity | TS 38.101-1, subclause 7.3 | ΔOTAREFSENS |
| OTA maximum input level | TS 38.101-2 , subclause 7.4 | ΔOTAREFSENS |
| OTA adjacent channel selectivity | TS 38.101-1, subclause 7.5 | ΔminSENS |
| OTA blocking characteristics | TS 38.101-1, subclause 7.6 | For IBB and NBB requirement.: - ΔOTAREFSENS - ΔminSENS For OOBB requirement: ΔminSENS |
| OTA spurious response | TS 38.101-1, subclause 7.7 | - ΔOTAREFSENS ΔminSENS |
| OTA intermodulation characteristics | TS 38.101-1, subclause 7.8 | - ΔOTAREFSENS - ΔminSENS |
| OTA spurious emissions | TS 38.101-1, subclause 7.9 | Scaled with 6ddB relaxation on top of basic receiver spurious emission requirement. |

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- TS 38.106, NR Repeater Radio Transmission and Reception; (Release 17)

- R4-2207290, WF on repeater RF requirements, Huawei, approved.

- R4-2205030,Repeater radiated power requirements, Ericsson, Noted

- TS 38.115-2,NR;Repeater conformance testing, Part 2: Radiated conformance testing.

- TS 38.115-1,NR;Repeater conformance testing, Part 1: Conducted conformance testing.

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- TR 38.867,Study on NR Network-controlled Repeaters

- R4-2217512 ,Post_2nd_round_summary_[104bis-e][314]NR_netcon_repeater, ZTE

- R4-2217506,LS to RAN1 on NCR-MT transmission and Beam correspondence, CMCC, approved.

- R4-2215488, discussion on NCR RF requirements, CMCC, Noted

- R4-2220140 Summary for [105][314] NR_netcon_repeater.

- R4-2303759, Topic summary for [106][314] NR_netcon_repeater, Noted.

- R4-2305931 WF on system parameters and RF requirements for NCR_final,ZTE, approved.

- R4-2309893,WF for system parameter and NCR RF requirements, Approved

- R4-2313903,WF for NCR RF requirements, ZTE, Approved.

- R4-2314690,draft Big CR to TS 38.101-1: 4Tx requirements,Huawei, HiSilicon

- R4-2316996, WF on NCR RF requirement, ZTE, approved.