3GPP TSG-RAN4 Meeting#109	R4-2320347

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | ❌ | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of introduction of NCR into TS 38.106: Clause 1 ~4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce the spec impact in clause 1~4 due to NCR node |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the NCR into the clause 1~4 in TS 38.106. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NCR is not defined in the TS38.106 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.115-1 and 38.115-2 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

1	Scope

The present document establishes the minimum RF characteristics of NR Repeater and network controlled repeater. For network controlled repeater (NCR), requirements for NCR-Fwd and NCR-MT apply..

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 25.942: "RF system scenarios".

[4]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”.

[7]	3GPP TS 38.115-1: “NR; Repeater conformance testing - Part 1: Conducted conformance testing”.

[8]	3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing”.

[9]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[10]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[11]	Void

[12]	Void

[13]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[14]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”.

[15]	Void

[16]	Void

[17]	Void

[18]	Void

[19]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[20]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Antenna connector: connector at the conducted interface of the repeater type 1-C

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

Beam peak direction: direction where the maximum EIRP is found

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

directional requirement: requirement which is applied in a specific direction within the OTA coverage range.

Equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

Fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

gap between passbands: frequency gap between two consecutive passbands that belong to the same operating band, where the RF requirements in the gap are based on co-existence for un-coordinated operation

Inter-passband gap: The frequency gap between two supported consecutive passbands that belong to different operating bands.

Maximum passband output power: mean power level measured per passband at the antenna connector, during the transmitter ON state in a specified reference condition

Maximum passband TRP output power: mean power level measured per passband during the transmitter ON state in a specified reference condition and corresponding to the declared rated passband TRP output power (Prated,p,,TRP)

Measurement bandwidth: RF bandwidth in which an emission level is specified

multi-band connector: Antenna Connector for a Multi-band repeater.

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Nominal channel bandwidth: Bandwidth calculated as min(100MHz, BWpassband) in FR1 or min(400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

Non-contiguous spectrum: spectrum consisting of two or more passbands separated by inter-passband gap(s).

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA coverage range: a common range of directions within which OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

Passband: The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.

passband edge: Frequency at the edge of the passband

Radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

Rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction during the transmitter ON period

Rated passband output power: mean power level associated with a passband the manufacturer has declared to be available at the antenna connector, during the transmitter ON period in a specified reference condition

Rated passband TRP output power: mean power level declared by the manufacturer per passband, that the manufacturer has declared to be available at the RIB during the transmitter ON state

Rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the antenna connector, during the transmitter ON state in a specified reference condition

Rated total TRP output power: mean power level associated with a particular operating band, that the manufacturer has declared to be available at the RIB during the transmitter ON state in a specified reference condition

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

Repeater type 1-C: Repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

Repeater type 2-O: Repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Requirement set: one of the NR requirements set as defined for NR repeater

single-band connector: Repeater type 1-C antenna connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the repeater.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWNominal	Nominal channel bandwidth

BWPassband	Passband bandwidth

f	Separation between the passband edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper passband edge

Foffset,low	Frequency offset from FC,low to the lower passband edge

f_offset	Separation between the passband edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the operating band

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

PEM,n54,ind	Declared emission level for Band n54; ind = a, b, c, d, e, f

Prated,p,AC	Rated passband output power per antenna connector

Prated,t,AC	Rated total output power declared per antenna connector

Prated,p,EIRP 	Rated passband EIRP output power

Prated,p,TRP	Rated passband TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

Pin,p,AC	Input power intended to produce the maximum rated output power (Prated,p,AC) at the antenna connector

Pin,p,EIRP	Input power intended to produce the maximum rated output power (Prated,p,TRP) at the RIB

Prated,out,FBWhigh	The rated output EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,out,FBWlow	The rated output EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Pmax,p,AC	Maximum passband output power measured per antenna connector

Pmax,p,EIRP	Maximum passband EIRP output power when repeater is configured at the rated passband TRP output power (Prated,p,TRP)

Pmax,p,TRP	Maximum passband TRP output power measured per RIB

Wgap	Inter passband Bandwidth gap size

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

AoA	Angle of Arrival

BW	Bandwidth

CACLR	Cumulative ACLR

CP-OFDM	Cyclic Prefix-OFDM

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

EIRP	Effective Isotropic Radiated Power

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RIB	Radiated Interface Boundary

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

TRP	Total Radiated Power

UL	Uplink

WA	Wide Area

4	General

4.1	Relationship between Minimum Requirements and Test Requirements

Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification TS 38.115-1 [7] or TS 38.115-2 [8].

The minimum requirements given in this specification make no allowance for measurement uncertainty. The test specifications TS 38.115-1 [7] and TS 38.115-2 [8] define test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in recommendation ITU-R M.1545 [6].

4.2	Conducted and radiated requirement reference points

4.2.1	Repeater type 1-C

4.2.1A	Network controlled Repeater type 1-C

For NCR type 1-C, the NCR-Fwd RF requirements are applied at the NCR antenna connector (BS-side connector or UE-side connector) for downlink or uplink for the configuration in normal operating conditions.

For NCR type 1-C, the NCR-MT RF requirements are applied at the NCR antenna connector (BS-side connector) for the configuration in normal operating conditions.

Figure 4.2.1-1A: Network controlled Repeater type 1-C downlink and uplink interface

NOTE 1: the NCR-MT and NCR-Fwd may have the same or separate antenna connectors.

4.2.1B	Network controlled Repeater type 1-H

For NCR type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

For NCR type 1-H, the NCR-Fwd conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector or UE-side TAB connector) for downlink or uplink for the configuration in normal operating conditions.

For NCR type 1-H, the NCR-MT conducted RF requirements are applied at the NCR individual or groups of TAB connectors at the transceiver array boundary (BS-side TAB connector) for the configuration in normal operating conditions.

Figure 4.2.1-1B: Network controlled Repeater type 1-H downlink and uplink interface

NOTE 1: the NCR-MT and NCR-Fwd may have the same or separate TAB connectors.

4.2.1C	Network controlled Repeater type 1-O

For NCR type 1-O, the radiated characteristics for NCR-Fwd and NCR-MT are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

For NCR type 1-O, the NCR-MT conducted RF requirements are applied at the BS side RIB for the configuration in normal operating conditions.

Figure 4.2.1-1C: Network controlled Repeater type 1-O downlink and uplink interface

NOTE 1: the NCR-MT and NCR-Fwd may have the same or separate RIB

4.2.2	Repeater type 2-O

For repeater type 2-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

4.2.2A	Network controlled Repeater type 2-O

For NCR type 2-O, the radiated characteristics for NCR-Fwd and NCR-MT are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

For NCR type 2-O, the NCR-MT conducted RF requirements are applied at the BS side RIB for the configuration in normal operating conditions.

Figure 4.2.2-1A: Radiated reference points for network controlled repeater type 2-O

NOTE 1: the NCR-MT and NCR-Fwd may have the same or separate RIB

4.3	Repeater classes

4.3.1	Repeater class for downlink

The requirements in this specification apply to downlink Wide Area repeaters, downlink Medium Range repeaters and downlink Local Area repeaters unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For repeater type 1-C and type 2-O, repeater downlink classes are defined as indicated below:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell scenarios with a repeater to UE minimum distance along the ground equal to 35 m.

-	Medium Range repeaters are characterised by requirements derived from Micro Cell scenarios with a repeater to UE minimum distance along the ground equal to 5 m.

-	Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios.

-	Note: The requirements in this specification for LA 1-C repeaters apply to 1-C repeaters with declared output power less than or equal to LA rated output power limits as in table 6.2.1-1.

4.3.2	Repeater class for uplink

The requirements in this specification apply to uplink Wide Area repeaters and uplink Local Area repeaters unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For repeater type 1-C and type 2-O, repeater uplink classes are defined as indicated below:

-	Wide Area repeaters are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area repeaters are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

4.3A	Network controlled repeater classes

4.3A.1	Network controlled repeater class for downlink

The requirements in this specification apply to downlink Wide Area NCR, downlink Medium Range NCR and downlink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for repeater with and without connectors.

For NCR type 1-C, type 1-H, type 1-O and type 2-O, NCR downlink classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell scenarios with a NCR to UE minimum distance along the ground equal to 35 m.

-	Medium Range NCR are characterised by requirements derived from Micro Cell scenarios with a NCR to UE minimum distance along the ground equal to 5 m.

-	Local Area NCR are characterised by requirements derived from Pico Cell scenarios with a NCR to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios.

-	Note: The requirements in this specification for LA NCR type 1-C apply to NCR type 1-C with declared output power less than or equal to LA rated output power limits as in table 6.2.1-1.

4.3A.2	Network controlled repeater class for uplink

The requirements in this specification apply to uplink Wide Area NCR and uplink Local Area NCR unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR with and without connectors.

For NCR type 1-C, type 1-H, type 1-O and type 2-O, NCR uplink classes are defined as indicated below:

-	Wide Area NCR are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area NCR are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.4-1  lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.2 | Repeater output power | For Band n41 and n90 operation in Japan, additional output power limits shall be applied. |
| 6.2.4, 7.3.4 | Repeater output power, OTA repeater output power: Additional requirements | These requirements may be applied regionally as additional repeater output power requirements. |
| 6.5.2 | Adjacent Channel Leakage Power Ratio | For Band n41 and n90 operation in Japan, absolute ACLR limits shall be applied to the sum of the absolute ACLR power over all antenna connectors for repeater type 1-C. |
| 6.5.3.2, 7.5.3.2 | Operating band unwanted emission, OTA operating band unwanted emissions | Category A or Category B operating band unwanted emissions limits may be applied regionally. |
| 6.5.3.2.5.1 | Operating band unwanted emission | The repeater may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.5.3.2.5.2 | Operating band unwanted emission | The repeater operating in Band n20 may have to comply with the additional requirements for protection of DTT, when deployed in certain regions. |
| 6.5.3.2 | Operating band unwanted emissions | For Band n41 and n90 operation in Japan, the operating band unwanted emissions limits shall be applied to the sum of the emission power over all antenna connectors for repeater type 1-C. |
| 6.5.4.2.1, 7.5.4.2 | Tx spurious emissions, OTA Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [5], may apply regionally. |
| 6.5.4.2.2, 7.5.4.2.3 | Tx spurious emissions: additional requirements, OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the repeater operating band. |
| 6.5.4.2 | Transmitter spurious emissions | For Band n41 and n90 operation in Japan, the sum of the spurious emissions over all antenna connectors for repeater type 1-C shall not exceed the basic limits. |
| 6.5.5.1 | Receiver spurious emissions | For Band n41 and n90 operation in Japan, the sum of receiver spurious emissions requirements over all antenna connectors for repeater type 1-C shall not exceed minimum requirements defined in clause 6.5.5.2. |
| 6.7.2 | Input intermodulation | Interfering signal positions that are partially or completely outside of any downlink operating band of the repeater are not excluded from the requirement in Japan in Band n77, n78, n79. |
| 6.8 | Output intermodulation | For Band n41 and n90 operation in Japan, the repeater may have to comply with the additional requirements, when deployed in certain regions. |

4.5	Applicability of requirements

In table 4.5-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.5-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | Repeater type 1-C | Repeater type 2-O |
| Repeater output power | 6.2 |  |
| Frequency stability | 6.3 |  |
| Out of band gain | 6.4 |  |
| Unwanted emissions | 6.5 |  |
| Error Vector Magnitude | 6.6 | NA |
| Input intermodulation | 6.7 |  |
| Output intermodulation | 6.8 |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 |  |
| Transmit ON/OFF power | 6.10 |  |
| Repeater output power |  | 7.2 |
| OTA frequency stability |  | 7.3 |
| OTA out of band gain |  | 7.4 |
| OTA unwanted emissions |  | 7.5 |
| OTA Error Vector Magnitude | NA | 7.6 |
| OTA input intermodulation |  | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  | 7.8 |
| OTA transmit ON/OFF power |  | 7.9 |

Table 4.5-1a: Requirement set applicability for NCR-Fwd

| Requirement | Requirement set |  |  |  |
| --- | --- | --- | --- | --- |
|  | NCR-Fwd type 1-C | NCR-Fwd type 1-H | NCR-Fwd type 1-O | NCR-Fwd type 2-O |
| Repeater output power | 6.2 | 6.2 |  |  |
| Frequency stability | 6.3 | 6.3 |  |  |
| Out of band gain | 6.4 | 6.4 |  |  |
| Unwanted emissions | 6.5 | 6.5 |  |  |
| Error Vector Magnitude | 6.6 | 6.6 | NA | NA |
| Input intermodulation | 6.7 | 6.7 |  |  |
| Output intermodulation | 6.8 | 6.8 |  |  |
| Adjacent Channel Rejection Ratio (ACRR) | 6.9 | 6.9 |  |  |
| Transmit ON/OFF power | 6.10 | 6.10 |  |  |
| Repeater output power |  |  | 7.2 | 7.2 |
| OTA frequency stability |  |  | 7.3 | 7.3 |
| OTA out of band gain |  |  | 7.4 | 7.4 |
| OTA unwanted emissions |  |  | 7.5 | 7.5 |
| OTA Error Vector Magnitude | NA | NA | 7.6 | 7.6 |
| OTA input intermodulation |  |  | 7.7 | 7.7 |
| OTA Adjacent Channel Rejection Ratio (ACRR) |  |  | 7.8 | 7.8 |
| OTA transmit ON/OFF power |  |  | 7.9 | 7.9 |

Table 4.5-1b: Requirement set applicability for NCR-MT

|  | NCR-MT type 1-C | NCR-MT type 1-H | NCR-MT type 1-O | NCR-MT type 2-O |
| --- | --- | --- | --- | --- |
| Conducted transmitter power | 6.2.3.2 | 6.2.3.2 |  |  |
| Conducted output power dynamics | 6.11 | 6.11 |  |  |
| Conducted transmit signal quality | 6.12 | 6.12 |  |  |
| Conducted output RF spectrum emissions | 6.5 | 6.5 |  |  |
| Diversity characteristics | 6.13 | 6.13 | NA | NA |
| Conducted reference sensitivity | 6.14 | 6.14 |  |  |
| Conducted maximum input level | 6.15 | 6.15 |  |  |
| Conducted adjacent channel selectivity | 6.16 | 6.16 |  |  |
| Conducted blocking characteristics | 6.17 | 6.17 |  |  |
| Conducted spurious response | 6.18 | 6.18 |  |  |
| Conducted intermodulation characteristics | 6.19 | 6.19 |  |  |
| Conducted spurious emissions | 6.5.5 | 6.5.5 |  |  |
| Radiated transmitter power |  |  | 7.2 | 7.2 |
| Radiated output power dynamics |  |  | 7.10 | 7.10 |
| Radiated transmit signal quality |  |  | 7.11 | 7.11 |
| Radiated output RF spectrum emissions |  |  | 7.5 | 7.5 |
| Diversity characteristics | NA | NA | 7.12 | 7.12 |
| Radiated reference sensitivity |  |  | 7.13 | 7.13 |
| Radiated maximum input level |  |  | 7.14 | 7.14 |
| Radiated adjacent channel selectivity |  |  | 7.15 | 7.15 |
| Radiated blocking characteristics |  |  | 7.16 | 7.16 |
| Radiated spurious response |  |  | 7.17 | 7.17 |
| Radiated intermodulation characteristics |  |  | 7.18 | 7.18 |
| Radiated spurious emissions |  |  | 7.19 | 7.19 |

4.6	Requirements for contiguous and non-contiguous spectrum

A spectrum allocation where a repeater operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for repeater configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For repeater operation in non-contiguous spectrum, some requirements apply at the repeater passband edges. For each such requirement, it is stated how the limits apply relative to the repeater gap between passbands and the Inter-passband gap respectively.

For NCR operation in non-contiguous spectrum, some requirements apply at the NCR-Fwd passband edges or NCR-MT carrier edges. For each such requirement, it is stated how the limits apply relative to the NCR gap between passbands and the Inter-passband gap respectively.

4.7	Requirements for repeater capable of multi-band operation

For multi-band connector or multi-band RIB, the RF requirements in clauses 6 and 7 apply separately to each supported operating band unless otherwise stated. For some requirements, it is explicitly stated that specific additions or exclusions to the requirement apply at multi-band connector(s), and multi-band RIB(s) as detailed in the requirement clause. For repeater or NCR capable of multi-band operation, various structures in terms of combinations of different downlink and uplink implementations (multi-band or single band) with mapping to one or more antenna connectors for repeater type 1-C, NCR type 1-C, type 1-H and type 1-O in different ways are possible. For multi-band connector(s) the exclusions or provisions for multi-band apply. For single-band connector(s), the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, output intermodulation, ACRR and receiver spurious emissions requirements apply to this connector that is mapped to single-band.

-	If the repeater or NCR is configured for single-band operation, single-band requirements shall apply to this antenna connector configured for single-band operation and no exclusions or provisions for multi-band capable repeater are applicable. Single-band requirements are tested separately at the antenna connector configured for single-band operation, with all other antenna connectors terminated.

<End of change>