3GPP TSG-RAN4 Meeting#109	R4-2320348

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | ❌ | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of introduction of NCR into TS 38.106: Clause 7.12 to 7.19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-10-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce the NCR-MT radiated receiver requirement in clause 7.12 to 7.19 in TS 38.106. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the NCR-MT radiated receiver requirement in clause 7.12 to 7.19 in TS 38.106. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Radiated NCR-MT receiver requirements are not defined in the TS38.106 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12~7.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.115-1 and 38.115-2 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

7.12	Diversity characteristics

7.12.1	General

7.12.2	Minimum requirement for NCR-MT

The OTA diversity characteristic is specified the same as conducted diversity characteristic for FR1 NCR type 1-C, 1-H in sub-clause 6.13.

7.13	OTA reference sensitivity

7.13.1	General

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.13.2	Minimum requirement for NCR-MT type 1-O

The wide area NCR-MT reference sensitivity level is specified the same as the Wide Area NCR-MT reference sensitivity level requirement  in TS 38.174 [xx], subclause 10.3.3.2.

The local area NCR-MT reference sensitivity level is specified as reference sensitivity power level in TS 38.101-1 [xx], subclause 7.3 minus ΔOTAREFSENS arrives from any direction within the OTA REFSENS RoAoA where the ΔOTAREFSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT.

7.13.3	Minimum requirement for NCR-MT type 2-O

The wide area NCR-MT reference sensitivity level is specified the same as the Wide Area IAB-MT reference sensitivity level requirement  in TS 38.174 [xx], subclause 10.3.3.3.

The local area NCR-MT reference sensitivity level is specified the same as reference sensitivity power level for power class 3 in TS 38.101-2 [xx], subclause 7.3.2.3.

7.14	OTA maximum input level

7.14.1	General

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

7.14.2	Minimum requirement for NCR-MT type 1-O

The local area NCR-MT maximum input power is specified the same as  maximum input power specified in TS 38.101-1 [xx], subclause 7.4 with power level minus ΔOTAREFSENS.

7.14.3	Minimum requirement for NCR-MT type 2-O

The local area NCR-MT maximum input power is specified the same as maximum input power in TS 38.101-2 [xx], subclause 7.4.

7.15	OTA adjacent channel selectivity

7.15.1	General

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

7.15.2	Minimum requirement for NCR-MT type 1-O

The wide area NCR-MT ACS requirement is specified the same as the Wide Area IAB-MT ACS requirement  in TS 38.174 [xx], subclause 10.5.1.5.

The local area NCR-MT ACS requirement is specified same as ACS requirement  in TS 38.101-1 [xx], subclause 7.5 with power level minus ΔminSENS where the ΔminSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

7.15.2	Minimum requirement for NCR-MT type 2-O

The wide area NCR-MT ACS requirement is specified the same as the Wide Area IAB-MT ACS requirement  in TS 38.174 [xx], subclause 10.5.1.4.

The local area NCR-MT reference sensitivity level is specified the same as ACS requirement in TS 38.101-2 [xx], subclause 7.5.

7.16	OTA blocking characteristics

7.16.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

7.16.2	Minimum requirement for NCR-MT type 1-O

The wide area NCR-MT blocking requirement is specified the same as the Wide Area IAB-MT blocking requirement  in TS 38.174 [xx], subclause 10.5.2.5.

The local area NCR-MT blocking requirement is specified same as blocking requirement  in TS 38.101-1 [xx], subclause 7.6 with power level minus ΔOTAREFSENS where the ΔOTAREFSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA..

.The local area NCR-MT blocking requirement is specified same as blocking requirement  in TS 38.101-1 [xx], subclause 7.6 with power level minus ΔminSENS where the ΔminSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when- the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

7.16.3	Minimum requirement for NCR-MT type 2-O

The wide area NCR-MT blocking requirement is specified the same as the Wide Area IAB-MT blocking requirement  in TS 38.174 [xx], subclause 10.5.2.4.

The local area NCR-MT blocking requirement is specified the same as blocking requirement in TS 38.101-2 [xx], subclause 7.6.

7.17	OTA spurious response

7.17.1	General

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.16 is not met.

7.17.2	Minimum requirement for NCR-MT type 1-O

The local area NCR-MT spurious response requirement is specified same as receiver spurious response requirement  in TS 38.101-1 [xx], subclause 7.7 with power level minus ΔOTAREFSENS where the ΔOTAREFSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.. .

7.18	OTA intermodulation characteristics

7.18.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

7.18.2	Minimum requirement for NCR-MT type 1-O

The wide area NCR-MT intermodulation requirement is specified the same as the Wide Area IAB-MT intermodulation requirement  in TS 38.174 [xx], subclause 10.8.4.

The local area NCR-MT intermodulation requirement is specified same as intermodulation requirement  in TS 38.101-1 [xx], subclause 7.8 with power level minus ΔOTAREFSENS where the ΔOTAREFSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

The local area NCR-MT intermodulation requirement is specified same as intermodulation requirement  in TS 38.101-1 [xx], subclause 7.8 with power level minus ΔminSENS where the ΔminSENS definition  in the clause 10.1 in TS 38.174 applies for NCR-MT. The requirement shall apply at the RIB when- the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

7.19	OTA spurious emissions

7.19.1	General

The spurious emissions power is the power of emissions generated or amplified in a receiver. The spurious emissions power level is measured as TRP.

7.19.2	Minimum requirement for NCR-MT type 1-O

The wide area NCR-MT receiver spurious emission requirement is specified the same as the Wide Area IAB-MT receiver spurious emission requirement  in TS 38.174 [xx], subclause 10.7.3.1.

The local area NCR-MT receiver spurious emission requirement is specified same as intermodulation requirement  in TS 38.101-1 [xx], subclause 7.9 with scaling factor 6dB relaxation on top of basic limit.

7.19.3	Minimum requirement for NCR-MT type 2-O

The wide area NCR-MT receiver spurious emission requirement is specified the same as the Wide Area IAB-MT receiver spurious emission requirement  in TS 38.174 [xx], subclause 10.7.3.2.

The local area NCR-MT receiver spurious emission requirement is specified the same as receiver spurious emission requirement in TS 38.101-2 [xx], subclause 7.9.

<End of change>