3GPP TSG-WG4 Meeting # 109	R4-2320358

# Chicago, US,  13 - 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR # 6 General aspects: Introduction (including general aspects of PRS measurements with bandwidth aggregation and CPP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update general aspects of positioning measurement in RRC_INACTIVE state when UE is configured: with eRDX cycle <= 10.24s, eDRX cycle is > 10.24s, to perform bandwidth aggregated positioning measurements, to perform RSCPD with RSTD, and to perform RSCP with UE Rx-Tx time difference measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated clause 5.6.1 to capture aspects related to positioning measurement in RRC_INACTIVE state when UE is configured: with eRDX cycle <= 10.24s, eDRX cycle is > 10.24s, to perform bandwidth aggregated positioning measurements, to perform RSCPD with RSTD, and to perform RSCP with UE Rx-Tx time difference measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning measurement in RRC_INACTIVE state when UE is configured: with eRDX cycle <= 10.24s, eDRX cycle is > 10.24s, to perform bandwidth aggregated positioning measurements, to perform RSCPD with RSTD, and to perform RSCP with UE Rx-Tx time difference measurement is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE>

## 5.6	NR measurements for positioning

### 5.6.1	Introduction

This clause contains requirements for UE capable of performing NR positioning measurements defined in TS 38.215 [4], including RSTD, PRS-RSRP, UE Rx-Tx time difference, PRS-RSRPP, RSTD based on aggregated PRS resources from multiple PFLs, UE Rx-Tx time difference based on aggregated PRS resources and aggregated SRS resources from multiple PFLs, RSCPD reported together with RSTD, RSCP reported together with UE Rx-Tx time difference in RRC_INACTIVE state.

The requirements in clauses 5.6.2, 5.6.2.x1, 5.6.3, 5.6.4, 5.6.4.x1, 5.6.5, 5.6.x1, and 5.6.x2 are applicable to PRS resources that do not collide with other DL signals/channels which include SSB, SIB1, CORESET0, MSG2/MSGB, paging and DL SDT. In addition, a UE is not expected to receive PRS resources that collide with a time interval starting at symbol m and ending at symbol m + N2, where symbol m is the last symbol in which the UE is configured to receive PDCCH and N2 is defined in clause 6.4 of [26, TS 38.214] for the subcarrier spacing μ of the DL PRS.

If a PRS resource is outside or partially overlapped with the intitial DL BWP, a PRS resource instance collides with another DL signals/channels if any portion of the other DL signal/channel overlaps with the time interval starting X symbols before the PRS instance and ending X symbols after the PRS instance, taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD. Where X is defined in Table 5.6.1-1.

Table 5.6.1-1: Value of X number of symbols

| FR |  | NR Slot length (ms) | X symbols |
| --- | --- | --- | --- |
| FR1 | 0 | 1 | 7 |
|  | 1 | 0.5 | 14 |
|  | 2 | 0.25 | 28 |
| FR2 | 2 | 0.25 | 14 |
|  | 3 | 0.125 | 28 |
| Note 1: The FR1 value applies if one or both of the serving cell and the positioning frequency layer are in FR1. FR2 value applies if both of the serving cell and the positioning frequency layer are in FR2. |  |  |  |

Measurement requirements specified in clauses 5.6.2, 5.6.2.x1, 5.6.3, 5.6.4, 5.6.4.x1, 5.6.5, 5.6.x1, and 5.6.x2 shall apply for any DRX, eDRX_INACTIVE ≤ 10.24s and eDRX_INACTIVE >10.24s configuration specified in TS 38.331 [2].

The requirements in clauses 5.6.2, 5.6.2.x1, 5.6.3, 5.6.4, 5.6.4.x1, 5.6.5, 5.6.x1, and 5.6.x2 are applicable provided that the cell selection procedure for the selected PLMN defined in TS 38.304 [1] is not triggered during PRS measurement period.

Requirements in clause 5.6.1A shall apply when UE is configured to perform RSTD, PRS-RSRP, UE Rx-Tx time difference, PRS-RSRPP, RSTD based on aggregated PRS resources from multiple PFLs, UE Rx-Tx time difference based on aggregated PRS resources and aggregated SRS resources from multiple PFLs, RSCPD reported together with RSTD, RSCP reported together with UE Rx-Tx time difference measurements in RRC_INACTIVE state and eDRX cycle is larger than the configured PRS measurement reporting periodicity (if PRS measurement is configured) or the configured SRS transmission periodicity (if SRS transmission for positioning is configured).

The requirements in clauses 5.6.2, 5.6.2.x1, 5.6.3, 5.6.4, 5.6.4.x1, and 5.6.5 apply provided that:

- UE is not configured with eDRX cycle and all PRS resources within a PFL or effective PFL are within up to 2 separate windows within TPRS,i for each positioning frequency layer i as defined in clauses 5.6.2, 5.6.3, 5.6.4 and 5.6.5, where each window is up to 10ms.

- Or UE is configured with eDRX cycle and positioning measurements are started by UE within PTW or outside of the PTW.

The requirements in clauses 5.6.x1 and 5.6.x2 apply provided that the UE is configured with a time window (TRSCPD) for RSCPD measurement together with RSTD measurement or with a time window (TRSCP) for RSCP measurement with UE Rx-Tx measurements as specified in TS 37.355 [34].

The UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

When the UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

<END OF CHANGE>