# 3GPP TSG-RAN WG4 Meeting # 109	R4-2320376

# Chicago, US, November 13 – 17, 2023

Agenda Item:			8.13.3.2

Source:					Qualcomm Incorporated

Title:                       TP to TR 38.876 on ATG UE Maximum input level

Document for:		Approval

# Introduction

Based on the agreements of ATG UE maximum input level requirements in RAN4#108bis, we propose to approve the following TP.

# Draft TP on TR 38.876

---------------------------------------------------------Start of the changes -------------------------------------------------------------

#### 7.1.3.4 Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. For ATG, the maximum input level requirements are defined in Table 7.1.3.4-1 based on two types of UE antenna, i.e., omni-directional antenna and phased array antenna.

Table 7.1.3.4-1: Maximum input level for ATG

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Omni-directional antenna: receiver characteristics specified at the antenna connector(s) | Phased array antenna: receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

---------------------------------------------------------End of the changes---------------------------------------------------------------