3GPP TSG-RAN WG4 Meeting #109	R4-2320377

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to introduce ATG UE RF Rx requirements - part 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce ATG UE RF Rx requirements including 7.3J Reference sensitivity for ATG, 7.4J Maximum input level for ATG and Adjacent channel selectivity for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce requirements in clause 7.3J, 7.4J and 7.5J. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no corresponding ATG UE RF Rx requirements in current spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3J, 7.4J, 7.5J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

## 7.3J	Reference sensitivity for ATG

### 7.3J.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the CPE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3J.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [4] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [4]) with parameters specified in Table 7.3J.2-1, Table 7.3J.2-2 and , Table 7.3J.2-3.

Table 7.3J.2-1: ATG UE with two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

Table 7.3J.2-2: ATG UE with two antenna port reference sensitivity QPSK REFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n78 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n79 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |

For ATG UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3J.2-3for the applicable operating bands.

Table 7.3J.2-3: ATG UE with four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n1, n39 | -2.7 |
| n78, n79 | -2.2 |

Table 7.3J.2-4: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 |  | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 |  | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 |  | 301 | 301 | 301 |  |  |  |  |  |  |
| n39 | 15 | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n78 | 15 |  | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  | 50 |  | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 |  | 50 |  | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 |  | 24 |  | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |

## 7.4J	Maximum input level for ATG

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4J-1.

Table 7.4J-1: Maximum input level for ATG

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Omni-directional antenna: receiver characteristics specified at the antenna connector(s) | Phased array antenna: receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

## 7.5J	Adjacent channel selectivity for ATG

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirements specified in Table 7.5J-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5J-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -42 dBm with omni-directional antenna and -30dBm with phased array antenna for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5J-3 and Table 7.5J-4 for verification of the requirements specified in Table 7.5J-1, and as in Table 7.5J-5 and Table 7.5J-6 for verification of the requirements specified in Table 7.5J-2. For these test parameters, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5J-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5J-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 |

Table 7.5J-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5J-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -56.5 |  | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -425 -306 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. NOTE 5: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 6: Pinterferer shall be set to -30dBm for phased array antenna. |  |  |  |  |  |

Table 7.5J-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5J-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | -56.5 |
| Pinterferer | dBm | -424 -305 |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for phased array antenna. |  |  |

## <<End of Change>>