3GPP TSG-RAN WG4 Meeting # 109	R4-2320416

Chicago, America, November 13 – November 17, 2023

Agenda item:	5.21.3

Source:	MediaTek Inc.

Title:	Discussion on the interoperability and testability aspects of AIML RAN4 requirements

Document for:	Discussion

- Introduction

In RAN4#108bis meeting, a table is provided to summarize issues to be discussed in describing the options for the testing of 2-sided model [1]. In this contribution, we provide our inputs to the left parts in the table.

- Discussion

There are 4 options test decoders in the 2-sided model.

- Option 1: Test decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: Test decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained - Option 3: The Test decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The Test decoder(s) are partially specified and captured in RAN4 spec.

In Table. 1, we provide our analysis/view on different options for the left parts.

Table 1. Summary of test decoder design options for 2-sided models

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | Up to TE vendor. Alignment with DUT/infra vendors may be required. |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type 1/2/3 Depending on DUT implementation | Type 1/2/3 Depending on cooperation between DUT and infra/TE vendors | Type 1/2/3 Depending on cooperation between DUT and TE vendors | Type 1/2/3 Depending on cooperation between DUT and TE vendors |
| Test decoder verification procedure at TE and/or DUT | Not needed for DUT Needed for TE to verify the compatibility | Not needed for DUT Needed for TE to verify the compatibility | Not needed for DUT Not needed for TE | Needed for DUT Not needed for TE |
| Feasibility of test decoder verification procedure | DUT side: NA TE side: DUT needs to cooperate with TE | DUT side: infra vendor needs to cooperate with DUT TE side: infra vendor needs to cooperate with TE | DUT side: NA TE side: NA | DUT side: TE needs to cooperate with DUT TE side: NA |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | The test decoder provided by DUT vendor may not reflect the actual decoder implemented by infra vendor | The test decoder provided by infra vender may reflect the performance in the field since infra vendors may use the same or similar decoder in the field as the test decoder | Depends on the test decoder decided during discussion | Depends on the test decoder decided during discussion |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | TE needs to support various test decoders from DUT vendor | TE needs to support various test decoders from infra vendor | Single implementation, TE develops the test decoder fully specified in RAN4 specification | Single implementation, TE develops the test decoder partially specified in RAN4 specification and also the unspecified part. |
| Specification Effort (e.g. test decoder) | little effort Need some limitation on test decoder to ensure it can be implemented on TE side | little effort Need some limitation on test decoder to ensure it can be implemented on TE side | Consensus of a reference model in RAN4 may be a challenging task | Less heavy workload than fully specify test decoder in Option 3 |
| Confidentiality/IP issues | Need to be considered. Model exposure is required from DUT to TE | Need to be considered. Model exposure is required from infra vender to TE | No issue | No issue |
| Applicability to different scenarios/conditions/ configurations | Maybe | Maybe | Maybe | Maybe |
| Complexity of actual testing procedure for the ecosystem | High Need cooperation between DUT and TE | High Need cooperation between infra vender and TE | Low TE only needs to implement the test decoder | Low TE only needs to implement the test decoder |

- Conclusion

In this contribution, we provide our inputs to the left parts in comparison table for the 2-sided model.

- Reference

- R4-2314740, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”