3GPP TSG-RAN WG4 Meeting #109	R4-2320427

Chicago, US, Nov 13 - 17, 2023

Source:	ZTE Corporation

Title:	Discussion on receiving time difference

Agenda Item:	8.7.2.6

Document for:	Approval

- Introduction

In last RAN4 meeting, RAN4 has discussed the receiving time difference for multi-Rx. Some controversial issues were concluded with multiple options in [1].

Issue 3-1-1: Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability - Way forward: Companies are requested to bring further analysis Issue 3-1-2: How to capture RTD applicability conditions in multi-RX WI - Way forward: Further discuss in next meeting. - Option 1: This condition is mentioned for each relevant multi-RX RRM requirement such as scheduling/measurement restriction, dual TCI state switching, L1-RSRP for GBBR. - Option 2: In clause 3.6 “Applicability of requirements in this specification version,” it is stated that RTD < CP as a condition for the relevant requirements to apply

In this document, we provide our views on these remaining issues.

- Discussion

In previous meeting, RAN4 has approved the following agreements in terms of RTD constraint:

Agreement: - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI. - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

The main intention is to avoid additional work for defining measurement/scheduling restriction considering the RTD > CP case. While regarding the requirements of L1-RSRP measurement, for GBBR measurement, it has been assumed that a single panel is used at a time instance. So in fact whether RTD > CP or not, which has no impact on the requirements of GBBR measurement. So regarding the applicability of GBBR measurement requirements, the condition of RTD > CP can be ignored. Besides, the applicability of measurement/scheduling restriction should be constraint to the case of RTD < CP.

Proposal 1: The restriction of RTD < CP is not set as the applicability condition of GBBR measurement requirements. Besides, the applicability of measurement/scheduling restriction should be constraint to the case of RTD < CP.

- Conclusion

In this contribution, we have the following proposal around the receiving time difference:

Proposal 1: The restriction of RTD < CP is not set as the applicability condition of GBBR measurement requirements. Besides, the applicability of measurement/scheduling restriction should be constraint to the case of RTD < CP.

- Reference

- R4-2317427 “WF on FR2 multi-Rx chain DL reception (part2)”, Ericsson.