,GPP TSG-RAN WG4 Meeting # 109	R4-2320460

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft #26: SL RSTD and SL PRS-RSRP measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SL positioning is supported by other Rel-18 specifications but no RRM requirements exist in 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction to SL positioning requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No general description text in 38.133 for the SL positioning requirements being introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections for SL positioning requirements: 12A.2, 12A.2.1, 12A.2.2, 12A.2.3, 12A.2.4, 12A.2.5, 12A.3, 12A.3.1, 12A.3.2, 12A.3.3, 12A.3.4, 12A.3.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 12A.2	SL RSTD measurements

Editor’s note: SL-PRS based RSTD measurement requirements for UE in in-coverage scenario.

### 12A.2.1	Introduction

The requirements in clause 12A.2 shall apply provided the UE has received [SLPP-RequestLocationInformation] message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL RSTD measurements defined in TS 38.215 [4] based on SL PRS.

### 12A.2.2	Requirements Applicability

The requirements in clause 12A.2 apply for periodic and triggered RSTD measurements, provided:

-	SL RSTD related side conditions given in clause TBD for FR1 are fulfilled, for a corresponding Band.

### 12A.2.3	Measurement Capability

UE SL RSTD measurement capability is as indicated by the UE in [sl-TDOA-ProvideCapabilities], according to TS 38.355 [37].

### 12A.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH or SCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH or SCCH. The delay uncertainty is: 2 x TTIDCCH/SCCH where TTIDCCH/SCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses TBD.

The SL RSTD measurements performed and reported according to this section shall meet the SL RSTD measurement accuracy requirements in clause TBD, for each measured SL-PRS resource.

### 12A.2.5 Measurements Period Requirements

When physical layer receives last of [sl-TDOA-ProvideAssistanceData] message and [sl-TDOA-RequestLocationInformation] message from LMF or another UE via SLPP [37], the UE shall be able to perform at least [TBD] SL RSTD measurements, defined in TS 38.215 [4], during the measurement period TSL RSTD,Total defined as:

TSL RSTD,Total=TBD

FFS: If the synchronization reference source changes at the measuring UE, while the UE is performing the SL RSTD measurement, then the UE shall continue performing the SL RSTD measurement after the synchronization reference source change.

[If the synchronization reference source changes at a UE transmitting SL-PRS used for the SL RSTD measurements, while the measuring UE is performing the SL RSTD measurement, then the measuring UE shall restart the SL RSTD measurement and meet the following requirement:

TSL RSTD,restart=K+1*TSL RSTD, Total ,

where K is the number of restarts due to the synchronization source change at a UE transmitting SL-PRS used for the SL RSTD measurements.]

## 12A.3	SL PRS-RSRP measurements

Editor’s note: SL-PRS based RSRP measurement requirements for UE in in-coverage scenario

### 12A.3.1	Introduction

The requirements in clause 12A.3 shall apply provided the UE has received [SLPP-RequestLocationInformation] message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL PRS-RSRP measurements defined in TS 38.215 [4] based on SL PRS.

### 12A.3.2	Requirements Applicability

The requirements in clause 12A.3 apply for periodic and triggered SL PRS-RSRP measurements, provided:

-	SL PRS-RSRP related side conditions given in clause TBD for FR1 are fulfilled, for a corresponding Band.

### 12A.3.3	Measurement Capability

UE SL PRS-RSRP measurement capability is as indicated by the UE in [sl-TDOA-ProvideCapabilities, sl-RTT-ProvideCapabilities, sl-AOA-ProvideCapabilities, or sl-TOA-ProvideCapabilities], according to TS 38.355 [37].

### 12A.3.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH or SCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH or SCCH. The delay uncertainty is: 2 x TTIDCCH/SCCH where TTIDCCH/SCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses TBD.

The SL PRS-RSRP measurements performed and reported according to this section shall meet the SL PRS-RSRP measurement accuracy requirements in clause TBD, for each measured SL-PRS resource.

### 12A.3.5 Measurements Period Requirements

When physical layer receives last of:

- [sl-TDOA-ProvideAssistanceData] message and [sl-TDOA-RequestLocationInformation] message, or

- [sl-AOA-ProvideAssistanceData] message and [sl-AOA-RequestLocationInformation] message, or

- [sl-TOA-ProvideAssistanceData] message and [sl-TOA-RequestLocationInformation] message, or

- [sl-RTT-ProvideAssistanceData] message and [sl-RTT-RequestLocationInformation] message,

from LMF or another UE via SLPP [37], the UE shall be able to perform at least [TBD] SL PRS-RSRP measurements, defined in TS 38.215 [4], during the measurement period TSL PRS-RSRP,Total defined as:

TSL PRS-RSRP,Total=TBD

If the synchronization reference source changes at the measuring UE, while the UE is performing the SL PRS-RSRP measurement, then the UE shall continue performing the SL PRS-RSRP measurement after the synchronization reference source change, while meeting the measurement period TSL PRS-RSRP,Total defined in this clause and the accuracy requirements in clause TBD.