3GPP TSG-RAN WG4 Meeting #109 	R4-2320462

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.7.2.2

Source: 	Ericsson

Title:	On L1-RSRP measurements

Document for:	Discussion

# Introduction

The agreements from RAN4#108bis are summarized in [22] and [23] for the 1st and 2nd threads, respectively.

# Discussion

## Fast beam sweeping applicability

In RAN4#108bis, faster beam sweeping capability applicability was discussed as below [22]:

Issue 1-1-1: General proposals:

- Proposals

  - Option 1: Fast beam sweeping is always enabled if UE supports the UE capability.

  - Option 2: Fast beam sweeping is always enabled if UE supports the UE capability and when UE indicates preference of multi-Rx operation

- Proposal 1: Fast beam sweeping is the UE capability, and it is always enabled in a UE supporting this capability as long as the UE is in multi-rx operation.

## Measurement period for GBBR L1-RSRP

In RAN4#108bis, the following was discussed [22]:

Issue 1-2-2 a: measurement period for SSB based L1-RSRP

- Way forward: Companies are requested to bring further analysis.

  - option 1:  N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise, N=8

  - Option 2: N = 8 + K, where K is the number of SSBs in each CMR set (Apple)

Issue 1-2-2 b: measurement period for CSI-RS based L1-RSRP

- Way forward: Companies are requested to bring further analysis.

  - Option 1: For CSI-RS based L1-RSRP measurements in FR2, the existing L1-RSRP measurement period is reused when configured for GBBR.

  - Option 2: N = ceil(maxNumberRxBeam / K) + 1, where K is the number of CSI-RSs in each CMR set

- Proposal 2: N = [reducedNumberRxBeam] for UE supporting faster beam sweeping under multi-rx operation; otherwise N=8.

- Proposal 3: A single value for the reducedNumberRxBeam is preferred, e.g., N=4.

## UE behaviour and transition period

In RAN4#108bis, the following was discussed [22]:

Issue 1-3-3: UE behaviour at transitions between single-RX and multi-RX operation modes

- Way forward: Companies are requested to bring further analysis to next meeting

When one or more conditions violated, the UE shall continue the measurement, and a transition period needs to be specified, at least for faster beam sweeping.

- Proposal 4: When one or more conditions violated, the UE shall continue the measurement.

- Proposal 5: Transition period is needed at least for faster beam sweeping, when going between single-rx and multi-rx operations.

- Proposal 6:  During the transition period, the more relaxed requirement (i.e., single-rx) apply.

- Proposal 7: Transition period length can be defined as one measurement period corresponding to the requirement applicable during the transition period.

- Proposal 8: No transition period is needed for scheduling restriction and scheduling availability enhancements.

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1: Fast beam sweeping is the UE capability, and it is always enabled in a UE supporting this capability as long as the UE is in multi-rx operation.

- Proposal 2: N = [reducedNumberRxBeam] for UE supporting faster beam sweeping under multi-rx operation; otherwise N=8.

- Proposal 3: A single value for the reducedNumberRxBeam is preferred, e.g., N=4.

- Proposal 4: When one or more conditions violated, the UE shall continue the measurement.

- Proposal 5: Transition period is needed at least for faster beam sweeping, when going between single-rx and multi-rx operations.

- Proposal 6:  During the transition period, the more relaxed requirement (i.e., single-rx) apply.

- Proposal 7: Transition period length can be defined as one measurement period corresponding to the requirement applicable during the transition period.

- Proposal 8: No transition period is needed for scheduling restriction and scheduling availability enhancements.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.

[16] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, May 2023.

[17] R4-2310047, WF on FR2_multiRx_part2, Ericsson, May 2023.

[18] R4-2309952, Topic summary for [106][209] FR2_multiRx _part1, vivo, May 2023.

[19] R4-2309953, Topic summary for [107][208] FR2_multiRx_part2, Ericsson, May 2023.

[20] R4-2314477, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Aug. 2023.

[21] R4-2314283, WF on FR2 multi-Rx chain DL reception (part2), Ericsson, Aug. 2023.

[22] R4-2317332, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Oct. 2023.

[23] R4-2317427, WF on FR2_multiRx_part2, Ericsson, Oct. 2023.