3GPP TSG-RAN WG4 Meeting #109	R4-2320471

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	[TEI16] RRM requirements impact due to new BS signalling for inter-band EN-DC with overlapping bands

Agenda item:	4.4

Document for:	Discussion

Introduction

At RAN4#108bis meeting, it was agreed to take the new BS signaling into RRM specification implementation as cited below [1]. In this paper, we are discussing what RRM requirements need to be adapted based on the new BS signaling.

Issue 1-1: New RRC signalling for NR-CA

- Agreement:

  - RAN4 shall take the newly introduced BS signaling into RRM specification implementation for UE supporting [intraBandNRCA-NonCollocated-r18].

  - Discuss the wording for the RRM CR based on RAN2 design on the BS signaling.

Issue 1-2: New RRC signalling for EN-DC

- Agreement:

  - For EN-DC, wait for the conclusion from RF session.

Discussion

We discussed the RRM requirements impact due to new BS signaling for intra-band non-collocated CA in [1]. We believe the same principle shall be applied to inter-band EN-DC with overlapping DL bands. Therefore, the MRTD/MTTD requirements and interruption requirements need to be adapted based on the new BS signaling.

Proposal 1: The MRTD/MTTD requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Proposal 2: Interruption requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Besides, RAN2 had further discussion on the new BS signaling in response to the LS and reached the following agreements:

RAN2#123bis Agreements:

- RAN2 agree to introduce a new RRC signaling to indicate capability restriction is type 1 or type 2 only for Rel-18 UE and not to introduce for Rel-17/16/15 UE

- Detailed statements capturing detailed RAN4 spec references in RAN2 specs without terminologies “Type1” and “Type2”.

- For EN-DC - Some Rel-18 UE support the new BS signaling and some Rel-18 UE do not support.  A new UE capability will be introduced for this

- Introduce a new UE capability which indicates the support of the new BS signaling per UE. This UE capability is only applicable to the UE indicating “interBandMRDC-WithOverlapDL-Bands-r16”.       The new capability is included in MRDC container

- Introduce a new Per BC UE capability to indicate the supporting TDD-TDD intra-band Non-Collocated NR-CA.

-    UE supporting the new UE capability supports both Type 1 and Type 2 capability requirements, while the UE not supporting the new UE capability supports Type 1 capability requirement only.

- Introduce two separate indications under IE ”CellGroupConfig” in TS38.331, one is for EN-DC operation and the other is for NR-CA

Based on the agreements, the UE need additional indicate the support of the new BS signaling for inter-band EN-DC with overlapping bands in order to operation Type 1 or Type 2 based on the new BS signaling.

Observation #1: From RAN2#123bis agreement, a UE needs to additionally indicate the support of the new BS signaling in order to operation Type 1 or Type 2 based on the new BS signaling.

Proposal 3: For inter-band EN-DC with overlapping bands, the RRM requirements need to be adapted based on the new BS signaling only if UE indicates both the capability interBandMRDC-WithOverlapDL-Bands-r16 and additionally [SupportNewBSsignaling].

- Conclusion

This contribution discusses the potential impact to RRM requirements for non-collocated FR1 intra-band scenarios due to the new BS signaling agreed in RAN#101 and further agreements in RAN2#123bis. The observations and proposals are summarized as below:

Proposal 1: The MRTD/MTTD requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Proposal 2: Interruption requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Observation #1: From RAN2#123bis agreement, a UE needs to additionally indicate the support of the new BS signaling in order to operation Type 1 or Type 2 based on the new BS signaling.

Proposal 3: For inter-band EN-DC with overlapping bands, the RRM requirements need to be adapted based on the new BS signaling only if UE indicates both the capability interBandMRDC-WithOverlapDL-Bands-r16 and additionally [SupportNewBSsignaling].

References

- RP-232692, LS on signaling support for intra-band non-collocated NR-CA,EN-DC, KDDI

- R4-2319013, RRM requirements impact due to new BS signaling in intra-band non-collocated CA, Nokia, Nokia Shanghai Bells