3GPP TSG-RAN WG4 Meeting #108bis	R4-2320496

Xiamen, China, 9th ~13th October 2023

Source: 	Apple

Title: 	further discussion on MTTD/MRTD requirement for FDD-FDD inter-band EN-DC/NE-DC with overlapping DL frequency

Agenda Item:	11.3.4

Document for:	Discussion

# Introduction

MRTD for FDD-FDD inter-band EN-DC:

- If asyncIntraBandENDC UE capability is supported, the current requirement for asynchronous EN-DC as specified in TS 38.133 Table 7.6.2-1 apply.

- If asyncIntraBandENDC UE capability is not supported,

  - The current requirement for synchronous inter-band EN-DC as specified in TS 38.133 Clause 7.6.2.1 should apply for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario).

  - The current requirement for synchronous intra-band collocated EN-DC as specified in TS 38.133 Clause 7.6.3 should apply for UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

In the last meeting, MRTD/MTTD requirement applicability for interBandMRDC-WithOverlapDL-Bands-r16 was discussed and the following agreement on FDD MRTD and MTTD have been captured in the LS to RAN2 [1].

MTTD for FDD-FDD inter-band EN-DC:

- If asyncIntraBandENDC UE capability is supported, the current requirement for asynchronous EN-DC as specified in TS 38.133 Table 7.5.2-1 apply.

- If asyncIntraBandENDC UE capability is not supported,

  - The current requirement for synchronous inter-band EN-DC as specified in TS 38.133 Clause 7.5.2.1 should apply for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to non-collocated scenario).

  - The current requirement for synchronous intra-band collocated EN-DC as specified in TS 38.133 Clause 7.5.3 should apply for UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 (corresponding to collocated scenario).

While it was pointed out offline that this agreement may need some further discussion. This contribution will discuss this issue further.

# Discussion

In the last meeting, the above conclusion was motivated by the fact that there is an inter-band (NG)EN-DC combination scenario in the ayncIntraBandENDC signaling. And it indeed does not clearly preclude FDD-FDD inter-band EN-DC with DL overlapping frequency from the signaling description. Then it was understood that FDD-FDD inter-band EN-DC with overlapping DL frequency case is also included in this signaling and the async MRTD/MTTD requirement in 38.133 is optional for FDD-FDD inter-band EN-DC/NE-DC with overlapping DL frequency.

After last RAN4 meeting, it was pointed out that for inter-band EN-DC, all UEs support asynchronous DC between LTE and NR for inter-band LTE-NR in Rel-15 according to [2]. The 4th scenario of “Inter-band (NG)EN-DC combination” was added later after Rel-15. Although the description used “where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band”, we think here the important aspect is to address that there is overlapping between 2 bands. It is suggested the group to confirm whether the 4th scenario includes FDD-FDD inter-band EN-DC with overlapping DL frequency or it is only limited to TDD-TDD inter-band EN-DC with overlapping DL frequency.

Proposal 1: RAN4 to confirm whether the 4th scenario in asyncIntraBandENDC also include FDD-FDD inter-band EN-DC with overlapping DL frequency or not.

- If yes, confirm the agreement in previous meeting and agree the endorsed CR in [3] from the last meeting.

- If no, update the endorsed CR in [3] to reflect that async MRTD/MTTD requirements are mandatory for FDD-FDD inter-band EN-DC with overlapping DL frequency. And send updated LS on asyncIntraBandENDC accordingly.

# Summary

This contribution further discussed the MRTD/MTTD requirement applicability and the following proposal is provided:

Proposal 1: RAN4 to confirm whether the 4th scenario in asyncIntraBandENDC also include FDD-FDD inter-band EN-DC with overlapping DL frequency or not.

- If yes, confirm the agreement in previous meeting and agree the endorsed CR in [3] from the last meeting.

- If no, update the endorsed CR in [3] to reflect that async MRTD/MTTD requirements are mandatory for FDD-FDD inter-band EN-DC with overlapping DL frequency. And send updated LS on asyncIntraBandENDC accordingly.

# References

[1] R4-2317401, Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306, Apple, RAN4#108bis.

[2] R4-1809935, Clarification on Sync/Async EN-DC scenarios, Samsung

[3] R4-2318627, On MRTD/MTTD requirement for inter-band non-collocated EN-DC/NE-DC (R16)