3GPP TSG-RAN WG4 Meeting #109	R4-2320500

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.113 | CR | 0130 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.113 on framework for EMC-specific manufacturer's declarations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Based on the Draft CR Endorsed in R4-2316932 during RAN4#108bis meeting (Xiamen), a formal CR is provided for the manufacturer declaration framework. This is done separately from the EMC simplifications changes, to separate the legacy maintenance aspects from the Rel-18 WI outcomes on the EMC testing simplifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3.3: add new abbreviation of DEMC. - 4.6, 5.1: new section for EMC-specific manufacturer declarations, aligning the approach with RF test specifications. - 6.2: adding more detailed cross-references to the declarations in RF test specs. - 9.4, 9.5: new cross-references added for EMC-specific declarations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No framework for EMC-specific manufacturer’s declarations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.6, 5.1, 6.2, 9.4, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AC	Alternating Current

AMN	Artificial Mains Network

ARFCN	Absolute Radio Frequency Channel Number

BC	Band Category

BER	Bit Error Ratio

BLER	Block Error Ratio

CDN	Coupling/Decoupling Network

CS	Capability Set

DC	Direct Current

DEMC	EMC-specific manufacturer declaration

E-UTRA	Evolved Universal Terrestrial Radio Access

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

EMC	Electromagnetic Compatibility

EPC	Evolved Packet Core

ESD	Electrostatic discharge

EUT	Equipment Under Test

FR	Frequency Range

FRC	Fixed Reference Channel

MB-MSR	Multi-Band Multi-Standard Radio

MSR	Multi-Standard Radio

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NTC	Test Configuration for Non-contiguous operation

RAT	Radio Access Technology

RF	Radio frequency

rms	root mean square

TC	Test Configuration

UARFCN	UTRA Absolute Radio Frequency Channel Number

UTRA	Universal Terrestrial Radio Access

# 4	Test conditions

---------- end of changes----------

---------- start of changes----------

## 4.5	BS test configurations

The present clause defines the BS test configurations that shall be used for demonstrating conformance. This is specified in table 4.5-1, 4.5-1a, 4.5-1b, and 4.5-1c for multi-RAT capable MSR Base Stations, in table 4.5-2 for single-RAT capable BS and in table 4.5-3 for multi-band capable BS. For other BS supporting more than one RAT (other than MSR BS), table 4.5-2 applies separately for each RAT supported.

The test configurations apply according to the declared RAT Capability Set (CS) of the MSR Base Station according to subclause 4.7 of TS 37.141 [11] and the Band Category of the declared operating band (BC1, BC2 or BC3), as listed in the heading of each table.

The test configurations (TCx) are defined in TS 37.141 [11], subclause 4.8.

For a BS declared to be capable of contiguous operation only, the test configuration(s) in tables 4.5-1 and 4.5-2 denoted by a "C" shall be used for testing.

For a BS declared to be capable of contiguous and non-contiguous operation and where the parameters in the manufacture's declaration according to subclause 4.7.2 of TS 37.141 [11] are identical for contiguous and non-contiguous operation, the test configurations denoted by "CNC" shall be used.

For a BS declared to be capable of contiguous and non-contiguous operation and where the parameters in the manufacture's declaration according to subclause 4.7.2 of TS 37.141 [11] are not identical for contiguous and non-contiguous operation, the test configurations denoted by "C/NC" shall be used for testing.

For a BS declared to support NB-IoT operating in-band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NI" shall be used for testing.

For a BS declared to support NB-IoT operating in guard band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NG" shall be used for testing.

For a BS declared to support NB-IoT operating in guard band and in-band, the test configuration(s) in table 4.5-1 and 4.5-2 denoted by "NG" or/and "NI" shall be used for testing.

For a BS declared to support NB-IoT standalone, the test configuration(s) in table 4.5-1a, 4.5-1b and 4.5-2 and entries that refer to single-RAT specifications shall be used for testing.

For immunity tests:

-	The communication link for the RAT(s) listed in the table shall be established according to subclause 4.2.

-	Tests for ports relating to the RAT(s) supported shall be performed according to subclause 4.1.

Table 4.5-1: Test configurations for single-band Multi-RAT capable MSR BS (CS3-6)

| Capability Set | UTRA + E-UTRA. NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 3) |  |  | GSM+ UTRA (CS 4) | GSM + E-UTRA, NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS 5) | GSM + UTRA + E-UTRA, (CS 6) | GSM+UTRA/ E-UTRA, UTRA+ E-UTRA, NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) (CS7) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC2 | BC2 | BC2 |
| Emission tests | C: TC3a CNC: NTC3 C/NC: TC3a and NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a and NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC4a CNC: NTC4a C/NC: TC4a and NTC4a | C: TC4b CNC: NTC4b C/NC: TC4b and NTC4b NI: TC15 NG: TC18 | C: TC4c CNC: NTC4c C/NC: TC4c and NTC4c | C: (TC4a, TC3a) (Note 4), TC4b CNC: (NTC4a, NTC3) (Note 4), NTC4b C/NC: (TC4a, NTC4a,TC3a, NTC3) (Note 4), TC4b,NTC4b NI: TC15,(TC16) (Note 4) NG: TC18, (TC19) (Note 4) |
| Immunity tests | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3a CNC: NTC3 C/NC: TC3a, NTC3 NI: TC16 NG: TC19 | C: TC3b NI: TC16 NG: TC19 | C: TC5a CNC: NTC5a C/NC: TC5a, NTC5a | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 | C: TC5b CNC: NTC5c, C/NC: TC5b, NTC5c | C: TC5b CNC: NTC5b C/NC: TC5b, NTC5b NI: TC15 NG: TC18 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. NOTE 4: For Band 3, the test configuration is only applicable if UTRA is declared to be supported in Band 3. |  |  |  |  |  |  |  |

Table 4.5-1a: Test configurations for single-band Multi-RAT capable MSR BS (CS9-13)

| Capability Set | GSM+NB-IoT standalone (CS 9) | UTRA + NB-IoT standalone (CS 10) |  | E-UTRA + NB-IoT standalone (CS 11) |  |  | GSM+UTRA+NB-IoT standalone (CS 12) | GSM+ E-UTRA+ NB-IoT standalone (CS 13) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC2 | BC1 | BC2 | BC1 | BC2 | BC3 | BC2 | BC2 |
| Emission tests | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |
| Immunity tests | TC9 | TC10 | TC10 | TC11 | TC11 | TC11 | TC12 | TC13 |

Table 4.5-1b: Test configurations for single-band Multi-RAT capable MSR BS (CS14-17)

| Capability Set | UTRA + E-UTRA + NB-IoT standalone (CS 14) |  |  | GSM + UTRA + E-UTRA + NB-IoT standalone (CS 15) | NR + E-UTRA NB-IoT in-band(Note2) NB-IoT guard band(Note3) (CS 16) |  | NR + NB-IoT standalone + E-UTRA NB-IoT in-band(Note2) NB-IoT guard band(Note3) (CS 17) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC2 | BC1 and BC2 | BC3 | BC1 and BC2 | BC3 |
| Emission tests | TC14 | TC14 | TC14 | TC13, (TC12，TC13) (NOTE4) | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 |
| Immunity tests | TC14 | TC14 | TC14 | TC13 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C, NI, NG: TC21 CNC, NCNI, NCNG: NTC21 C/NC, C/NCNI, C/NCNG: NTC21, TC21 | C: TC22 NI: TC22 NG: TC22 | C: TC22 NI: TC22 NG: TC22 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. NOTE 4: For Band 3, the test configuration is only applicable if UTRA is declared to be supported in Band 3. |  |  |  |  |  |  |  |  |

Table 4.5-1c: Test configurations for single-band Multi-RAT capable MSR BS (CS18-19)

| Capability Set | GSM + NR + E-UTRA NB-IoT in-band (NOTE 2) NB-IoT guard band (NOTE3) (CS 18) | UTRA + NR + E-UTRA NB-IoT in-band (NOTE 2) NB-IoT guard band (NOTE 3) (CS19) |
| --- | --- | --- |
| BS test case | BC2 | BC2 |
| Emission tests | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| Immunity tests | C, NI, NG: TC21a CNC, NCNI, NCNG: NTC21a C/NC, C/NCNI, C/NCNG: NTC21a, TC21a | C, NI, NG: TC21b CNC, NCNI, NCNG: NTC21b C/NC, C/NCNI, C/NCNG: NTC21b, TC21b |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. |  |  |

Table 4.5-2: Test configurations for single-band Single-RAT multi-carrier capable BS

| Capability Set | UTRA (MC) capable BS (CS1) |  |  | E-UTRA (MC) capable BS (CS2) NB-IoT in-band, NB-IoT guard band (Note 2, Note 3) |  |  | NB-IoT (MC) capable BS (CS8) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS test case | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 | BC1 | BC2 | BC3 |
| Emission tests | C: TC1a CNC: NTC1a C/NC: TC1a and NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a and NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2 and NTC2 NI: TC17 NG: TC20 | TC8 | TC8 | TC8 |
| Immunity tests | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1a CNC: NTC1a C/NC: TC1a, NTC1a | C: TC1b | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | C: TC2 CNC: NTC2 C/NC: TC2, NTC2 NI: TC17 NG: TC20 | TC8 | TC8 | TC8 |
| NOTE 1: Void NOTE 2: The support of NB-IoT in-band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NI" shall not be used for testing. NOTE 3: The support of NB-IoT guard band operation is optional and declared by the manufacturer. If not supported, the test configurations denoted by "NG" shall not be used for testing. |  |  |  |  |  |  |  |  |  |

Table 4.5-3: Test configurations for multi-band capable BS (CS1-CS7, CS16 and CS18-CS19)

| Capability Set | Multi-band testing |  |
| --- | --- | --- |
| BS test case | BC1/BC2 | BC3 |
| Emission tests | TC7b | TC7b |
| Immunity tests | TC7b | TC7b |

## 4.6	Manufacturer declarations

The following EMC-specific manufacturer’s declarations listed in table 4.6-1 are required to be provided by the manufacturer for MSR BS requirements testing.

NOTE:	The below listed manufacturer’s declarations are supplementary to declarations covered in TS 37.141 [11].

Table 4.6-1 EMC-specific manufacturer declarations

| Declaration identifier | Declaration | Description |  |
| --- | --- | --- | --- |
| DEMC.1 | Declaration of ports intended to be used with cables less than 3 m | Declaration of any ports intended to be used with cables less than 3 m. |  |
| DEMC.2 | Common and/or RAT-specific active RF components | Declaration of common and/or RAT-specific active RF components and other HW blocks for a communication link in MSR BS or other BS supporting more than one RAT. |
| DEMC.3 | Common and/or band-specific active RF components | Declaration of common and/or band-specific active RF components and other HW blocks for a communication link in BS capable of multi-band operation. |

# 5	Performance assessment

## 5.1	General

The following information shall be recorded in or annexed to the test report:

-	the primary functions of the radio equipment to be tested during and after the EMC testing;

-	the intended functions of the radio equipment which shall be in accordance with the documentation accompanying the equipment;

-	the method to be used to verify that a communications link is established and maintained;

-	the user-control functions and stored data that are required for normal operation and the method to be used to assess whether these have been lost after EMC stress;

-	the ancillary equipment to be combined with the radio equipment for testing (where applicable);

-	the information about ancillary equipment intended to be used with the radio equipment;

-	information about the common and/or RAT-specific active RF components and other HW blocks for a communication link in MSR BS or other BS supporting more than one RAT (see declaration DEMC.2);

-	information about the common and/or band-specific active RF components and other HW blocks for a communication link in BS capable of multi-band operation (see declaration DEMC.3);

-	an exhaustive list of ports, classified as either power or signal/control. Power ports shall further be classified as AC or DC power.

Performance assessment of a BS with multiple enclosures may be done separately for the BS part with the Radio digital unit and the Radio unit respectively, according to the manufacturer's choice.

A communication link used by more than one RAT or more than one operating band, shall be assessed on all RATs and operating bands. Communication link(s) and/or radio performance parameters for the RATs and operating bands can during the test be assessed simultaneously or separately for each RAT and band, depending on the test environment capability.

## 5.2	Assessment of performance in Downlink

---------- end of changes----------

---------- start of changes----------

## 6.2	Performance criteria for transient phenomena for BS

At the conclusion of each exposure the EUT shall operate with no user noticeable loss of the communication link.

At the conclusion of the total test comprising the series of individual exposures the EUT shall operate as intended with no loss of user control functions or stored data, as specified by the manufacturer, and the communication link shall have been maintained.

The below characteristics should be chosen based on manufacture declaration as defined in related single RAT BS RF testing specification, and are defined as:

-	The E-UTRA channel bandwidth and bearer information data rate are defined in table 6.1.1-1, based on declaration defined in TS 36.141 [9].

-	The UTRA bearer information data rate are defined in table 6.1.2-1, based on declaration defined in TS 25.141 [7].

-	The GSM/EDGE the test BER assessment are defined in clause 6.1.3.1 for downlink and clasue 6.1.3.2 for uplink, while the criteria is defined in clause 6.2, based on declaration defined in TS 51.021 [10].

-	The NB-IoT sub-carrier spacing and reference measurement channel are defined in table 6.1.4-1, based on declaration defined in TS 36.141 [9].

-	The NR channel bandwidth, sub-carrier spacing and bearer information data rate are defined in table 6.1.5-1, based on declaration defined in TS 38.141-1 [36], and/or TS 38.141-2 [37].

## 6.3	Performance criteria for continuous phenomena for Ancillary equipment

---------- end of changes----------

---------- start of changes----------

## 9.4	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.6-1) that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.4.1	Definition

---------- end of changes----------

---------- start of changes----------

## 9.5	RF common mode (0,15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in DEMC.1 (see table 4.6-1) that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the clamp injection method, where appropriate, see IEC 61000-4-6 [26].

### 9.5.1	Definition

---------- end of changes----------