# 3GPP TSG-RAN WG4 Meeting # 109	R4-2320517

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1089 | rev | - | Current version: | 17.11.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-3 (Rel-17): Correction of an invalid channel bandwidth in 3DL/2UL inter-band reference sensitivity testing for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Ericsson, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_3BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An invalid 5MHz channel bandwidth of either band n77 or n78 is used in a few MSD test points due to dual uplink operation for EN-DC in NR FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the invalid 5MHz to the valid 10MHz channel bandwidth for the band n77 and n78 in Table 7.3B.2.3.5.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalided channel bandwidth will be applied in the reference sensitivity testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### << Start of change >>

###### 7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  |  | UL LCRB | DL Fc (MHz) |  |  |  |  | MSD (dB) |  |  |  | IMD order |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1818.5 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2139 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 5 |  |  |  | IMD4 |  |
|  | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 675 |  | 5 |  |  | 25 | 629 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n3A-n28A | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1818.5 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n3A-n41A | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 5 |  |  | N/A | 2507.5 |  |  |  |  | 5.0 |  |  |  | IMD5 |  |
| DC_1A_n3A-n75A | n75 |  | N/A |  | 5 |  |  | N/A | 1480 |  |  |  |  | 15.2 |  |  |  | IMD34 |  |
|  | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1960 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n3A-n79A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4950 |  |  |  |  | 4.7 |  |  |  | IMD5 |  |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 718 |  | 5 |  |  | 25 | 773 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2653 |  |  |  |  | 30.0 |  |  |  | IMD2 |  |
| DC_1A-7A_n40A | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2630 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | n40 |  | 2390 |  | 5 |  |  | 25 | 2390 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
|  | 7 |  | 2530 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2310 |  | 5 |  |  | 25 | 2310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n77A | 1 |  | 1955 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n77(2A) | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3410 |  |  |  |  | 1.5 |  |  |  | IMD5 |  |
| DC_1A_n8A-n77A | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n77(2A) | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3960 |  |  |  |  | 8.8 |  |  |  | IMD3 |  |
| DC_1A_n8A-n77A | 1 |  | 1955 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n77(2A) | n77 |  | 3410 |  | 10 |  |  | 50 | 3410 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | N/A |  | 5 |  |  | N/A | 955 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_1A-8A_n78A | 1 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n77(2A) | 8 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD5 |  |
|  | n78 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-3A_n77A DC_1A-3A_n77(2A) DC_1A-3A_n77(3A) DC_1A-3C_n77A DC_1A-3A_n77C DC_1A-3C_n77(2A) | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1807.5 |  |  |  |  | 31.5 |  |  |  | IMD2 |  |
|  | n77 |  | 3757.5 |  | 10 |  |  | 50 | 3757.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1870 |  |  |  |  | 8.5 |  |  |  | IMD4 |  |
|  | n77 |  | 3980 |  | 10 |  |  | 50 | 3980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 31.0 |  |  |  | IMD2 |  |
|  | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3915 |  | 10 |  |  | 50 | 3915 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78C DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1807.5 |  |  |  |  | 31.2 |  |  |  | IMD2 |  |
|  | n78 |  | 3757.5 |  | 10 |  |  | 50 | 3757.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2125 |  |  |  |  | 2.8 |  |  |  | IMD5 |  |
|  | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3725 |  | 10 |  |  | 50 | 3725 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3700 |  |  |  |  | 28.4 |  |  |  | IMD2 |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3360 |  |  |  |  | 11.2 |  |  |  | IMD4 |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1807.5 |  |  |  |  | 31.5 |  |  |  | IMD2 |  |
|  | n77 |  | 3757.5 |  | 10 |  |  | 50 | 3757.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1870 |  |  |  |  | 8.5 |  |  |  | IMD4 |  |
|  | n77 |  | 3980 |  | 10 |  |  | 50 | 3980 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n3A-n78A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3700 |  |  |  |  | 28.4 |  |  |  | IMD2 |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1830 |  |  |  |  | 27.9 |  |  |  | IMD2 |  |
|  | n78 |  | 3780 |  | 10 |  |  | 50 | 3780 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) | 1 |  | N/A |  | 5 |  |  | N/A | 2122 |  |  |  |  | 18.1 |  |  |  | IMD3 |  |
|  | 5 |  | 829 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3780 |  | 10 |  |  | 50 | 3780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 3.1 |  |  |  | IMD5 |  |
|  | n77 |  | 3405 |  | 10 |  |  | 50 | 3405 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-5A_n78A DC_1A-5A_n78C | 1 |  | N/A |  | 5 |  |  | N/A | 2122 |  |  |  |  | 18.1 |  |  |  | IMD3 |  |
|  | 5 |  | 829 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3780 |  | 10 |  |  | 50 | 3780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 3.1 |  |  |  | IMD5 |  |
|  | n78 |  | 3405 |  | 10 |  |  | 50 | 3405 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2627.5 |  |  |  |  | 9.1 |  |  |  | IMD44 |  |
|  | n77 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | 7 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3580 |  | 10 |  |  | 50 | 3580 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-1A-7A_n78A DC_1A-7A-7A_n78C | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2627.5 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | 7 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3580 |  | 10 |  |  | 50 | 3580 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n7A-n78A DC_1A_n7B-n78A DC_1A_n7A-n78(2A) | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | N/A |  | 5 |  |  | N/A | 2627.5 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 10.1 |  |  |  | IMD4 |  |
| DC_1A-3A_n79A | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 3.6 |  |  |  | IMD5 |  |
|  | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4860 |  | 40 |  |  | 216 | 4860 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-5A_n79A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 882.5 |  |  |  |  | 18.3 |  |  |  | IMD3 |  |
|  | n79 |  | 4782.5 |  | 40 |  |  | 216 | 4782.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 882.5 |  |  |  |  | 8.9 |  |  |  | IMD4 |  |
|  | n79 |  | 4907.5 |  | 40 |  |  | 216 | 4907.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 8.1 |  |  |  | IMD4 |  |
|  | 5 |  | 837.5 |  | 5 |  |  | 25 | 882.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4652.5 |  | 40 |  |  | 216 | 4652.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n28A | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 950 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_1A-8A_n40A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 8.0 |  |  |  | IMD4 |  |
|  | n40 |  | 2395 |  | 5 |  |  | 25 | 2395 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2135 |  |  |  |  | 5.3 |  |  |  | IMD5 |  |
|  | 8 |  | 885 |  | 5 |  |  | 25 | 930 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2395 |  | 5 |  |  | 25 | 2395 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n77A | 1 |  | 1955 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 |  | 3410 |  | 10 |  |  | 50 | 3410 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 955 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_1A-8A_n77A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-8A_n77(2A) DC_1A-8A_n77(3A) | n77 |  | 3960 |  | 10 |  |  | 50 | 3960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 14.4 |  |  |  | IMD3 |  |
| DC_1A-8A_n79A | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4815 |  | 40 |  |  | 216 | 4815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 945 |  |  |  |  | 15.8 |  |  |  | IMD3 |  |
| DC_1A-8A_n79A | 8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4845 |  | 40 |  |  | 216 | 4845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2145 |  |  |  |  | 8.2 |  |  |  | IMD4 |  |
| DC_1A_n8A-n40A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 8.0 |  |  |  | IMD4 |  |
|  | n40 |  | 2395 |  | 5 |  |  | 25 | 2395 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n8A-n78A | 1 |  | 1945 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3745 |  |  |  |  | 14.9 |  |  |  | IMD3 |  |
|  | 1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
|  | n78 |  | 3380 |  | 10 |  |  | 50 | 3330 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-11A_n3A | 1 |  | 1960 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1480 |  |  |  |  | 15.2 |  |  |  | IMD3 |  |
| DC_1A-11A_n28A | 11 |  | 1440 |  | 5 |  |  | 25 | 1488 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 710 |  | 5 |  |  | 25 | 765 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 28.3 |  |  |  | IMD21 |  |
| DC_1A-11A_n41A | 11 |  | 1442 |  | 5 |  |  | 25 | 1490 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2520 |  | 10 |  |  | 50 | 2520 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2156 |  |  |  |  | 10.2 |  |  |  | IMD4 |  |
|  | 1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2685 |  | 10 |  |  | 50 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1490 |  |  |  |  | 10.6 |  |  |  | IMD4 |  |
| DC_1A-11A_n77A DC_1A-11A_n77(2A) DC_1A-11A_n77(3A) | 1 |  | 1955 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1486 |  |  |  |  | 31.4 |  |  |  | IMD2 |  |
|  | n77 |  | 3441 |  | 10 |  |  | 50 | 3441 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
|  | 11 |  | 1438 |  | 5 |  |  | 25 | 1486 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3578 |  | 10 |  |  | 50 | 3578 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-11A_n78A DC_1A-11A_n78(2A) | 1 |  | 1955 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1486 |  |  |  |  | 31.4 |  |  |  | IMD2 |  |
|  | n78 |  | 3441 |  | 10 |  |  | 50 | 3441 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
|  | 11 |  | 1438 |  | 5 |  |  | 25 | 1486 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3578 |  | 10 |  |  | 50 | 3578 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-11A_n79A | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1483 |  |  |  |  | 10.2 |  |  |  | IMD4 |  |
|  | n79 |  | 4427 |  | 40 |  |  | 216 | 4427 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2118 |  |  |  |  | 15.6 |  |  |  | IMD3 |  |
|  | 11 |  | 1431 |  | 5 |  |  | 25 | 1479 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4980 |  | 40 |  |  | 216 | 4980 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD5 |  |
|  | n77 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
|  | 18 |  | 825 |  | 5 |  |  | 25 | 870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3770 |  | 10 |  |  | 50 | 3770 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD5 |  |
|  | n78 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
|  | 18 |  | 819 |  | 5 |  |  | 25 | 864 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3758 |  | 10 |  |  | 50 | 3758 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-18A_n79A | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | N/A |  | 5 |  |  | N/A | 867.5 |  |  |  |  | 18.3 |  |  |  | IMD3 |  |
|  | n79 |  | 4737.5 |  | 40 |  |  | 216 | 4737.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | N/A |  | 5 |  |  | N/A | 865 |  |  |  |  | 8.9 |  |  |  | IMD4 |  |
|  | n79 |  | 4925 |  | 40 |  |  | 216 | 4925 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2125 |  |  |  |  | 8.1 |  |  |  | IMD4 |  |
|  | 18 |  | 822.5 |  | 5 |  |  | 25 | 867.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4592.5 |  | 40 |  |  | 216 | 4592.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 |  | N/A |  | 5 |  |  | N/A | 2130 |  |  |  |  | 17.8 |  |  |  | IMD3 |  |
|  | 19 |  | 832.5 |  | 5 |  |  | 25 | 877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 5.1 |  |  |  | IMD5 |  |
|  | n77, n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-19A_n79A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  | N/A |  | 5 |  |  | N/A | 882.5 |  |  |  |  | 18.3 |  |  |  | IMD3 |  |
|  | n79 |  | 4782.5 |  | 40 |  |  | 216 | 4782.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 8.1 |  |  |  | IMD4 |  |
|  | 19 |  | 837.5 |  | 5 |  |  | 25 | 882.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4652.5 |  | 40 |  |  | 216 | 4652.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n28A-n41A | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 718 |  | 5 |  |  | 25 | 773 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 10 |  |  | N/A | 2653 |  |  |  |  | 30.1 |  |  |  | IMD2 |  |
|  | 1 |  | 1923 |  | 5 |  |  | 25 | 2113 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 762 |  |  |  |  | 29.3 |  |  |  | IMD2 |  |
|  | n41 |  | 2685 |  | 10 |  |  | 50 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 10 |  |  | N/A | 785 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n41 |  | 2510 |  | 10 |  |  | 50 | 2510 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-20A_n7A | 1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 800 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n7 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-20A_n8A | 1 |  | 1925 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 805 |  |  |  |  | 11.5 |  |  |  | IMD4 |  |
|  | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-20A_n38A | 1 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD5 |  |
|  | n38 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-20A_n78A | 1 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 20.3 |  |  |  | IMD3 |  |
| DC_1A-20A_n78(2A) | 20 |  | 835 |  | 5 |  |  | 25 | 794 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3790 |  | 10 |  |  | 50 | 3790 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 810 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
|  | n78 |  | 3330 |  | 10 |  |  | 50 | 3330 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-21A_n28A10 | 1 |  | N/A |  | 5 |  |  | N/A | 2165.3 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 735.5 |  | 5 |  |  | 25 | 790.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 |  | N/A |  | 5 |  |  | N/A | 2154.6 |  |  |  |  | 30.6 |  |  |  | IMD2 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | 3605 |  | 10 |  |  | 50 | 3605 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2154.6 |  |  |  |  | 3.6 |  |  |  | IMD5 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | 3647 |  | 10 |  |  | 50 | 3647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  | N/A |  | 5 |  |  | N/A | 1500 |  |  |  |  | 31.5 |  |  |  | IMD2 |  |
|  | n77, n78 |  | 3450 |  | 10 |  |  | 50 | 3450 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  | N/A |  | 5 |  |  | N/A | 1500 |  |  |  |  | 2.9 |  |  |  | IMD5 |  |
|  | n77, n78 |  | 3675 |  | 10 |  |  | 50 | 3675 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-21A_n79A | 1 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD4 |  |
|  | n79 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n3A | 1 |  | N/A |  | 5 |  |  | N/A | 2139 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 10 |  |  | N/A | 785 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n7 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n40A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 780 |  |  |  |  | 8.9 |  |  |  | IMD4 |  |
|  | n40 |  | 2340 |  | 5 |  |  | 25 | 2340 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n77A | 1 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | 28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3630 |  | 10 |  |  | 50 | 3630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 794 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
|  | n77 |  | 3352 |  | 10 |  |  | 50 | 3352 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n78A | 1 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | 28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3630 |  | 10 |  |  | 50 | 3630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 794 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
|  | n78 |  | 3352 |  | 10 |  |  | 50 | 3352 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-28A_n79A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 788 |  |  |  |  | 15.2 |  |  |  | IMD3 |  |
|  | n79 |  | 4648 |  | 40 |  |  | 216 | 4648 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1925 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 795 |  |  |  |  | 10.0 |  |  |  | IMD4 |  |
|  | n79 |  | 4980 |  | 40 |  |  | 216 | 4980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2167.5 |  |  |  |  | 1.2 |  |  |  | IMD4 |  |
|  | 28 |  | 745.5 |  | 5 |  |  | 25 | 800.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 | 4420 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2125 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | 28 |  | 718 |  | 5 |  |  | 25 | 773 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4807 |  | 40 |  |  | 216 | 4807 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n28A-n40A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | N/A |  | 5 |  |  | N/A | 2374 |  |  |  |  | 10.1 |  |  |  | IMD4 |  |
|  | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 768 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | n40 |  | 2314 |  | 5 |  |  | 25 | 2314 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n28A-n78A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 733 |  | 5 |  |  | 25 | 788 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3416 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3320 |  | 10 |  |  | 50 | 3320 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 790 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_1A_n28A-n79A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 788 |  |  |  |  | 15.2 |  |  |  | IMD39 |  |
|  | n79 |  | 4648 |  | 40 |  |  | 216 | 4648 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4630 |  |  |  |  | 14.9 |  |  |  | IMD34 |  |
| DC_1A-32A_n3A | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1480 |  |  |  |  | 15.2 |  |  |  | IMD34 |  |
|  | 1 |  | 1960 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-32A_n78A DC_1A-32A_n78C DC_1A-32A_n78(2A) | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1470 |  |  |  |  | 31.8 |  |  |  | IMD2 |  |
|  | n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1470 |  |  |  |  | 0 |  |  |  | IMD5 |  |
|  | n78 |  | 3630 |  | 10 |  |  | 50 | 3630 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-38A_n78A DC_1A-38A_n78(2A) | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 38 |  | N/A |  | 5 |  |  | N/A | 2590 |  |  |  |  | 12.7 |  |  |  | IMD4 |  |
|  | n78 |  | 3320 |  | 10 |  |  | 50 | 3320 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n38A-n78A | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | N/A |  | 10 |  |  | N/A | 2590 |  |  |  |  | 12.7 |  |  |  | IMD4 |  |
|  | n78 |  | 3320 |  | 10 |  |  | 50 | 3320 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2340 |  |  |  |  | 10.6 |  |  |  | IMD4 |  |
|  | n78 |  | 3450 |  | 10 |  |  | 50 | 3450 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | 40 |  | 2360 |  | 5 |  |  | 25 | 2360 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3430 |  | 10 |  |  | 50 | 3430 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2340 |  | 5 |  |  | 25 | 2340 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3450 |  |  |  |  | 9.8 |  |  |  | IMD4 |  |
|  | 1 |  | 1960 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | N/A |  | 5 |  |  | N/A | 2360 |  |  |  |  | 10.6 |  |  |  | IMD4 |  |
|  | n78 |  | 3520 |  | 10 |  |  | 50 | 3520 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2507.5 |  |  |  |  | 5.0 |  |  |  | IMD5 |  |
|  | n3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-41A_n28A | 1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 10 |  |  | N/A | 2653 |  |  |  |  | 30 |  |  |  | IMD2 |  |
|  | n28 |  | 718 |  | 5 |  |  | 25 | 773 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2510 |  |  |  |  | N/A |  |  |  | IMD4 |  |
|  | n77 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 9.3 |  |  |  | IMD4 |  |
|  | 41 |  | 2640 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3710 |  | 10 |  |  | 50 | 3710 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | 11.0 |  |  |  | N/A |  |
|  | 41 |  | 2510 |  | 5 |  |  | 25 | 2510 |  |  |  |  | N/A |  |  |  | IMD5 |  |
|  | n77 |  | 4150 |  | 10 |  |  | 50 | 4150 |  |  |  |  | N/A |  |  |  |  |  |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 9.3 |  |  |  | IMD4 |  |
|  | 41 |  | 2640 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3710 |  | 10 |  |  | 50 | 3710 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2515 |  |  |  |  | 12 |  |  |  | IMD4 |  |
|  | n78 |  | 3410 |  | 10 |  |  | 50 | 3410 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n41A-n77A DC_1A_n41A-n77(2A) DC_1A_n41A-n78A DC_1A_n41A-n78(2A) | 1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 10 |  |  | N/A | 2515 |  |  |  |  | 11.5 |  |  |  | IMD4 |  |
|  | n78 |  | 3410 |  | 10 |  |  | 50 | 3410 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2650 |  | 10 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3330 |  |  |  |  | 19.6 |  |  |  | IMD3 |  |
| DC_1A-41A_n79A | 1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2530 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | n79 |  | 4500 |  | 40 |  |  | 216 | 4500 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A-42A_n3A | 1 |  | 1922.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1782.5 |  | 5 |  |  | 25 | 1877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  | N/A |  | 5 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_1A-42A_n28A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 733 |  | 5 |  |  | 25 | 788 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  | N/A |  | 5 |  |  | N/A | 3416 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
| DC_1A-42A_n28A | 42 |  | 3580 |  | 5 |  |  | 25 | 3580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 723 |  | 5 |  |  | 25 | 778 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2134 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
| DC_1A-42A_n79A | 1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 | 4420 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  | N/A |  | 5 |  |  | N/A | 3490 |  |  |  |  | 4.8 |  |  |  | IMD5 |  |
|  | 42 |  | 3402.5 |  | 5 |  |  | 25 | 3402.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4640 |  | 40 |  |  | 216 | 4640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2165 |  |  |  |  | 15.5 |  |  |  | IMD3 |  |
|  | 42 |  | 3450 |  | 5 |  |  | 25 | 3450 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4520 |  | 40 |  |  | 216 | 4520 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 9.3 |  |  |  | IMD4 |  |
| DC_1A_SUL_n77A-n80A | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | n80 |  | 1760 |  | 5 |  |  | 25 | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_SUL_n77A-n80A | 1 |  | 1922.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n80 |  | 1782.5 |  | 5 |  |  | 25 | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n75 |  | N/A |  | 5 |  |  | N/A | 1470 |  |  |  |  | 30.4 |  |  |  | IMD2 |  |
|  | n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_1A_n78A-n79A | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3410 |  | 10 |  |  | 50 | 3410 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4870 |  |  |  |  | 15.9 |  |  |  | IMD3 |  |
|  | 1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4670 |  | 40 |  |  | 216 | 4670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3490 |  |  |  |  | 4.6 |  |  |  | IMD5 |  |
| DC_1A_SUL_n78A-n80A | 1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | n80 |  | 1760 |  | 5 |  |  | 25 | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 1 |  | 1922.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n80 |  | 1782.5 |  | 5 |  |  | 25 | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_2A-(n)66AA | 2 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2145 |  |  |  |  | 2.8 |  |  |  | IMD5 |  |
|  | n66 |  | 1750 |  | 5 |  |  | 25 | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |
| DC_2A_n2A-n66A | 2 |  | 1875 |  | 5 |  |  | 25 | 1955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1975 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | n66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n2A-n77A | 2 |  | 1875 |  | 5 |  |  | 25 | 1955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | n77 |  | 3810 |  | 10 |  |  | 50 | 3810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1965 |  |  |  |  | 8.0 |  |  |  | IMD44 |  |
|  | n77 |  | 3735 |  | 10 |  |  | 50 | 3735 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n2A-n78A | 2 |  | 1852.5 |  | 5 |  |  | 25 | 1932.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1942.5 |  |  |  |  | 26 |  |  |  | IMD24 |  |
|  | n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-4A_n28A | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 4 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-4A_n41A | 2 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 4 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2685 |  | 10 |  |  | 50 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n12A8 | 2 |  | N/A |  | 5 |  |  | N/A | 1980 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 5 |  | 840 |  | 5 |  |  | 25 | 885 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n12 |  | 705 |  | 5 |  |  | 25 | 735 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n30A | 2 |  | 1870 |  | 5 |  |  | 25 | 1959 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 9.7 |  |  |  | IMD4 |  |
|  | n30 |  | 2310 |  | 10 |  |  | 50 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 |  | N/A |  | 5 |  |  | N/A | 1962 |  |  |  |  | 15.6 |  |  |  | IMD3 |  |
|  | 5 |  | 839 |  | 5 |  |  | 25 | 884 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n48 |  | 3640 |  | 5 |  |  | 25 | 3640 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n71A | 2 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 686.5 |  | 5 |  |  | 25 | 640.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 891.5 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_2A_n5A-n77A | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3540 |  |  |  |  | 16.0 |  |  |  | IMD3 |  |
| DC_2A_n5A-n77A11 | 2 |  | 1907 |  | 5 |  |  | 25 | 1987 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | N/A |  | 5 |  |  | N/A | 889 |  |  |  |  | 3.8 |  |  |  | IMD5 |  |
|  | n77 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n77A11 | 2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n77C11 DC_2A-5A_n77(2A)11 DC_2A-2A-5A_n77A11 | 5 |  | N/A |  | 5 |  |  | N/A | 887.5 |  |  |  |  | 3.8 |  |  |  | IMD5 |  |
| DC_2A-2A-5A_n77C11 | n77 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1987 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3680 |  | 10 |  |  | 50 | 3680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-5A_n78A DC_2A-5A_n78(2A) | 2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 887.5 |  |  |  |  | 3.8 |  |  |  | IMD5 |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1987 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3680 |  | 10 |  |  | 50 | 3680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 |  | 1855 |  | 10 |  |  | 50 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2685 |  |  |  |  | 30.0 |  |  |  | IMD2 |  |
|  | n5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-7A_n28A DC_2A-7C_n28A | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 29.0 |  |  |  | IMD2 |  |
|  | n28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3525 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1860 |  | 5 |  |  | 25 | 1940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2660 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | n77 |  | 4120 |  | 10 |  |  | 50 | 4120 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3525 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2525 |  | 5 |  |  | 25 | 2645 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3775 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_2-8_n2 | 2 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 4 |  |  |  | IMD4 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n5A DC_2A-2A-12A_n5A | 2 |  | N/A |  | 5 |  |  | N/A | 1980 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 12 |  | 705 |  | 5 |  |  | 25 | 735 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 840 |  | 5 |  |  | 25 | 885 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 731.5 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n7 |  | 2502.5 |  | 5 |  |  | 25 | 2622.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 |  | N/A |  | 5 |  |  | N/A | 1952 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | 12 |  | 708 |  | 5 |  |  | 50 | 738 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2660 |  | 10 |  |  | 50 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 738 |  |  |  |  | 28.7 |  |  |  | IMD24 |  |
|  | n41 |  | 2638 |  | 10 |  |  | 50 | 2638 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n66A | 2 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD4 |  |
|  | 12 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n77A DC_2A-12A_n77(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 16.5 |  |  |  | IMD39,11 |  |
| DC_2A-2A-12A_n77A | 12 |  | 707.5 |  | 5 |  |  | 25 | 737.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3375 |  | 10 |  |  | 50 | 3375 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1954 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 12 |  | 708 |  | 5 |  |  | 25 | 738 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3370 |  | 10 |  |  | 50 | 3370 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 |  | N/A |  | 5 |  |  | N/A | 1983.5 |  |  |  |  | 15.6 |  |  |  | IMD3 |  |
|  | 13 |  | 784.5 |  | 5 |  |  | 25 | 753.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n48 |  | 3552.5 |  | 5 |  |  | 25 | 3552.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 6.2 |  |  |  | IMD4 |  |
|  | 13 |  | 780 |  | 10 |  |  | 50 | 749 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1750 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-13A_n77A | 2 |  | N/A |  | 5 |  |  | N/A | 1944 |  |  |  |  | 16.0 |  |  |  | IMD3 |  |
| DC_2A-13A_n77C | 13 |  | 783 |  | 5 |  |  | 25 | 752 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-2A-13A_n77A DC_2A-2A-13A_n77C | n77 |  | 3510 |  | 10 |  |  | 50 | 3510 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-14A_n77A DC_2A-14A_n77(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1954 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
| DC_2A-2A-14A_n77A | 14 |  | 793 |  | 5 |  |  | 25 | 763 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3540 |  | 10 |  |  | 50 | 3540 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2_n25-n66 | 2 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | n66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1963.3 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | n66 |  | 1750 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n38A-n71A | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | N/A |  | 10 |  |  | N/A | 2586 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | n71 |  | 686 |  | 5 |  |  | 25 | 640 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n38A-n78A | 2 |  | 1870 |  | 5 |  |  | 25 | 1950 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | 2610 |  | 10 |  |  | 50 | 2610 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3350 |  |  |  |  | 14.8 |  |  |  | IMD3 |  |
| DC_2A-14A_n66A | 2 |  | N/A |  | 5 |  |  | N/A | 1954 |  |  |  |  | 7.2 |  |  |  | IMD4 |  |
|  | 14 |  | 793 |  | 5 |  |  | 25 | 763 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-28A_n66A | 2 |  | N/A |  | 5 |  |  | N/A | 1980 |  |  |  |  | 11 |  |  |  | IMD4 |  |
|  | 28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-30A_n77A DC_2A-30A_n77(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1986 |  |  |  |  | 8.6 |  |  |  | IMD411 |  |
| DC_2A-2A-30A_n77A | 30 |  | 2312 |  | 5 |  |  | 25 | 2357 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1905 |  | 5 |  |  | 25 | 1985 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A | 2354 |  |  |  |  | 10.6 |  |  |  | IMD411 |  |
|  | n77 |  | 3361 |  | 10 |  |  | 50 | 3361 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1860 |  | 5 |  |  | 25 | 1940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A | 2354 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | n77 |  | 3967 |  | 10 |  |  | 50 | 3967 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-38A_n78A | 2 |  | N/A |  | 5 |  |  | N/A | 1932.5 |  |  |  |  | 16 |  |  |  | IMD39 |  |
|  | 38 |  | 2617.5 |  | 5 |  |  | 25 | 2617.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n41A-n71A | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2530 |  | 10 |  |  | 50 | 2530 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | N/A |  | 5 |  |  | N/A | 630 |  |  |  |  | 28.7 |  |  |  | IMD2 |  |
|  | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 10 |  |  | N/A | 2586 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | n71 |  | 686 |  | 5 |  |  | 50 | 640 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-2A-46A_n5A5 DC_2A-2A-46C_n5A5 DC_2A-2A-46D_n5A5 | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD4, IMD5 |  |
|  | n5 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 DC_2A-46E_n66A5 | 2 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD3, IMD5 |  |
|  | n66 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-46A_n77A5 DC_2A-46A-46A_n77A5 | 2 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD2, IMD3 |  |
|  | n77 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 |  | 1853 |  | 5 |  |  | 25 | 1933 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | 3590 |  | 20 |  |  | 100 | 3590 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1969 |  |  |  |  | 12 |  |  |  | IMD4 |  |
| DC_2A-48A_n5A | 2 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 16.9 |  |  |  | IMD3 |  |
| DC_2A-48C_n5A | 48 |  | 3610 |  | 10 |  |  | 50 | 3610 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-48D_n5A | n5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-48E_n5A | 2 |  | 1890 |  | 5 |  |  | 25 | 1970 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 5 |  |  | N/A | 3570 |  |  |  |  | 16.2 |  |  |  | IMD3 |  |
|  | n5 |  | 840 |  | 5 |  |  | 25 | 885 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 28.3 |  |  |  | IMD2 |  |
|  | 48 |  | 3695 |  | 5 |  |  | 25 | 3695 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1735 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n48A-n66A | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-48E_n66A | n48 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1980 |  |  |  |  | 20 |  |  |  | IMD3 |  |
| DC_2A-66A_n2A | 66 |  | 1730 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A-66A_n2A | n2 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n5A | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 7.2 |  |  |  | IMD4 |  |
|  | n5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n25A | 2 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | 2 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | n25 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2112.5 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | n25 |  | 1912.5 |  | 5 |  |  | 25 | 1992.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n28A | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
|  | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2685 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 |  | 1905 |  | 5 |  |  | 25 | 1985 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1755 |  | 5 |  |  | 25 | 2155 |  |  |  |  | 12.1 |  |  |  | IMD4 |  |
|  | n48 |  | 3560 |  | 5 |  |  | 25 | 3560 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 28.3 |  |  |  | IMD5 |  |
|  | 66 |  | 1735 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n48 |  | 3695 |  | 5 |  |  | 25 | 3695 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n77A | 2 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 |  | N/A |  | 5 |  |  | N/A | 2115 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | n77 |  | 3970 |  | 10 |  |  | 50 | 3970 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | M/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 10.4 |  |  |  | IMD4 |  |
|  | n77 |  | 3500 |  | 10 |  |  | 50 | 3500 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1885 |  | 5 |  |  | 25 | 1965 |  |  |  |  | M/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2175 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
|  | n77 |  | 3915 |  | 10 |  |  | 50 | 3915 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
|  | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3720 |  | 10 |  |  | 50 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | 66 |  | 1775 |  | 5 |  |  | 25 | 2195 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3385 |  | 10 |  |  | 50 | 3385 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n77A | 2 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_2A-66A_n77C DC_2A-66A_n77(2A) DC_2A-2A-66A_n77A DC_2A-2A-66A_n77C DC_2A-66A-66A_n77A DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77A DC_2A-2A-66A-66A_n77C | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3540 |  | 10 |  |  | 50 | 3540 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2115 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | n77 |  | 3970 |  | 10 |  |  | 50 | 3970 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1853 |  | 5 |  |  | 25 | 1933 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1713 |  | 5 |  |  | 25 | 2113 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3566 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-2A-66A_n78A | 66/n66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | 10.3 |  |  |  | IMD4 |  |
|  | n78 |  | 3480 |  | 10 |  |  | 50 | 3480 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
| DC_2A-2A-66A_n78A | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3700 |  | 10 |  |  | 50 | 3700 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
| DC_2A-2A-66A_n78A | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 2.1 |  |  |  | IMD5 |  |
| DC_2A-2A-66A_n78A | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3620 |  | 10 |  |  | 50 | 3620 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n66A-n78A DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | 2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3340 |  |  |  |  | 8.9 |  |  |  | IMD4 |  |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 |  | N/A |  | 5 |  |  | N/A | 1942 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | 71 |  | 668 |  | 5 |  |  | 25 | 622 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | 2610 |  | 10 |  |  | 50 | 2610 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 |  | N/A |  | 5 |  |  | N/A | 1942 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | 71 |  | 668 |  | 5 |  |  | 25 | 622 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2610 |  | 10 |  |  | 50 | 2610 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  | N/A |  | 5 |  |  | N/A | 630 |  |  |  |  | 28.7 |  |  |  | IMD24 |  |
|  | n41 |  | 2530 |  | 10 |  |  | 50 | 2530 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 |  | N/A |  | 5 |  |  | N/A | 1954 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3340 |  | 10 |  |  | 50 | 3340 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_2A_n71A-n78A | 2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 695.5 |  | 5 |  |  | 25 | 649.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3305 |  |  |  |  | 8 |  |  |  | IMD3 |  |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2139 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
| DC_3A_n1A-n40A | n1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1735 |  | 5 |  |  | 25 | 1830 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2380 |  |  |  |  | 8.0 |  |  |  | IMD5 |  |
| DC_3A_n1A-n41A | 3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 5 |  |  | N/A | 2507.5 |  |  |  |  | 5.0 |  |  |  | IMD5 |  |
| DC_3A_n1A-n77A | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3700 |  |  |  |  | 28.4 |  |  |  | IMD2 |  |
|  | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 31.0 |  |  |  | IMD2 |  |
|  | n77 |  | 3915 |  | 10 |  |  | 50 | 3915 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3700 |  |  |  |  | 28.4 |  |  |  | IMD2 |  |
|  | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2130 |  |  |  |  | 3.5 |  |  |  | IMD5 |  |
|  | n78 |  | 3720 |  | 10 |  |  | 50 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n3A-n41A | 3 |  | 1725 |  | 5 |  |  | 25 | 1820 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1865 |  |  |  |  | 8.2 |  |  |  | IMD4 |  |
|  | n41 |  | 2657.5 |  | 5 |  |  | 25 | 2657.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 17.3 |  |  |  | IMD3 |  |
|  | 5 |  | 845 |  | 5 |  |  | 25 | 804 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3510 |  | 10 |  |  | 50 | 3510 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-5A_n78A | 3 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD3 |  |
|  | 5 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-5A_n79A | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 18.5 |  |  |  | IMD3 |  |
|  | n79 |  | 4435 |  | 40 |  |  | 216 | 4435 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1877.5 |  |  |  |  | 0.2 |  |  |  | IMD4 |  |
|  | 5 |  | 842.5 |  | 5 |  |  | 25 | 887.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 | 4420 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A_n5A | 3 |  | 1780 |  | 10 |  |  | 50 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2625 |  |  |  |  | 30.0 |  |  |  | IMD21 |  |
|  | n5 |  | 845 |  | 5 |  |  | 25 | 890 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A_n8A | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 890 |  | 5 |  |  | 25 | 935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2670 |  |  |  |  | 29.0 |  |  |  | IMD2 IMD33 |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2682 |  |  |  |  | 16.9 |  |  |  | IMD3 |  |
|  | 7 |  | 2543 |  | 10 |  |  | 50 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1832.5 |  |  |  |  | 26.0 |  |  |  | IMD2 |  |
| DC_3A-18A_n3A | 3 |  | N/A |  | 5 |  |  | N/A | 1814 |  |  |  |  | 4 |  |  |  | IMD4 |  |
|  | 18 |  | 823 |  | 5 |  |  | 25 | 868 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3-18_n41 | 18 |  | N/A |  | 5 |  |  | N/A | 865 |  |  |  |  | 28.9 |  |  |  | IMD2 |  |
|  | 3 |  | 1765 |  | 5 |  |  | 25 | 1860 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2630 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | N/A |  | 5 |  |  | N/A | 865 |  |  |  |  | 19.0 |  |  |  | IMD3 |  |
|  | 3 |  | 1725 |  | 5 |  |  | 25 | 1820 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2585 |  | 5 |  |  | 25 | 2585 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 28.8 |  |  |  | IMD2 |  |
|  | n41 |  | 2670 |  | 10 |  |  | 50 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 18 |  | 820 |  | 5 |  |  | 25 | 865 |  |  |  |  | MSD |  |  |  | N/A |  |
| DC_3A-18A_n77A DC_3A-18A_n77(2A)DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD3 |  |
|  | 18 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-19A_n77A DC_3A-19A_n78A | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 17.3 |  |  |  | IMD3 |  |
|  | 19 |  | 835 |  | 5 |  |  | 25 | 880 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | 3520 |  | 10 |  |  | 50 | 3520 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n7A-n28A | 3 |  | 1747 |  | 5 |  |  | 25 | 1842 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3C_n7A-n28A | n7 |  | 2543 |  | 5 |  |  | 25 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 796.0 |  |  |  |  | 20.0 |  |  |  | IMD2 |  |
|  | 3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | N/A |  | 5 |  |  | N/A | 2682 |  |  |  |  | 17.0 |  |  |  | IMD3 |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A_n40A | 3 |  | N/A |  | 5 |  |  | N/A | 1866.6 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | 7 |  | 2530 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2310 |  | 5 |  |  | 25 | 2310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A_n77A | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 17.6 |  |  |  | IMD3 |  |
| DC_3A-7A_n77(2A) | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A-7A_n77(2A) | n77 |  | 3310 |  | 10 |  |  | 50 | 3310 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3475 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1715 |  | 5 |  |  | 25 | 1810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2670 |  |  |  |  | 5.2 |  |  |  | IMD5 |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 | 4190 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2640 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | n77 |  | 3900 |  | 10 |  |  | 50 | 3900 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A-7A_n78C | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 17.6 |  |  |  | IMD3 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3310 |  | 10 |  |  | 50 | 3310 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3475 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-8A_n40A | 3 |  | N/A |  | 5 |  |  | N/A | 1874 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | 8 |  | 912 |  | 5 |  |  | 25 | 957 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2305 |  | 5 |  |  | 25 | 2305 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 |  | 1715 |  | 5 |  |  | 25 | 1810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 | 4190 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 955 |  |  |  |  | 9.7 |  |  |  | IMD4 |  |
| DC_3A-8A_n77A DC_3A-8A_n77(2A) DC_3A-8A_n77(3A) DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3640 |  | 10 |  |  | 50 | 3640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3640 |  | 10 |  |  | 50 | 3640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
| DC_3A_n8A-n78A | 3 |  | 1740 |  | 5 |  |  | 25 | 1835 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3540 |  |  |  |  | 16.3 |  |  |  | IMD3 |  |
| DC_3A-8A_n79A | 3 |  | 1755 |  | 5 |  |  | 25 | 1850 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4465 |  | 40 |  |  | 216 | 4465 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 955 |  |  |  |  | 15.3 |  |  |  | IMD3 |  |
| DC_3A-8A_n79A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4580 |  | 40 |  |  | 216 | 4580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 8.8 |  |  |  | IMD4 |  |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n7A-n78(2A) | n7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3C_n7A-n78(2A) | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3675 |  | 10 |  |  | 50 | 3675 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1491 |  |  |  |  | 8.8 |  |  |  | IMD4 |  |
|  | 11 |  | 1435.4 |  | 5 |  |  | 25 | 1483.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3905 |  | 10 |  |  | 50 | 3905 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1848 |  |  |  |  | 3.4 |  |  |  | IMD57 |  |
| DC_3A-19A_n79A | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 19 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 18.5 |  |  |  | IMD3 |  |
|  | n79 |  | 4435 |  | 40 |  |  | 216 | 4435 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1877.5 |  |  |  |  | 0.2 |  |  |  | IMD4 |  |
|  | 19 |  | 842.5 |  | 5 |  |  | 25 | 887.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 | 4420 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 |  | 1737 |  | 5 |  |  | 25 | 1832 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 806 |  |  |  |  | 10.5 |  |  |  | IMD2 |  |
|  | n7 |  | 2543 |  | 10 |  |  | 50 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-20A_n8A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 810 |  |  |  |  | 27 |  |  |  | IMD2 |  |
| DC_3A-20A_n8A | 3 |  | N/A |  | 5 |  |  | N/A | 1860 |  |  |  |  | 14.5 |  |  |  | IMD4 |  |
|  | n8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | 840 |  | 5 |  |  | 25 | 799 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 |  | 852 |  | 5 |  |  | 25 | 811 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 728 |  | 5 |  |  | 25 | 783 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1828 |  |  |  |  | 9.4 |  |  |  | IMD4 |  |
| DC_3A-20A_n38A | 3 |  | 1779 |  | 5 |  |  | 25 | 1874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 811 |  |  |  |  | 26.0 |  |  |  | IMD21 |  |
|  | n38 |  | 2590 |  | 10 |  |  | 50 | 2590 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 |  | N/A |  | 5 |  |  | N/A | 1839 |  |  |  |  | 26.0 |  |  |  | IMD2 |  |
|  | n41 |  | 2680 |  | 10 |  |  | 50 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | 841 |  | 10 |  |  | 50 | 800 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 |  | 1779 |  | 5 |  |  | 25 | 1874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2590 |  | 10 |  |  | 50 | 2590 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 811 |  |  |  |  | 26.0 |  |  |  | IMD2 |  |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2660 |  | 10 |  |  | 50 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 800 |  |  |  |  | 12.5 |  |  |  | IMD3 |  |
| DC_3_n20-n67 | 3 |  | 1765 |  | 5 |  |  | 25 | 1860 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n20 |  | 837 |  | 5 |  |  | 25 | 796 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n67 |  | N/A |  | 5 |  |  | 25 | 746 |  |  |  |  | 10.1 |  |  |  | IMD4 |  |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 17.3 |  |  |  | IMD3 |  |
|  | 20 |  | 845 |  | 5 |  |  | 25 | 804 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3510 |  | 10 |  |  | 50 | 3510 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n20A-n78A | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n20 |  | 845 |  | 5 |  |  | 25 | 804 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3420 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
| DC_3A-20A_n78A DC_3C-20A_n78A DC_3A-20A_n78(2A) | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 17.3 |  |  |  | IMD3 |  |
|  | 20 |  | 845 |  | 5 |  |  | 25 | 804 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3510 |  | 10 |  |  | 50 | 3510 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 |  | 1767.5 |  | 5 |  |  | 25 | 1862.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  | N/A |  | 5 |  |  | N/A | 1507.5 |  |  |  |  | 8.8 |  |  |  | IMD44 |  |
|  | n77, n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1862.5 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
|  | 21 |  | 1459.5 |  | 5 |  |  | 25 | 1507.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77, n78 |  | 3322 |  | 10 |  |  | 50 | 3322 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-21A_n77A | 3 |  | N/A |  | 5 |  |  | N/A | 1866.6 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3935 |  | 10 |  |  | 50 | 3935 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-21A_n79A | 3 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 21 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD3 |  |
|  | n79 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1869.2 |  |  |  |  | 17.8 |  |  |  | IMD3 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4770 |  | 40 |  |  | 216 | 4770 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n1A DC_3C-28A_n1A | 3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | 28 |  | 710 |  | 5 |  |  | 25 | 765 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1975 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 |  | N/A |  | 5 |  |  | N/A | 1830 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | 28 |  | 705 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 845 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 785 |  |  |  |  | 9.4 |  |  |  | IMD4 |  |
|  | n5 |  | 845 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 |  | N/A |  | 5 |  |  | N/A | 1832.5 |  |  |  |  | 26.0 |  |  |  | IMD2 |  |
|  | 28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2543 |  | 10 |  |  | 50 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1747 |  | 5 |  |  | 25 | 1842 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 796.0 |  |  |  |  | 20.0 |  |  |  | IMD2 |  |
|  | n7 |  | 2543 |  | 5 |  |  | 25 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n77A | 3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 770 |  |  |  |  | 15.3 |  |  |  | IMD3 |  |
|  | n77 |  | 4195 |  | 10 |  |  | 50 | 4195 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 17.0 |  |  |  | IMD3 |  |
|  | 28 |  | 735 |  | 5 |  |  | 25 | 790 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3320 |  | 10 |  |  | 50 | 3320 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 708 |  | 5 |  |  | 25 | 763 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n75 |  | N/A |  | 5 |  |  | N/A | 1436 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_3A_n28A-n77A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | 733 |  | 5 |  |  | 25 | 788 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 4173 |  |  |  |  | 15.9 |  |  |  | IMD3 |  |
|  | 3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 770 |  |  |  |  | 15.3 |  |  |  | IMD3 |  |
|  | n77 |  | 4195 |  | 10 |  |  | 50 | 4195 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n41A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2510 |  | 5 |  |  | 25 | 2510 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 790 |  |  |  |  | 26.0 |  |  |  | IMD21 |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1832.5 |  |  |  |  | 26.0 |  |  |  | IMD2 |  |
|  | n41 |  | 2543 |  | 10 |  |  | 50 | 2543 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n28A-n41A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 790 |  |  |  |  | 261 |  |  |  | IMD2 |  |
|  | n41 |  | 2510 |  | 5 |  |  | 25 | 2510 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 738 |  | 5 |  |  | 25 | 793 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 5 |  |  | N/A | 2518 |  |  |  |  | 27.4 |  |  |  | IMD2 |  |
|  | 3 |  | 1715 |  | 5 |  |  | 25 | 1810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 5 |  |  | N/A | 2687 |  |  |  |  | 15.9 |  |  |  | IMD3 |  |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 |  | N/A |  | 5 |  |  | N/A | 1870 |  |  |  |  | 17.3 |  |  |  | IMD3 |  |
|  | 28 |  | 740 |  | 5 |  |  | 25 | 760 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3350 |  | 10 |  |  | 25 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-28A_n79A | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 780 |  |  |  |  | 10.3 |  |  |  | IMD4 |  |
|  | n79 |  | 4530 |  | 40 |  |  | 216 | 4530 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1870 |  |  |  |  | 5.7 |  |  |  | IMD5 |  |
|  | 28 |  | 725 |  | 5 |  |  | 25 | 780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4770 |  | 40 |  |  | 216 | 4770 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 |  | 1750 |  | 5 |  |  | 25 | 1845 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3764 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
| DC_3A_n28A-n79A | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 780 |  |  |  |  | 10.3 |  |  |  | IMD4 |  |
|  | n79 |  | 4530 |  | 40 |  |  | 216 | 4530 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 725 |  | 5 |  |  | 25 | 780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4585 |  |  |  |  | 9.4 |  |  |  | IMD44 |  |
| DC_3A_SUL_n77A-n84A | 3 |  | 1782.5 |  | 5 |  |  | 25 | 1877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n84 |  | 1922.5 |  | 5 |  |  | 25 |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_3A_n40A-n78A | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2360 |  | 5 |  |  | 25 | 2360 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 4.8 |  |  |  | IMD5 |  |
|  | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | N/A |  | 5 |  |  | N/A | 2360 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | n78 |  | 3760 |  | 10 |  |  | 50 | 3760 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n40A-n79A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2330 |  | 5 |  |  | 25 | 2330 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4550 |  |  |  |  | 4.7 |  |  |  | IMD5 |  |
|  | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | N/A |  | 5 |  |  | N/A | 2330 |  |  |  |  | 3.2 |  |  |  | IMD5 |  |
|  | n79 |  | 4550 |  | 40 |  |  | 216 | 4550 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n41A-n79A | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2670 |  | 10 |  |  | 50 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4440 |  |  |  |  | 30.8 |  |  |  | IMD24 |  |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 |  | 1782.5 |  | 5 |  |  | 25 | 1877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 42 |  | N/A |  | 5 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
|  | n1 |  | 1922.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 |  | 1782.5 |  | 5 |  |  | 25 | 1877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n75 |  | N/A |  | 5 |  |  | N/A | 1514.5 |  |  |  |  | 10.0 |  |  |  | IMD2 |  |
| DC_3A_n78A-n79A | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3340 |  | 10 |  |  | 50 | 3340 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4910 |  |  |  |  | 16.3 |  |  |  | IMD3 |  |
|  | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4510 |  | 40 |  |  | 216 | 4510 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3710 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_3A_SUL_n78A-n82A | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | 4 |  |  |  | IMD4 |  |
|  | n82 |  | 840 |  | 5 |  |  | 25 |  |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A_SUL_n78A-n84A | 3 |  | 1782.5 |  | 5 |  |  | 25 | 1877.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n84 |  | 1922.5 |  | 5 |  |  | 25 |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3425 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_3A-21A_n79A | 3 |  | N/A |  | 5 |  |  | N/A | 1869.2 |  |  |  |  | 17.8 |  |  |  | IMD3 |  |
|  | 21 |  | 1450.4 |  | 5 |  |  | 25 | 1498.4 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4770 |  | 40 |  |  | 216 | 4770 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-32A_n1A | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3C-32A_n1A | 32 |  | N/A |  | 5 |  |  | N/A | 1480 |  |  |  |  | 15.2 |  |  |  | IMD34,16 |  |
|  | n1 |  | 1960 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C DC_3A-32A_n78(2A) | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1470 |  |  |  |  | 4.9 |  |  |  | IMD4 |  |
|  | n78 |  | 3720 |  | 10 |  |  | 50 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1475 |  |  |  |  | 0 |  |  |  | IMD5 |  |
|  | n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 |  | 2575 |  | 5 |  |  | 25 | 2575 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 725 |  | 5 |  |  | 25 | 780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 26 |  |  |  | IMD2 |  |
| DC_3A-38A_n78A DC_3C-38A_n78A | 3 |  | N/A |  | 5 |  |  | N/A | 1830 |  |  |  |  | 16.4 |  |  |  | IMD35 |  |
|  | 38 |  | 2615 |  | 5 |  |  | 25 | 2615 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1735 |  | 5 |  |  | 25 | 1830 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2380 |  |  |  |  | 8.0 |  |  |  | IMD5 |  |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 |  | N/A |  | 5 |  |  | N/A | 1870 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | 40 |  | 2390 |  | 5 |  |  | 25 | 2390 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3325 |  | 10 |  |  | 50 | 3325 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2360 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | n78 |  | 3760 |  | 10 |  |  | 50 | 3760 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 |  | N/A |  | 5 |  |  | N/A | 1865 |  |  |  |  | 8.2 |  |  |  | IMD4 |  |
|  | 41 |  | 2657.5 |  | 5 |  |  | 25 | 2657.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1725 |  | 5 |  |  | 25 | 1820 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 |  | 2543 |  | 10 |  |  | 50 | 2543 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 710.5 |  | 5 |  |  | 25 | 765.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1832.5 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | 3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 738 |  | 5 |  |  | 25 | 793 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2518 |  |  |  |  | 27.4 |  |  |  | IMD2 |  |
|  | 3 |  | 1715 |  | 5 |  |  | 25 | 1810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2687 |  |  |  |  | 15.9 |  |  |  | IMD3 |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A DC_3A_n41A-n77(2A) | 3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3900 |  | 10 |  |  | 50 | 3900 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41/n41 |  | N/A |  | 5 |  |  | N/A | 2640 |  |  |  |  | 5.3 |  |  |  | IMD5 |  |
|  | 41/n41 |  | 2620 |  | 5 |  |  | 25 | 2620 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1840 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 |  | 2620 |  | 5 |  |  | 25 | 2620 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1840 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
| DC_3A_n41A-n78A DC_3A_n41A-n78(2A) | 3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2560 |  | 10 |  |  | 50 | 2560 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
| DC_3A-41A_n79A | 3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4440 |  | 40 |  |  | 216 | 4440 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2670 |  |  |  |  | 30.2 |  |  |  | IMD2 |  |
|  | 41 |  | 2570 |  | 5 |  |  | 25 | 2570 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4420 |  | 40 |  |  | 216 | 4420 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
| DC_4A-7A_n28A | 4 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2685 |  |  |  |  | 18.0 |  |  |  | IMD3 |  |
|  | n28 |  | 745 |  | 5 |  |  | 25 | 800 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5_n1-n78 | 5 |  | 829 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2122 |  |  |  |  | 18.1 |  |  |  | IMD3 |  |
|  | n78 |  | 3780 |  | 10 |  |  | 50 | 3780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1950 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3610 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
| DC_5A_n2A-n77A11 | n2 |  | N/A |  | 5 |  |  | N/A | 1987 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3680 |  | 10 |  |  | 50 | 3680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n5A-n77A11 | 5 |  | 834 |  | 5 |  |  | 25 | 879 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | N/A |  | 5 |  |  | N/A | 889 |  |  |  |  | 8.3 |  |  |  | IMD4 |  |
|  | n77 |  | 3391 |  | 10 |  |  | 50 | 3391 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | N/A |  | 5 |  |  | N/A | 882 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |
|  | n77 |  | 4188 |  | 10 |  |  | 50 | 4188 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A_n7A | 5 |  | N/A |  | 5 |  |  | N/A | 879 |  |  |  |  | 12 |  |  |  | IMD34 |  |
|  | 7 |  | 2527 |  | 10 |  |  | 50 | 2647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2547 |  | 10 |  |  | 50 | 2667 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n2A-n78A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3540 |  |  |  |  | 16.0 |  |  |  | IMD3 |  |
|  | 5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1987 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | n78 |  | 3680 |  | 10 |  |  | 25 | 3680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n3A-n78A | 5 |  | 839 |  | 5 |  |  | 25 | 884 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3408 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 5 |  | 839 |  | 5 |  |  | 25 | 884 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3512 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | 5 |  | 839 |  | 5 |  |  | 25 | 884 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1862 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | n78 |  | 3540 |  | 10 |  |  | 50 | 3540 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 17.8 |  |  |  | IMD3 |  |
|  | 7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2624 |  |  |  |  | 29.0 |  |  |  | IMD21 |  |
|  | 66 |  | 1777.5 |  | 5 |  |  | 25 | 2177.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A_n71A | 5 |  | 835 |  | 5 |  |  | 25 | 880 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2660 |  |  |  |  | 6.5 |  |  |  | IMD5 |  |
|  | n71 |  | 680 |  | 5 |  |  | 25 | 634 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 844 |  | 5 |  |  | 25 | 889 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A_n77A | 7 |  | N/A |  | 5 |  |  | N/A | 2645 |  |  |  |  | 30.1 |  |  |  | IMD2 |  |
| DC_5A-7A_n77(2A) | n77 |  | 3489 |  | 10 |  |  | 50 | 3489 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A-7A_n77A | 5 |  | N/A |  | 5 |  |  | N/A | 879 |  |  |  |  | 30.2 |  |  |  | IMD21 |  |
| DC_5A-7A-7A_n77(2A) | 7 |  | 2550 |  | 5 |  |  | 25 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3429 |  | 10 |  |  | 50 | 3429 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A-7A_n78C | 5 |  | 844 |  | 5 |  |  | 25 | 889 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2645 |  |  |  |  | 30.1 |  |  |  | IMD2 |  |
|  | n78 |  | 3489 |  | 10 |  |  | 50 | 3489 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 879 |  |  |  |  | 30.2 |  |  |  | IMD2 |  |
|  | 7 |  | 2550 |  | 5 |  |  | 25 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3429 |  | 10 |  |  | 50 | 3429 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 875 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
|  | 7 |  | 2525 |  | 5 |  |  | 25 | 2645 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 |  | 844 |  | 5 |  |  | 25 | 889 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | N/A |  | 5 |  |  | N/A | 2645 |  |  |  |  | 30.1 |  |  |  | IMD2 |  |
|  | n78 |  | 3489 |  | 10 |  |  | 50 | 3489 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 835 |  | 5 |  |  | 25 | 880 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2540 |  | 5 |  |  | 25 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3375 |  |  |  |  | 29.7 |  |  |  | IMD2 |  |
| DC_5A-13A_n66A | 5 |  | 840 |  | 5 |  |  | 25 | 885 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 13 |  | N/A |  | 5 |  |  | N/A | 750 |  |  |  |  | 9.4 |  |  |  | IMD4 |  |
|  | n66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-13A_n77A11 | 5 |  | 840 |  | 5 |  |  | 25 | 885 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-13A_n77C11 | n77 |  | 4110 |  | 10 |  |  | 50 | 4110 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 13 |  | N/A |  | 5 |  |  | N/A | 750 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | 13 |  | 782 |  | 5 |  |  | 20 | 751 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4013 |  | 10 |  |  | 50 | 4013 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
| DC_5A-30A_n2A | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 8 |  |  |  | IMD4 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1870 |  | 5 |  |  | 25 | 1950 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-30A_n77A DC_5A-30A_n77(2A) | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 15.2 |  |  |  | IMD34 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3740 |  | 10 |  |  | 50 | 3740 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 835 |  | 5 |  |  | 25 | 880 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A | 2355 |  |  |  |  | 13.2 |  |  |  | IMD311 |  |
|  | n77 |  | 4025 |  | 10 |  |  | 50 | 4025 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n38A-n66A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | N/A |  | 10 |  |  | N/A | 2590 |  |  |  |  | 28.9 |  |  |  | IMD2 |  |
| DC_5A_41A_n78A | 5 |  | N/A |  | 5 |  |  | N/A | 885 |  |  |  |  | 30.2 |  |  |  | IMD2 |  |
|  | 41 |  | 2615 |  | 5 |  |  | 25 | 2615 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3500 |  | 10 |  |  | 50 | 3500 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 881.5 |  |  |  |  | 3.1 |  |  |  | IMD5 |  |
|  | 41 |  | 2620.5 |  | 5 |  |  | 25 | 2620.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3490 |  | 10 |  |  | 50 | 3490 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-41A_n79A | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 23.9 |  |  |  | IMD3 |  |
|  | 41 |  | 2665 |  | 5 |  |  | 25 | 2665 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4450 |  | 40 |  |  | 216 | 4450 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | N/A |  | 5 |  |  | N/A | 2517.5 |  |  |  |  | 1.8 |  |  |  | IMD4 |  |
|  | n79 |  | 4980 |  | 40 |  |  | 216 | 4980 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-46A_n66A | 5 |  | 847 |  | 5 |  |  | 25 | 892 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 46 |  | N/A |  | 10 |  |  | N/A | 5163 |  |  |  |  | 9.04 |  |  |  | IMD4 |  |
|  | n66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-48A_n12A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 5 |  |  | N/A | 3650 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | n12 |  | 705 |  | 5 |  |  | 25 | 735 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 875 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 48 |  | 3695 |  | 5 |  |  | 25 | 3695 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n12 |  | 705 |  | 5 |  |  | 25 | 735 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-48A_n71A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 5 |  |  | N/A | 3590 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | n71 |  | 690 |  | 5 |  |  | 25 | 644 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 48 |  | 3600 |  | 5 |  |  | 25 | 3600 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 680 |  | 5 |  |  | 25 | 634 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 |  | 834 |  | 5 |  |  | 25 | 879 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66B_n2A | 66 |  | N/A |  | 5 |  |  | N/A | 2132 |  |  |  |  | 7.2 |  |  |  | IMD4 |  |
|  | n2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 |  | N/A |  | 5 |  |  | N/A | 880 |  |  |  |  | 18.0 |  |  |  | IMD3 |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n30A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2125 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | n30 |  | 2307.5 |  | 5 |  |  | 50 | 2352.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n71A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2161 |  |  |  |  | 13 |  |  |  | IMD3 |  |
|  | n71 |  | 665.5 |  | 5 |  |  | 25 | 619.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 891.5 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 665.5 |  | 5 |  |  | 25 | 619.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n77A | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n77C DC_5A-66A_n77(2A) DC_5A-66A-66A_n77A DC_5A-66A-66A_n77C | 66 |  | N/A |  | 5 |  |  | N/A | 2142 |  |  |  |  | 13.2 |  |  |  | IMD3 |  |
|  | n77 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A-66A-66A_n78A | 66 |  | N/A |  | 5 |  |  | N/A | 2142 |  |  |  |  | 13.2 |  |  |  | IMD3 |  |
|  | n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n66A-n78A | 5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3420 |  |  |  |  | 16.6 |  |  |  | IMD3 |  |
|  | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2142 |  |  |  |  | 13.2 |  |  |  | IMD3 |  |
|  | n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_5A_n66A-n77A | 5 |  | 826.5 |  | 5 |  |  | 25 | 871.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2142 |  |  |  |  | 13.2 |  |  |  | IMD3 |  |
|  | n77 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | 845 |  | 5 |  |  | 25 | 890 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1785 |  | 5 |  |  | 25 | 2185 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3475 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
| DC_7A_n1A-n40A | 7 |  | 2540 |  | 5 |  |  | 25 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2335 |  | 5 |  |  | 25 | 2335 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2130 |  |  |  |  | 15.2 |  |  |  | IMD3 |  |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 10.1 |  |  |  | IMD4 |  |
|  | 7 |  | 2530 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2160 |  |  |  |  | 9.0 |  |  |  | IMD4 |  |
|  | n78 |  | 3610 |  | 10 |  |  | 50 | 3610 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n2A-n71A | 7 |  | 2530 |  | 5 |  |  | 25 | 2530 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | N/A |  | 5 |  |  | N/A | 630 |  |  |  |  | 28.7 |  |  |  | IMD2 |  |
| DC_7A_n2A-n78A | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | n78 |  | 3525 |  | 10 |  |  | 50 | 3525 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2525 |  | 5 |  |  | 25 | 2645 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3775 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_7A_n3A-n78A | 7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 15.6 |  |  |  | IMD3 |  |
|  | n78 |  | 3310 |  | 10 |  |  | 50 | 3310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n8A-n40A | 7 |  | 2530 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 905 |  | 5 |  |  | 25 | 950 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | N/A |  | 5 |  |  | N/A | 2345 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
| DC_7A-8A_n3A | n3 |  | 1735 |  | 5 |  |  | 25 | 1830 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2530 |  | 10 |  |  | 50 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 18.0 |  |  |  | IMD3 |  |
| DC_7A-8A_n3A | n3 |  | 1780 |  | 5 |  |  | 25 | 1875 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | 890 |  | 5 |  |  | 25 | 935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A |  | 2670 |  |  |  |  | 29.0 |  |  | IMD2+IMD33 |  |
| DC_7A-8A_n77A | 7 |  | 2530 |  | 5 |  |  | 25 |  | 2650 |  |  |  |  | N/A |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 30.5 |  |  |  | IMD2 |  |
|  | n77 |  | 3470 |  | 10 |  |  | 50 | 3470 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-8A_n77A | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 3.1 |  |  |  | IMD5 |  |
|  | n77 |  | 3310 |  | 10 |  |  | 50 | 3310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-8A_n77A | 7 |  | N/A |  | 5 |  |  | N/A | 2650 |  |  |  |  | 28 |  |  |  | IMD2 |  |
|  | 8 |  | 895 |  | 5 |  |  | 25 | 940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3545 |  | 10 |  |  | 50 | 3545 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-8A_n78A | 7 |  | 2530 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 30.5 |  |  |  | IMD2 |  |
|  | n78 |  | 3470 |  | 10 |  |  | 50 | 3470 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-8A_n78A | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 3.1 |  |  |  | IMD5 |  |
|  | n78 |  | 3310 |  | 10 |  |  | 50 | 3310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-8A_n78A | 7 |  | N/A |  | 5 |  |  | N/A | 2650 |  |  |  |  | 28 |  |  |  | IMD2 |  |
|  | 8 |  | 895 |  | 5 |  |  | 25 | 940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3545 |  | 10 |  |  | 50 | 3545 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n8A-n78A | 7 |  | 2555 |  | 5 |  |  | 25 | 2675 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3455 |  |  |  |  | 28.5 |  |  |  | IMD2 |  |
|  | 7 |  | 2555 |  | 5 |  |  | 25 | 2675 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | N/A |  | 5 |  |  | N/A | 945 |  |  |  |  | 29.7 |  |  |  | IMD2 |  |
|  | n78 |  | 3500 |  | 10 |  |  | 50 | 3500 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-12A_n2A | 7 |  | 2502.5 |  | 5 |  |  | 25 | 2622.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 731.5 |  |  |  |  | 5.3 |  |  |  | IMD5 |  |
|  | n2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2621 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
|  | 12 |  | 713.5 |  | 5 |  |  | 25 | 743.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-12A_n66A | 7 |  | 2515 |  | 5 |  |  | 25 | 2635 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 742 |  |  |  |  | 31 |  |  |  | IMD2 |  |
|  | n66 |  | 1773 |  | 5 |  |  | 25 | 2173 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | 7 |  | N/A |  | 5 |  |  | N/A | 2662 |  |  |  |  | 29.6 |  |  |  | IMD2 |  |
|  | 12 |  | 708 |  | 5 |  |  | 25 | 738 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3370 |  | 10 |  |  | 50 | 3370 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 740 |  |  |  |  | 30.8 |  |  |  | IMD24 |  |
|  | n78 |  | 3305 |  | 10 |  |  | 50 | 3305 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-13A_n66A | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 13 |  | N/A |  | 5 |  |  | N/A | 750 |  |  |  |  | 31 |  |  |  | IMD2 |  |
|  | n66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-13A_n66A | 7 |  | N/A |  | 5 |  |  | N/A | 2660 |  |  |  |  | 18 |  |  |  | IMD3 |  |
|  | 13 |  | 780 |  | 5 |  |  | 25 | 749 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 |  | N/A |  | 10 |  |  | N/A | 2662 |  |  |  |  | 27.6 |  |  |  | IMD2 |  |
|  | 13 |  | 782 |  | 5 |  |  | 25 | 751 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 800 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n3A | 7 |  | 2543 |  | 10 |  |  | 50 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 10 |  |  | N/A | 806 |  |  |  |  | 10.5 |  |  |  | IMD2 |  |
|  | n3 |  | 1737 |  | 5 |  |  | 25 | 1832 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2630 |  |  |  |  | 26.0 |  |  |  | IMD21 |  |
|  | 20 |  | 855 |  | 5 |  |  | 25 | 896 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1775 |  | 10 |  |  | 50 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n8A | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 885 |  | 5 |  |  | 25 | 930 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 795 |  |  |  |  | 17.4 |  |  |  | IMD3 |  |
| DC_7A-20A_n8A | 7 |  | N/A |  | 5 |  |  | N/A | 2640 |  |  |  |  | 21.1 |  |  |  | IMD3 |  |
|  | n8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | 840 |  | 5 |  |  | 25 | 799 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n8A | 7 |  | N/A |  | 5 |  |  | N/A | 2624 |  |  |  |  | 18.8 |  |  |  | IMD3 |  |
|  | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | 857 |  | 5 |  |  | 25 | 816 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n28A | 20 |  | 842 |  | 5 |  |  | 25 | 801 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 728 |  | 5 |  |  | 25 | 783 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2640 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
| DC_7A-20A_n78A | 7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n78(2A) | 20 |  | N/A |  | 5 |  |  | N/A | 810 |  |  |  |  | 30.5 |  |  |  | IMD2 |  |
|  | n78 |  | 3370 |  | 10 |  |  | 50 | 3370 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n78A | 7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n78(2A) | 20 |  | N/A |  | 5 |  |  | N/A | 810 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
|  | n78 |  | 3435 |  | 10 |  |  | 50 | 3435 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-20A_n78A | 7 |  | N/A |  | 5 |  |  | N/A | 2675 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
| DC_7A-20A_n78(2A) | 20 |  | 845 |  | 5 |  |  | 25 | 804 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3520 |  | 10 |  |  | 50 | 3520 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 |  | 2550 |  | 5 |  |  | 25 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | n77 |  | 3525 |  | 10 |  |  | 50 | 3525 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2660 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | 25 |  | 1860 |  | 5 |  |  | 25 | 1940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4120 |  | 10 |  |  | 50 | 4120 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 |  | 2550 |  | 5 |  |  | 25 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1950 |  |  |  |  | 8.6 |  |  |  | IMD4 |  |
|  | n78 |  | 3525 |  | 10 |  |  | 50 | 3525 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n1A | 7 |  | 2535 |  | 5 |  |  | 25 | 2655 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-7A-28A_n1A | 28 |  | N/A |  | 5 |  |  | N/A | 780 |  |  |  |  | 4.3 |  |  |  | IMD5 |  |
|  | n1 |  | 1950 |  | 5 |  |  | 25 | 2165 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2665 |  |  |  |  | 29.0 |  |  |  | IMD2 |  |
|  | 28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1935 |  | 5 |  |  | 25 | 2125 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n2A | 7 |  | N/A |  | 10 |  |  | N/A | 2630 |  |  |  |  | 27.6 |  |  |  | IMD2 |  |
|  | 28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 |  | 2543 |  | 5 |  |  | 25 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 796.0 |  |  |  |  | 20.0 |  |  |  | IMD2 |  |
|  | n3 |  | 1747 |  | 5 |  |  | 25 | 1842 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2685 |  |  |  |  | 18 |  |  |  | IMD3 |  |
|  | 28 |  | 745 |  | 5 |  |  | 25 | 800 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1715 |  | 5 |  |  | 25 | 1810 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 |  | 2540 |  | 5 |  |  | 25 | 2725 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 776 |  |  |  |  | 4.4 |  |  |  | IMD5 |  |
|  | n5 |  | 829 |  | 5 |  |  | 25 | 854 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2630 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 28 |  | 730 |  | 5 |  |  | 25 | 785 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 840 |  | 5 |  |  | 25 | 874 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n40A | 7 |  | N/A |  | 5 |  |  | N/A | 2630 |  |  |  |  | 5.9 |  |  |  | IMD5 |  |
|  | 28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2310 |  | 5 |  |  | 25 | 2310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 |  | N/A |  | 10 |  |  | N/A | 2682 |  |  |  |  | 16.9 |  |  |  | IMD3 |  |
|  | 28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1712.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2543 |  | 5 |  |  | 25 | 2663 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 796 |  |  |  |  | 20.0 |  |  |  | IMD2 |  |
|  | n66 |  | 1747 |  | 5 |  |  | 25 | 2147 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-28A_n78A | 7 |  | 2567.5 |  | 5 |  |  | 25 | 2687.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 782.5 |  |  |  |  | 28.8 |  |  |  | IMD2 |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2567.5 |  | 5 |  |  | 25 | 2687.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | N/A |  | 5 |  |  | N/A | 782.5 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
|  | n78 |  | 3460 |  | 10 |  |  | 50 | 3460 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2650 |  |  |  |  | 30.5 |  |  |  | IMD2 |  |
|  | 28 |  | 740 |  | 5 |  |  | 25 | 795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3390 |  | 10 |  |  | 50 | 3390 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 745 |  | 5 |  |  | 25 | 800 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3310 |  |  |  |  | 29.7 |  |  |  | IMD2 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3365 |  | 10 |  |  | 50 | 3365 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 800 |  |  |  |  | 28.8 |  |  |  | IMD2 |  |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 |  | 2540 |  | 5 |  |  | 25 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 29 |  | N/A |  | N/A |  |  | N/A | 720 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
|  | n78 |  | 3450 |  | 10 |  |  | 50 | 3450 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-32A_n1A | n1 |  | 1977.5 |  | 5 |  |  | 25 | 2167.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2502.5 |  | 5 |  |  | 25 | 2622.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1454.5 |  |  |  |  | 15.2 |  |  |  | IMD3 |  |
| DC_7A-32A_n3A | 7 |  | 1775 |  | 5 |  |  | 25 | 1870 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 2510 |  | 10 |  |  | 50 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1470 |  |  |  |  | 10.5 |  |  |  | IMD4 |  |
| DC_7A-32A_n78A | n78 |  | 3560.5 |  | 10 |  |  | 50 | 3560.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2517.5 |  | 5 |  |  | 25 | 2637.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1474.5 |  |  |  |  | 17.6 |  |  |  | IMD3 |  |
|  | n78 |  | 3311 |  | 10 |  |  | 50 | 3311 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 32 |  | N/A |  | 5 |  |  | N/A | 1492 |  |  |  |  | 4.9 |  |  |  | IMD4 |  |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2650 |  |  |  |  | 32.1 |  |  |  | IMD3 |  |
|  | 40 |  | 2310 |  | 5 |  |  | 25 | 2310 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 |  | N/A |  | 5 |  |  | N/A | 2630 |  |  |  |  | 10.1 |  |  |  | IMD4 |  |
|  | 40 |  | 2310 |  | 5 |  |  | 25 | 2310 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3625 |  | 10 |  |  | 50 | 3625 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2510 |  | 5 |  |  | 25 | 2630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2310 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | n78 |  | 3785 |  | 10 |  |  | 50 | 3785 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-46A_n78A6 | 7 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD2, IMD5 |  |
|  | n78 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 |  | N/A |  | 10 |  |  | N/A | 2625 |  |  |  |  | 30.0 |  |  |  | IMD26 |  |
|  | 66 |  | 1775 |  | 10 |  |  | 50 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 846.5 |  | 5 |  |  | 25 | 891.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 |  | N/A |  | 10 |  |  | N/A | 2675 |  |  |  |  | 15 |  |  |  | IMD4 |  |
|  | 66 |  | 1730 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2515 |  | 10 |  |  | 50 | 2635 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-66A_n28A | 7 |  | N/A |  | 5 |  |  | N/A | 2685 |  |  |  |  | 18.0 |  |  |  | IMD3 |  |
|  | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 745 |  | 5 |  |  | 25 | 800 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | n77 |  | 3625 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2670 |  |  |  |  | 5.2 |  |  |  | IMD5 |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 | 4190 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 5 |  |  | N/A | 2640 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | n77 |  | 3900 |  | 10 |  |  | 50 | 3900 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | n77 |  | 3625 |  | 10 |  |  | 50 | 3625 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 |  | 2542 |  | 5 |  |  | 25 | 2662 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3344 |  |  |  |  | 16.0 |  |  |  | IMD3 |  |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 4040 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 |  | 2550 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66/n66 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 8.7 |  |  |  | IMD4 |  |
|  | n78 |  | 3625 |  | 10 |  |  | 50 | 3475 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 |  | 2542 |  | 5 |  |  | 25 | 2662 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3344 |  |  |  |  | 16.0 |  |  |  | IMD3 |  |
| DC_7A-71A_n2A | n2 |  | 1859 |  | 5 |  |  | 25 | 1933 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2505 |  | 5 |  |  | 25 | 2625 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  | N/A |  | 5 |  |  | N/A | 646 |  |  |  |  | 30.8 |  |  |  | IMD2 |  |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | 7 |  | N/A |  | 5 |  |  | N/A | 2670 |  |  |  |  | 29.6 |  |  |  | IMD2 |  |
|  | 71 |  | 680 |  | 5 |  |  | 25 | 634 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | 2540 |  | 5 |  |  | 25 | 2660 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  | N/A |  | 5 |  |  | N/A | 640 |  |  |  |  | 3.0 |  |  |  | IMD5 |  |
|  | n78 |  | 3490 |  | 10 |  |  | 50 | 3490 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_n71A-n78A | 7 |  | 2550 |  | 5 |  |  | 25 | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3714 |  |  |  |  | 9.7 |  |  |  | IMD4 |  |
|  | 7 |  | 2555 |  | 5 |  |  | 25 | 2675 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3520 |  | 10 |  |  | 50 | 3520 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | N/A |  | 5 |  |  | N/A | 625 |  |  |  |  | 3.9 |  |  |  | IMD5 |  |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 |  | 2520 |  | 5 |  |  | 25 | 2640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3600 |  | 10 |  |  | 50 | 3600 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 10 |  |  | N/A | 4680 |  |  |  |  | 20.6 |  |  |  | IMD34,9,13 |  |
|  | 7 |  | 2565 |  | 5 |  |  | 25 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3770 |  |  |  |  | 6.4 |  |  |  | IMD413 |  |
|  | n79 |  | 4450 |  | 10 |  |  | 50 | 4450 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_7A_SUL_n78A-n80A | n80 |  | 1730 |  | 5 |  |  | 25 |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 7 |  | N/A |  | 10 |  |  | N/A | 2655 |  |  |  |  | 13 |  |  |  | IMD4 |  |
| DC_8A_n1A-n28A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1965 |  | 5 |  |  | 25 | 2155 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 765 |  |  |  |  | 11.6 |  |  |  | IMD4 |  |
| DC_8A_n1A-n40A | 8 |  | 885 |  | 5 |  |  | 25 | 930 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2395 |  | 5 |  |  | 25 | 2395 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2135 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_8A_n1A-n77A | 8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1945 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3745 |  |  |  |  | 14.9 |  |  |  | IMD31 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3960 |  | 10 |  |  | 50 | 3960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 14.4 |  |  |  | IMD3 |  |
| DC_8A_n1A-n78A | 8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1945 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3745 |  |  |  |  | 14.9 |  |  |  | IMD3 |  |
| DC_8A-(n)3AA | 8 |  | 897.5 |  | 5 |  |  | 25 | 942.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 3 |  | N/A |  | 5 |  |  | N/A | 1835 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | n3 |  | 1747.5 |  | 10 |  |  | 50 | 1842.5 |  |  |  |  | 6.4 |  |  |  | IMD5 |  |
| DC_8A_n3A-n28A | 8 |  | 912.5 |  | 5 |  |  | 25 | 957.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1712.5 |  | 5 |  |  | 25 | 1807.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 800 |  |  |  |  | 30.4 |  |  |  | IMD2 |  |
| DC_8A_n3A-n77A DC_8A_n3A-n77(2A) | 8 |  | 900 |  | 5 |  |  | 25 | 945 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1740 |  | 5 |  |  | 25 | 1835 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3540 |  |  |  |  | 16.3 |  |  |  | IMD3 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3640 |  | 10 |  |  | 50 | 3640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
| DC_8A_n3A-n78A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3550 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1730 |  | 5 |  |  | 25 | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3370 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1820 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | n78 |  | 3640 |  | 10 |  |  | 50 | 3640 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_8A_n3A-n79A | 8 |  | 885 |  | 5 |  |  | 25 | 930 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4425 |  |  |  |  | 15.7 |  |  |  | IMD39 |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4580 |  | 40 |  |  | 216 | 4580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1850 |  |  |  |  | 8.8 |  |  |  | IMD4 |  |
| DC_8A-11A_n1A | 11 |  | 1435 |  | 5 |  |  | 25 | 1483 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1940 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 16.6 |  |  |  | IMD35 |  |
| DC_8A-11A_n77A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3311 |  | 10 |  |  | 50 | 3311 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1491 |  |  |  |  | 18.8 |  |  |  | IMD3 |  |
| DC_8A-11A_n77A | 11 |  | 1430.5 |  | 5 |  |  | 25 | 1478.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3791 |  | 10 |  |  | 50 | 3791 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 18.2 |  |  |  | IMD3 |  |
| DC_8A-11A_n78A | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3311 |  | 10 |  |  | 50 | 3311 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1491 |  |  |  |  | 18.8 |  |  |  | IMD3 |  |
| DC_8A-11A_n78A | 11 |  | 1430.5 |  | 5 |  |  | 25 | 1478.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3791 |  | 10 |  |  | 50 | 3791 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 18.2 |  |  |  | IMD3 |  |
| DC_8A-11A_n79A | 8 |  | 882.5 |  | 5 |  |  | 25 | 927.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4980 |  | 40 |  |  | 216 | 4980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 11 |  | N/A |  | 5 |  |  | N/A | 1478.4 |  |  |  |  | 1.2 |  |  |  | IMD5 |  |
| DC_8A-11A_n79A | 11 |  | 1435 |  | 5 |  |  | 25 | 1483 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4810 |  | 40 |  |  | 216 | 4810 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 2.8 |  |  |  | IMD5 |  |
| DC_8-20_n1 | n1 |  | 1925 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 805 |  |  |  |  | 11.5 |  |  |  | IMD4 |  |
| DC_8-20_n3 | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 810 |  |  |  |  | 27 |  |  |  | IMD24 |  |
|  | n3 |  | 1770 |  | 5 |  |  | 25 | 1865 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 930 |  |  |  |  | 27 |  |  |  | IMD24 |  |
|  | 20 |  | 840 |  | 5 |  |  | 25 | 799 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_8A-20A_n28A | 8 |  | N/A |  | 5 |  |  | N/A | 946 |  |  |  |  | [23.5] |  |  |  | IMD3 |  |
|  | 20 |  | 837 |  | 5 |  |  | 25 | 796 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 728 |  | 5 |  |  | 25 | 773 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_8A-20A_n78A | 8 |  | 890 |  | 5 |  |  | 25 | 935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3470 |  | 10 |  |  | 50 | 3470 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | N/A |  | 5 |  |  | N/A | 800 |  |  |  |  | 12.1 |  |  |  | IMD4 |  |
|  | 8 |  | N/A |  | 5 |  |  | N/A | 940 |  |  |  |  | 12.1 |  |  |  | IMD4 |  |
|  | n78 |  | 3481 |  | 10 |  |  | 50 | 3481 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 20 |  | 847 |  | 5 |  |  | 25 | 806 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_8A_n28A-n77A |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 |  | N/A |  |  | N/A |  |  |  |
|  |  | n28 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3473 | 10.3 |  |  |  |  | IMD4 |  |  |
|  |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 765 | 11.6 |  |  |  |  | IMD4 |  |  |
|  |  | n77 |  | 3495 |  | 10 | 50 |  |  |  | 3495 | N/A |  |  |  |  | N/A |  |  |
| DC_8A_n28A-n78A |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 725 |  | 5 | 25 |  |  |  | 780 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3455 | 10.3 |  |  |  |  | IMD4 |  |  |
|  |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 765 | 11.6 |  |  |  |  | IMD4 |  |  |
|  |  | n78 |  | 3495 |  | 10 | 50 |  |  |  | 3495 | N/A |  |  |  |  | N/A |  |  |
| DC_8A_n28A-n79A |  | 8 |  | 912.5 |  | 5 | 25 |  |  |  | 957.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 745.5 |  | 5 | 25 |  |  |  | 800.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4420 | 0.0 |  |  |  |  | IMD5 |  |  |
|  |  | 8 |  | 905 |  | 5 | 25 |  |  |  | 950 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4420 |  | 40 | 216 |  |  |  | 4420 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 800 | 3.9 |  |  |  |  | IMD5 |  |  |
| DC_8A-32A_n78A |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3311 |  | 10 | 50 |  |  |  | 3311 | N/A |  |  |  |  | N/A |  |  |
|  |  | 32 |  | N/A |  | 5 | N/A |  |  |  | 1491 | 18.8 |  |  |  |  | IMD3 |  |  |
| DC_8A_n39A-n79A |  | 8 |  | 900 |  | 5 | 25 |  |  |  | 945 | N/A |  |  |  |  | N/A |  |  |
|  |  | n39 |  | 1890 |  | 10 | 50 |  |  |  | 1890 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4680 | 15.9 |  |  |  |  | IMD3 |  |  |
| DC_8A_n39A-n79A |  | 8 |  | 890 |  | 5 | 25 |  |  |  | 935 | N/A |  |  |  |  | N/A |  |  |
|  |  | n39 |  | 1890 |  | 10 | 50 |  |  |  | 1890 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4560 | 12.1 |  |  |  |  | IMD4 |  |  |
| DC_8A_n39A-n79A |  | 8 |  | 897.5 |  | 5 | 25 |  |  |  | 942.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n39 |  | N/A |  | 10 | N/A |  |  |  | 1907.5 | 13.8 |  |  |  |  | IMD4 |  |  |
|  |  | n79 |  | 4600 |  | 40 | 216 |  |  |  | 4600 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-40A_n1A DC_8A-40C_n1A |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 930 | 8.0 |  |  |  |  | IMD4 |  |  |
|  |  | 40 |  | 2395 |  | 5 | 25 |  |  |  | 2395 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1930 |  | 5 | 25 |  |  |  | 2120 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-40A_n78A DC_8A-40C_n78A |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 950 | 30.5 |  |  |  |  | IMD2 |  |  |
|  |  | 40 |  | 2380 |  | 5 | 25 |  |  |  | 2380 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3330 |  | 10 | 50 |  |  |  | 3330 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 935 | 19.8 |  |  |  |  | IMD3 |  |  |
|  |  | 40 |  | 2320 |  | 5 | 25 |  |  |  | 2320 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3705 |  | 10 | 50 |  |  |  | 3705 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | 40 |  | N/A |  | 5 | N/A |  |  |  | 2395 | 28 |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | 3305 |  | 10 | 50 |  |  |  | 3305 | N/A |  |  |  |  | N/A |  |  |
| DC_8A_n40A-n79A |  | 8 |  | 885 |  | 5 | 25 |  |  |  | 930 | N/A |  |  |  |  | N/A |  |  |
|  |  | n40 |  | 2305 |  | 5 | 25 |  |  |  | 2305 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4960 | 10.7 |  |  |  |  | IMD4 |  |  |
|  |  | 8 |  | 885 |  | 5 | 25 |  |  |  | 930 | N/A |  |  |  |  | N/A |  |  |
|  |  | n40 |  | N/A |  | 5 | N/A |  |  |  | 2305 | 9.2 |  |  |  |  | IMD4 |  |  |
|  |  | n79 |  | 4960 |  | 40 | 216 |  |  |  | 4960 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-41A_n1A |  | 41 |  | 2500 |  | 5 | 25 |  |  |  | 2500 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-41C_n1A |  | n1 |  | 1977 |  | 5 | 25 |  |  |  | 2167 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 931 | 4.5 |  |  |  |  | IMD5 |  |  |
| DC_8A-41A_n3A |  | n3 |  | 1780 |  | 5 | 25 |  |  |  | 1875 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-41C_n3A |  | 8 |  | 885 |  | 5 | 25 |  |  |  | 930 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2665 | 27.4 |  |  |  |  | IMD21 |  |  |
|  |  | n3 |  | 1715 |  | 5 | 25 |  |  |  | 1810 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 950 | 28.9 |  |  |  |  | IMD21 |  |  |
|  |  | 41 |  | 2665 |  | 5 | 25 |  |  |  | 2665 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-41A_n77A |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 950 | 29.1 |  |  |  |  | IMD21, 4 |  |  |
| DC_8A-41C_n77A |  | 41 |  | 2630 |  | 10 | 50 |  |  |  | 2630 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3580 |  | 10 | 50 |  |  |  | 3580 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | 895 |  | 5 | 25 |  |  |  | 940 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2650 | 28.0 |  |  |  |  | IMD2 |  |  |
|  |  | n77 |  | 3545 |  | 10 | 50 |  |  |  | 3545 |  | N/A |  |  | N/A |  |  |  |
| DC_8A_n41A-n79A |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 |  | N/A |  |  | N/A |  |  |  |
|  |  | n41 |  | 2650 |  | 10 | 50 |  |  |  | 2650 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4470 | 16.3 |  |  |  |  | IMD3 |  |  |
|  |  | 8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | N/A |  | 10 | N/A |  |  |  | 2650 | 15.5 |  |  |  |  | IMD3 |  |  |
|  |  | n79 |  | 4470 |  | 40 | 216 |  |  |  | 4470 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-42A_n1A |  | 42 |  | 3405 |  | 10 | 50 |  |  |  | 3405 | N/A |  |  |  |  | N/A |  |  |
| DC_8A-42C_n1A |  | n1 |  | 1955 |  | 5 | 25 |  |  |  | 2145 | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 945 | 3.3 |  |  |  |  | IMD5 |  |  |
| DC_8A-42A_n3A |  | 8 |  | 900 |  | 5 | 25 |  |  |  | 945 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1740 |  | 5 | 25 |  |  |  | 1835 | N/A |  |  |  |  | N/A |  |  |
|  |  | 42 |  | N/A |  | 5 | N/A |  |  |  | 3540 | 16.3 |  |  |  |  | IMD3 |  |  |
| DC_8A-42A_n28A |  | 8 |  | 900 |  | 5 | 25 |  |  |  | 945 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | 42 |  | N/A |  | 5 | N/A |  |  |  | 3443 | 8.7 |  |  |  |  | IMD4 |  |  |
| DC_8A_SUL_n78A-n80A |  | n80 |  | 1755 |  | 10 | 50 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | N/A |  | 5 | N/A |  |  |  | 945 | 8 |  |  |  |  | IMD4 |  |  |
|  |  | n80 |  | 1750 |  | 10 | 50 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |
|  |  | 8 |  | 900 |  | 5 | 25 |  |  |  | 945 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3550 | 8 |  |  |  |  | IMD33 |  |  |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) |  | 11 |  | 1435 |  | 5 | 25 |  |  |  | 1483 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1940 |  | 5 | 25 |  |  |  | 2130 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3375 | 29.6 |  |  |  |  | IMD21 |  |  |
|  |  | 11 |  | 1438 |  | 5 | 25 |  |  |  | 1486 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3578 |  | 10 | 50 |  |  |  | 3578 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | N/A |  | 5 | N/A |  |  |  | 2140 | 30.8 |  |  |  |  | IMD21 |  |  |
| DC_11A_n3A-n28A |  | 11 |  | 1435 |  | 5 | 25 |  |  |  | 1483 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1753 |  | 5 | 25 |  |  |  | 1848 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 800 | 3.0 |  |  |  |  | IMD5 |  |  |
| DC_11A_n3A-n77A DC_11A_n3A-n77(2A) |  | 11 |  | 1440 |  | 5 | 25 |  |  |  | 1488 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1740 |  | 5 | 25 |  |  |  | 1835 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3780 | 10.8 |  |  |  |  | IMD4 |  |  |
|  |  | 11 |  | 1440 |  | 5 | 25 |  |  |  | 1488 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1870 | 29.0 |  |  |  |  | IMD2 |  |  |
|  |  | n77 |  | 3310 |  | 10 | 50 |  |  |  | 3310 | N/A |  |  |  |  | N/A |  |  |
| DC_11A_n3A-n79A |  | 11 |  | 1435 |  | 5 | 25 |  |  |  | 1483 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1770 |  | 5 | 25 |  |  |  | 1865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4640 | 16.2 |  |  |  |  | IMD3 |  |  |
|  |  | 11 |  | 1435 |  | 5 | 25 |  |  |  | 1483 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4735 |  | 40 | 216 |  |  |  | 4735 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1865 | 17.8 |  |  |  |  | IMD3 |  |  |
| DC_11A-18A_n77A |  | 11 |  | 1443 |  | 5 | 25 |  |  |  | 1491 | N/A |  |  |  |  | N/A |  |  |
| DC_11A-18A_n77(2A) |  | n77 |  | 3706 |  | 10 | 50 |  |  |  | 3706 | N/A |  |  |  |  | N/A |  |  |
|  |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 18.7 |  |  |  |  | IMD3 |  |  |
| DC_11A-18A_n78A |  | 11 |  | 1443 |  | 5 | 25 |  |  |  | 1491 | N/A |  |  |  |  | N/A |  |  |
| DC_11A-18A_n78(2A) |  | n78 |  | 3706 |  | 10 | 50 |  |  |  | 3706 | N/A |  |  |  |  | N/A |  |  |
|  |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 18.7 |  |  |  |  | IMD3 |  |  |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) |  | 11 |  | 1443 |  | 5 | 25 |  |  |  | 1491 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3629 | 17.5 |  |  |  |  | IMD3 |  |  |
|  |  | 11 |  | 1443 |  | 5 | 25 |  |  |  | 1491 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3684 |  | 10 | 50 |  |  |  | 3684 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 798 | 15.8 |  |  |  |  | IMD3 |  |  |
| DC_12A_n2A-n38A |  | 12 |  | 708 |  | 5 | 25 |  |  |  | 738 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | 1900 |  | 5 | 25 |  |  |  | 1980 | N/A |  |  |  |  | N/A |  |  |
|  |  | n38 |  | N/A |  | 10 | N/A |  |  |  | 2608 | 28.7 |  |  |  |  | IMD2 |  |  |
| DC_12A_n2A-n41A |  | 12 |  | 708 |  | 5 | 25 |  |  |  | 738 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | 1900 |  | 5 | 25 |  |  |  | 1980 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | N/A |  | 5 | N/A |  |  |  | 2608 | 28.7 |  |  |  |  | IMD2 |  |  |
| DC_12_n2-n78 |  | 12 |  | 707.5 |  | 5 | 25 |  |  |  | 737.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | N/A |  | 5 | N/A |  |  |  | 1960 | 16.5 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3375 |  | 10 | 50 |  |  |  | 3375 | N/A |  |  |  |  | N/A |  |  |
|  |  | 12 |  | 707.5 |  | 5 | 25 |  |  |  | 737.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | 1900 |  | 5 | 25 |  |  |  | 1980 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3315 | 16.0 |  |  |  |  | IMD3 |  |  |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) |  | 12 |  | 708 |  | 5 | 25 |  |  |  | 738 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2520 |  | 5 | 25 |  |  |  | 2640 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3624 | 9 |  |  |  |  | IMD4 |  |  |
|  |  | 12 |  | 708 |  | 5 | 25 |  |  |  | 738 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3370 |  | 10 | 50 |  |  |  | 3370 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | N/A |  | 5 | N/A |  |  |  | 2662 | 29.6 |  |  |  |  | IMD2 |  |  |
| DC_12A-30A_n2A |  | 12 |  | 708.5 |  | 5 | 25 |  |  |  | 738.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 30 |  | N/A |  | 5 | N/A |  |  |  | 2353 | 12.0 |  |  |  |  | IMD4 |  |  |
|  |  | n2 |  | 1885 |  | 5 | 25 |  |  |  | 1965 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-30A_n5A |  | 12 |  | 702 |  | 5 | 25 |  |  |  | 732 | N/A |  |  |  |  | N/A |  |  |
|  |  | 30 |  | N/A |  | 5 | N/A |  |  |  | 2355 | 18.8 |  |  |  |  | IMD3 |  |  |
|  |  | n5 |  | 826.5 |  | 5 | 25 |  |  |  | 871.5 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-30A_n77A DC_12A-30A_n77(2A) |  | 12 |  | N/A |  | 5 | N/A |  |  |  | 740 | 15.2 |  |  |  |  | IMD34 |  |  |
|  |  | 30 |  | 2310 |  | 5 | 25 |  |  |  | 2355 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3880 |  | 10 | 50 |  |  |  | 3880 | N/A |  |  |  |  | N/A |  |  |
|  |  | 12 |  | 707.5 |  | 5 | 25 |  |  |  | 737.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 30 |  | N/A |  | 5 | N/A |  |  |  | 2355 | 13.2 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3770 |  | 10 | 50 |  |  |  | 3770 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-66A_n5A DC_12A-66A-66A_n5A |  | 12 |  | N/A |  | 5 | N/A |  |  |  | 742 | 9.4 |  |  |  |  | IMD4 |  |  |
|  |  | 66 |  | 1745 |  | 5 | 25 |  |  |  | 2145 | N/A |  |  |  |  | N/A |  |  |
|  |  | n5 |  | 829 |  | 5 | 25 |  |  |  | 874 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-66A_n77A DC_12A-66A_n77(2A) |  | 12 |  | N/A |  | 5 | N/A |  |  |  | 740 | 15.2 |  |  |  |  | IMD311 |  |  |
| DC_12A-66A-66A_n77A |  | 66 |  | 1720 |  | 5 | 25 |  |  |  | 2120 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 4180 |  | 10 | 50 |  |  |  | 4180 | N/A |  |  |  |  | N/A |  |  |
|  |  | 12 |  | 707 |  | 5 | 25 |  |  |  | 737 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2126 | 13.2 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3540 |  | 10 | 50 |  |  |  | 3540 | N/A |  |  |  |  | N/A |  |  |
| DC_13A_n2A-n77A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | 1896 |  | 5 | 25 |  |  |  | 1976 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3460 | 17.3 |  |  |  |  | IMD3 |  |  |
|  |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n2 |  | N/A |  | 5 | N/A |  |  |  | 1960 | 16.0 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3524 |  | 10 | 50 |  |  |  | 3524 | N/A |  |  |  |  | N/A |  |  |
| DC_13A_n5A-n77A11 |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 4013 |  | 10 | 50 |  |  |  | 4013 | N/A |  |  |  |  | N/A |  |  |
|  |  | n5 |  | N/A |  | 5 | N/A |  |  |  | 885 | 4.5 |  |  |  |  | IMD5 |  |  |
| DC_13A_n25A-n66A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n25 |  | 1860 |  | 5 | 25 |  |  |  | 1940 | N/A |  |  |  |  | N/A |  |  |
|  |  | n66 |  | N/A |  | 5 | N/A |  |  |  | 2156 | 7.2 |  |  |  |  | IMD4 |  |  |
| DC_13A_n25A-n66A |  | 13 |  | 780 |  | 5 | 25 |  |  |  | 749 | N/A |  |  |  |  | N/A |  |  |
|  |  | n25 |  | N/A |  | 5 | N/A |  |  |  | 1940 | 6.2 |  |  |  |  | IMD4 |  |  |
|  |  | n66 |  | 1750 |  | 5 | 25 |  |  |  | 2150 | N/A |  |  |  |  | N/A |  |  |
| DC_13A_n48A-n66A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n48 |  | N/A |  | 5 | N/A |  |  |  | 3584 | 2.8 |  |  |  |  | IMD5 |  |  |
|  |  | n66 |  | 1716 |  | 5 | 25 |  |  |  | 2116 | N/A |  |  |  |  | N/A |  |  |
|  |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n48 |  | 3695 |  | 5 | 25 |  |  |  | 3695 | N/A |  |  |  |  | N/A |  |  |
|  |  | n66 |  | N/A |  | 5 | N/A |  |  |  | 2131 | 17.1 |  |  |  |  | IMD3 |  |  |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66B_n2A |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2156 | 7..2 |  |  |  |  | IMD4 |  |  |
| DC_13A-66C_n2A |  | n2 |  | 1860 |  | 5 | 25 |  |  |  | 1940 | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66A_n5A |  | 13 |  | N/A |  | 5 | N/A |  |  |  | 750 | 9.4 |  |  |  |  | IMD4 |  |  |
| DC_13A-66A-66A_n5A |  | 66 |  | 1770 |  | 5 | 25 |  |  |  | 2170 | N/A |  |  |  |  | N/A |  |  |
|  |  | n5 |  | 840 |  | 5 | 25 |  |  |  | 885 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-66A_n25A |  | 12 |  | 708.5 |  | 5 | 25 |  |  |  | 738.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | 1775 |  | 5 | 25 |  |  |  | 2175 | N/A |  |  |  |  | N/A |  |  |
|  |  | n25 |  | N/A |  | 5 | N/A |  |  |  | 1935 | 20 |  |  |  |  | IMD3 |  |  |
|  |  | 12 |  | 708.5 |  | 5 | 25 |  |  |  | 738.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2150 | 4 |  |  |  |  | IMD5 |  |  |
|  |  | n25 |  | 1883.3 |  | 5 | 25 |  |  |  | 1963.3 | N/A |  |  |  |  | N/A |  |  |
|  |  | 12 |  | 708.5 |  | 5 | 25 |  |  |  | 738.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2112.5 | 23 |  |  |  |  | IMD3 |  |  |
|  |  | n25 |  | 1912.5 |  | 5 | 25 |  |  |  | 1992.5 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-66A_n41A |  | 12 |  | N/A |  | 5 | N/A |  |  |  | 742 | 31 |  |  |  |  | IMD2 |  |  |
|  |  | 66 |  | 1773 |  | 5 | 25 |  |  |  | 2173 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | 2515 |  | 5 | 25 |  |  |  | 2515 | N/A |  |  |  |  | N/A |  |  |
| DC_12A-66A_n78A |  | 12 |  | 710 |  | 5 | 25 |  |  |  | 740 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2160 | 17.1 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3580 |  | 10 | 50 |  |  |  | 3580 | N/A |  |  |  |  | N/A |  |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) |  | 12 |  | 703 |  | 5 | 25 |  |  |  | 733 | N/A |  |  |  |  | N/A |  |  |
|  |  | n66 |  | N/A |  | 5 | N/A |  |  |  | 2140 | 16.5 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3546 |  | 10 | 50 |  |  |  | 3546 | N/A |  |  |  |  | N/A |  |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) |  | 12 |  | 703 |  | 5 | 25 |  |  |  | 733 | N/A |  |  |  |  | N/A |  |  |
|  |  | n66 |  | 1720 |  | 5 | 25 |  |  |  | 2120 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3754 | 4.1 |  |  |  |  | IMD5 |  |  |
| DC_13A_n7A-n78A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3432 |  | 10 | 50 |  |  |  | 3432 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | N/A |  | 5 | N/A |  |  |  | 2650 | 27.9 |  |  |  |  | IMD2 |  |  |
| DC_13A_n7A-n78A |  | 13 |  | 749 |  | 5 | 25 |  |  |  | 780 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2560 |  | 5 | 25 |  |  |  | 2680 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3622 | 9 |  |  |  |  | IMD4 |  |  |
| DC_13A_n7A-n78A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2530 |  | 5 | 25 |  |  |  | 2650 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3312 | 29.0 |  |  |  |  | IMD2 |  |  |
| DC_13A-46A_n2A5 |  | 13 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 46 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD4 |  |  |
|  |  | n2 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
| DC_13A-46A_n66A5 |  | 13 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 46 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD4, IMD5 |  |  |
|  |  | n66 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
| DC_13A-46A_n77A5 DC_13A-46A-46A_n77A5 |  | 13 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 46 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD3, IMD4, IMD5 |  |  |
|  |  | n77 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2131 | 17.1 |  |  |  |  | IMD3 |  |  |
|  |  | n48 |  | 3695 |  | 5 | 25 |  |  |  | 3695 | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66A_n77A |  | 13 |  | 782 |  | 5 | 25 |  |  |  | 751 | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66A_n77C DC_13A-66A-66A_n77A DC_13A-66A-66A_n77C |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2156 | 17.1 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3720 |  | 10 | 50 |  |  |  | 3720 | N/A |  |  |  |  | N/A |  |  |
| DC_13A-66A_n77A11 |  | 13 |  | N/A |  | 5 | N/A |  |  |  | 750 | 15.2 |  |  |  |  | IMD3 |  |  |
| DC_13A-66A_n77C11 DC_13A-66A-66A_n77A11 DC_13A-66A-66A_n77C11 |  | 66 |  | 1710 |  | 5 | 25 |  |  |  | 2110 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 4170 |  | 10 | 50 |  |  |  | 4170 | N/A |  |  |  |  | N/A |  |  |
| DC_14A-30A_n5A |  | 14 |  | 795 |  | 5 | 25 |  |  |  | 765 | N/A |  |  |  |  | N/A |  |  |
|  |  | 30 |  | N/A |  | 5 | N/A |  |  |  | 2353 | 5.9 |  |  |  |  | IMD5 |  |  |
|  |  | n5 |  | 827 |  | 5 | 25 |  |  |  | 872 | N/A |  |  |  |  | N/A |  |  |
| DC_14A-30A_n77A DC_14A-30A_n77(2A) |  | 14 |  | N/A |  | 5 | N/A |  |  |  | 763 | 15.2 |  |  |  |  | IMD34 |  |  |
|  |  | 30 |  | 2310 |  | 5 | 25 |  |  |  | 2355 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3857 |  | 10 | 50 |  |  |  | 3857 | N/A |  |  |  |  | N/A |  |  |
|  |  | 14 |  | 793 |  | 5 | 25 |  |  |  | 763 | N/A |  |  |  |  | N/A |  |  |
|  |  | 30 |  | N/A |  | 5 | N/A |  |  |  | 2355 | 13.2 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3941 |  | 10 | 50 |  |  |  | 3941 | N/A |  |  |  |  | N/A |  |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A |  | 14 |  | 793 |  | 5 | 25 |  |  |  | 763 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2162 | 7.6 |  |  |  |  | IMD4 |  |  |
|  |  | n2 |  | 1874 |  | 5 | 25 |  |  |  | 1954 | N/A |  |  |  |  | N/A |  |  |
| DC_14A-66A_n5A |  | 14 |  | N/A |  | 5 | N/A |  |  |  | 762 | 9.4 |  |  |  |  | IMD4 |  |  |
|  |  | 66 |  | 1740 |  | 5 | 25 |  |  |  | 2140 | N/A |  |  |  |  | N/A |  |  |
|  |  | n5 |  | 834 |  | 5 | 25 |  |  |  | 879 | N/A |  |  |  |  | N/A |  |  |
| DC_14A-66A_n77A DC_14A-66A_n77(2A) |  | 14 |  | N/A |  | 5 | N/A |  |  |  | 763 | 15.2 |  |  |  |  | IMD311 |  |  |
| DC_14A-66A-66A_n77A |  | 66 |  | 1712.5 |  | 5 | 25 |  |  |  | 2112.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 4188 |  | 10 | 50 |  |  |  | 4188 | N/A |  |  |  |  | N/A |  |  |
|  |  | 14 |  | 793 |  | 5 | 25 |  |  |  | 763 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2155 | 13.2 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3741 |  | 10 | 50 |  |  |  | 3741 | N/A |  |  |  |  | N/A |  |  |
| DC_18A_n3A-n41A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1720 |  | 5 | 25 |  |  |  | 1815 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | N/A |  | 10 | N/A |  |  |  | 2540 | 29.4 |  |  |  |  | IMD2 |  |  |
|  |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | 2670 |  | 10 | 50 |  |  |  | 2670 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1850 | 28.2 |  |  |  |  | IMD2 |  |  |
| DC_18A_n3A-n77A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1770 |  | 5 | 25 |  |  |  | 1865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 3410 | 16.3 |  |  |  |  | IMD3 |  |  |
|  |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1865 | 15.7 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3505 |  | 10 | 50 |  |  |  | 3505 | N/A |  |  |  |  | N/A |  |  |
| DC_18A_n3A-n78A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1750 |  | 5 | 25 |  |  |  | 1845 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3390 | 15.2 |  |  |  |  | IMD33 |  |  |
| DC_18A-28A_n77A DC_18A_n28A-n77A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28/n28 |  | N/A |  | 5 | N/A |  |  |  | 778 | 4.4 |  |  |  |  | IMD5 |  |  |
|  |  | n77 |  | 4058 |  | 10 | 50 |  |  |  | 4058 | N/A |  |  |  |  | N/A |  |  |
| DC_18A-28A_n77A |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 3.9 |  |  |  |  | IMD5 |  |  |
|  |  | 28 |  | 723 |  | 5 | 25 |  |  |  | 778 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3757 |  | 10 | 50 |  |  |  | 3757 | N/A |  |  |  |  | N/A |  |  |
| DC_18A-28A_n78A |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 864 | 3.8 |  |  |  |  | IMD5 |  |  |
|  |  | 28 |  | 723 |  | 5 | 25 |  |  |  | 778 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3756 |  | 10 | 50 |  |  |  | 3756 | N/A |  |  |  |  | N/A |  |  |
| DC_18A_n28A-n77A DC_18A_n28A-n78A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 710 |  | 5 | 25 |  |  |  | 765 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77/n78 |  | N/A |  | 10 | N/A |  |  |  | 3770 | 4.0 |  |  |  |  | IMD5 |  |  |
| DC_18A-41A_n3A DC_18A-41C_n3A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1725 |  | 5 | 25 |  |  |  | 1820 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2630 | 16.0 |  |  |  |  | IMD3 |  |  |
|  |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 28.9 |  |  |  |  | IMD21 |  |  |
|  |  | n3 |  | 1765 |  | 5 | 25 |  |  |  | 1860 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | 2630 |  | 5 | 25 |  |  |  | 2630 | N/A |  |  |  |  | N/A |  |  |
| DC_18A_n41A-n77A DC_18A_n41A-n77(2A) DC_18A_n41A-n78A DC_18A_n41A-n78(2A) |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 3.4 |  |  |  |  | IMD5 |  |  |
|  |  | n77 |  | 3527.5 |  | 10 | 50 |  |  |  | 3527.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | 2640 |  | 5 | 25 |  |  |  | 2640 | N/A |  |  |  |  | N/A |  |  |
| DC_18A_n41A-n77A DC_18A_n41A-n78A |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | 2570 |  | 5 | 25 |  |  |  | 2570 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77/n78 |  | N/A |  | 10 | N/A |  |  |  | 3390 | 30.1 |  |  |  |  | IMD2 |  |  |
|  |  | 18 |  | 820 |  | 5 | 25 |  |  |  | 865 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77/n78 |  | 3450 |  | 10 | 50 |  |  |  | 3450 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | N/A |  | 5 | N/A |  |  |  | 2630 | 28.5 |  |  |  |  | IMD2 |  |  |
| DC_18A-41A_n78A DC_18A-41C_n78A |  | 18 |  | N/A |  | 5 | N/A |  |  |  | 865 | 3.4 |  |  |  |  | IMD5 |  |  |
|  |  | n78 |  | 3527.5 |  | 10 | 50 |  |  |  | 3527.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | 2640 |  | 5 | 25 |  |  |  | 2640 | N/A |  |  |  |  | N/A |  |  |
| DC_19A_n1A-n77A DC_19A_n1A-n78A |  | 19 |  | 840 |  | 5 | 25 |  |  |  | 885 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1975 |  | 5 | 25 |  |  |  | 2165 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77/n78 |  | 3655 |  | 10 | 50 |  |  |  | 3655 | [21.4] |  |  |  |  | IMD3 |  |  |
|  |  | 19 |  | 832.5 |  | 5 | 25 |  |  |  | 877.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | N/A |  | 5 | N/A |  |  |  | 2130 | 17.8 |  |  |  |  | IMD3 |  |  |
|  |  | n77/n78 |  | 3795 |  | 10 | 50 |  |  |  | 3795 | N/A |  |  |  |  | N/A |  |  |
| DC_19A-21A_n77A DC_19A-21A_n78A |  | 19 |  | N/A |  | 5 | N/A |  |  |  | 882.5 | 18.7 |  |  |  |  | IMD3 |  |  |
|  |  | 21 |  | 1450.4 |  | 5 | 25 |  |  |  | 1498.4 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77, n78 |  | 3783.3 |  | 10 | 50 |  |  |  | 3783.3 | N/A |  |  |  |  | N/A |  |  |
|  |  | 19 |  | N/A |  | 5 | N/A |  |  |  | 882.5 | 13.2 |  |  |  |  | IMD4 |  |  |
|  |  | 21 |  | 1450.4 |  | 5 | 25 |  |  |  | 1498.4 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77, n78 |  | 3468.7 |  | 10 | 50 |  |  |  | 3468.7 | N/A |  |  |  |  | N/A |  |  |
| DC_19A-21A_n77A |  | 19 |  | 837.5 |  | 5 | 25 |  |  |  | 882.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 21 |  | N/A |  | 5 | N/A |  |  |  | 1502.5 | 9.0 |  |  |  |  | IMD4 |  |  |
|  |  | n77 |  | 4015 |  | 10 | 50 |  |  |  | 4015 | N/A |  |  |  |  | N/A |  |  |
| DC_19A-21A_n79A |  | 19 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD5 |  |  |
|  |  | 21 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 19 |  | 837.5 |  | 5 | 25 |  |  |  | 882.2 | N/A |  |  |  |  | N/A |  |  |
|  |  | 21 |  | N/A |  | 5 | N/A |  |  |  | 1500 | 3.8 |  |  |  |  | IMD5 |  |  |
|  |  | n79 |  | 4850 |  | 40 | 216 |  |  |  | 4850 | N/A |  |  |  |  | N/A |  |  |
| DC_20A_n1A-n78A |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1940 |  | 5 | 25 |  |  |  | 2130 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3630 | 16.0 |  |  |  |  | IMD3 |  |  |
|  |  | 20 |  | 835 |  | 5 | 25 |  |  |  | 794 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | N/A |  | 5 | N/A |  |  |  | 2120 | 15.3 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3790 |  | 10 | 50 |  |  |  | 3790 | N/A |  |  |  |  | N/A |  |  |
| DC_20_n3-n67 |  | 20 |  | 837 |  | 5 | 25 |  |  |  | 796 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1765 |  | 5 | 25 |  |  |  | 1860 | N/A |  |  |  |  | N/A |  |  |
|  |  | n67 |  | N/A |  | 5 | N/A |  |  |  | 746 | 9.4 |  |  |  |  | IMD4 |  |  |
| DC_20A_n3A-n78A |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1730 |  | 5 | 25 |  |  |  | 1825 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3420 | 16.1 |  |  |  |  | IMD3 |  |  |
|  |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1860 | 15.7 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3550 |  | 10 | 50 |  |  |  | 3550 | N/A |  |  |  |  | N/A |  |  |
| DC_20A_n7A-n78A |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | N/A |  | 5 | N/A |  |  |  | 2675 | 30.8 |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | 3520 |  | 10 | 50 |  |  |  | 3520 | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | 850 |  | 5 | 25 |  |  |  | 809 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2550 |  | 10 | 50 |  |  |  | 2675 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3400 | 28.8 |  |  |  |  | IMD21 |  |  |
| DC_20A_n8A-n78A |  | n8 |  | 910 |  | 5 | 25 |  |  |  | 955 | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | 837 |  | 5 | 25 |  |  |  | 796 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3567 | 10.3 |  |  |  |  | IMD4 |  |  |
|  |  | n8 |  | N/A |  | 5 | N/A |  |  |  | 940 | 12.1 |  |  |  |  | IMD4 |  |  |
|  |  | n78 |  | 3481 |  | 10 | 50 |  |  |  | 3481 | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | 847 |  | 5 | 25 |  |  |  | 806 | N/A |  |  |  |  | N/A |  |  |
| DC_20A-38A_n1A |  | n1 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD5 |  |  |
|  |  | 38 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
| DC_20A-38A_n78A DC_20A-38A_n78(2A |  | 20 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD2 |  |  |
|  |  | 38 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
|  |  | 38 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | N/A |  | N/A | N/A |  |  |  | N/A | N/A |  |  |  |  | N/A |  |  |
| DC_20A_n38A-n78A |  | 20 |  | 850 |  | 5 | 25 |  |  |  | 809 | N/A |  |  |  |  | N/A |  |  |
|  |  | n38 |  | N/A |  | 10 | N/A |  |  |  | 2600 | 30.9 |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | 3450 |  | 10 | 50 |  |  |  | 3450 | N/A |  |  |  |  | N/A |  |  |
| DC_20A_n7A-n28A |  | 20 |  | 857 |  | 5 | 25 |  |  |  | 816 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2512 |  | 5 | 25 |  |  |  | 2632 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 798 | 13.9 |  |  |  |  | IMD3 |  |  |
|  |  | 20 |  | 852 |  | 5 | 25 |  |  |  | 811 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | N/A |  | 10 | N/A |  |  |  | 2670 | 5.9 |  |  |  |  | IMD5 |  |  |
|  |  | n28 |  | 738 |  | 5 | 25 |  |  |  | 793 | N/A |  |  |  |  | N/A |  |  |
| DC_20A_SUL_n78A-n80A |  | 20 |  | N/A |  | 5 | N/A |  |  |  | 806 | 9 |  |  |  |  | IMD4 |  |  |
|  |  | n80 |  | 1735 |  | 5 | 25 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |
| DC_20A_n41A-n78A |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | N/A |  | 10 | N/A |  |  |  | 2675 | 29.8 |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | 3520 |  | 10 | 50 |  |  |  | 3520 | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | 850 |  | 5 | 25 |  |  |  | 809 | N/A |  |  |  |  | N/A |  |  |
|  |  | n41 |  | 2550 |  | 10 | 50 |  |  |  | 2550 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3400 | 28.8 |  |  |  |  | IMD2 |  |  |
| DC_21A_n1A-n77A DC_21A_n1A-n78A |  | 21 |  | 1450.4 |  | 5 | 25 |  |  |  | 1498.4 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | N/A |  | 5 | N/A |  |  |  | 2154.6 | 30.6 |  |  |  |  | IMD24 |  |  |
|  |  | n77/n78 |  | 3605 |  | 10 | 50 |  |  |  | 3605 | N/A |  |  |  |  | N/A |  |  |
| DC_21A-28A_n77A |  | 21 |  | 1452 |  | 5 | 25 |  |  |  | 1500 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 785.5 | 16.9 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3689.5 |  | 10 | 50 |  |  |  | 3689.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 21 |  | N/A |  | 5 | N/A |  |  |  | 1498.5 | 9.9 |  |  |  |  | IMD4 |  |  |
|  |  | 28 |  | 730.5 |  | 5 | 25 |  |  |  | 785.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3690 |  | 10 | 50 |  |  |  | 3690 | N/A |  |  |  |  | N/A |  |  |
| DC_21A-28A_n79A |  | 21 |  | N/A |  | 5 | N/A |  |  |  | 1498 | 5.2 |  |  |  |  | IMD5 |  |  |
|  |  | 28 |  | 730.5 |  | 5 | 25 |  |  |  | 785.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4420 |  | 40 | 216 |  |  |  | 4420 | N/A |  |  |  |  | N/A |  |  |
| DC_21A_n28A-n77A |  | 21 |  | 1452 |  | 5 | 25 |  |  |  | 1500 | N/A |  |  |  |  | N/A |  |  |
| DC_21A_n28A-n78A |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 785.5 | 16.9 |  |  |  |  | IMD39 |  |  |
|  |  | n77/n78 |  | 3689.5 |  | 10 | 50 |  |  |  | 3689.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 21 |  | 1452 |  | 5 | 25 |  |  |  | 1500 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 730.5 |  | 5 | 25 |  |  |  | 785.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77/n78 |  | N/A |  | 10 | N/A |  |  |  | 3634.5 | 17.3 |  |  |  |  | IMD39 |  |  |
| DC_21A_n28A-n79A 15 |  | 21 |  | 1450.4 |  | 5 | 25 |  |  |  | 1498.4 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 790.5 | 2.8 |  |  |  |  | IMD5 |  |  |
|  |  | n79 |  | 4980 |  | 40 | 216 |  |  |  | 4980 | N/A |  |  |  |  | N/A |  |  |
|  |  | 21 |  | 1460.4 |  | 5 | 25 |  |  |  | 1508.4 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | 735.5 |  | 5 | 25 |  |  |  | 790.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4420 | [6.3] |  |  |  |  | IMD44 |  |  |
| DC_21A-42A_n1A |  | 21 |  | N/A |  | 5 | N/A |  |  |  | 1500 | 31.4 |  |  |  |  | IMD2 |  |  |
|  |  | 42 |  | 3450 |  | 10 | 50 |  |  |  | 3450 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 | N/A |  |  |  |  | N/A |  |  |
| DC_28A_n1A-n40A |  | 28 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1930 |  | 5 | 25 |  |  |  | 2120 | N/A |  |  |  |  | N/A |  |  |
|  |  | n40 |  | N/A |  | 5 | N/A |  |  |  | 2374 | 10.1 |  |  |  |  | IMD4 |  |  |
| DC_28A_n1A-n78A |  | 28 |  | 733 |  | 5 | 25 |  |  |  | 788 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1950 |  | 5 | 25 |  |  |  | 2140 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3416 | 15.7 |  |  |  |  | IMD3 |  |  |
|  |  | 28 |  | 740 |  | 5 | 25 |  |  |  | 795 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | N/A |  | 5 | N/A |  |  |  | 2150 | 15.7 |  |  |  |  | IMD3 |  |  |
|  |  | n78 |  | 3630 |  | 10 | 50 |  |  |  | 3630 | N/A |  |  |  |  | N/A |  |  |
| DC_28A_n3A-n77A |  | 28 |  | 735 |  | 5 | 25 |  |  |  | 790 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | N/A |  | 5 | N/A |  |  |  | 1850 | 17.0 |  |  |  |  | IMD3 |  |  |
|  |  | n77 |  | 3320 |  | 10 | 50 |  |  |  | 3320 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | 733 |  | 5 | 25 |  |  |  | 788 | N/A |  |  |  |  | N/A |  |  |
|  |  | n3 |  | 1720 |  | 5 | 25 |  |  |  | 1815 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | N/A |  | 10 | N/A |  |  |  | 4173 | 15.9 |  |  |  |  | IMD3 |  |  |
| DC_28A_n7A-n78A DC_28A_n7B-n78A |  | 28 |  | 745 |  | 5 | 25 |  |  |  | 800 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2565 |  | 5 | 25 |  |  |  | 2685 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3310 | 29.7 |  |  |  |  | IMD2 |  |  |
|  |  | 28 |  | 740 |  | 5 | 25 |  |  |  | 795 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | N/A |  | 5 | N/A |  |  |  | 2650 | 30.5 |  |  |  |  | IMD2 |  |  |
|  |  | n78 |  | 3390 |  | 10 | 50 |  |  |  | 3390 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-38A_n1A |  | n1 |  | 1975 |  | 5 | 25 |  |  |  | 2165 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 775 | 4.5 |  |  |  |  | IMD5 |  |  |
|  |  | 38 |  | 2575 |  | 5 | 25 |  |  |  | 2575 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-41A_n77A |  | 28 |  | 738 |  | 5 | 25 |  |  |  | 793 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3380 |  | 10 | 50 |  |  |  | 3380 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2642 | 29.5 |  |  |  |  | IMD2 |  |  |
| DC_28A-41A_n77A |  | 41 |  | 2642 |  | 5 | 25 |  |  |  | 2642 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3440 |  | 10 | 50 |  |  |  | 3440 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 798 | 30.8 |  |  |  |  | IMD2 |  |  |
| DC_28A-41A_n77A |  | 41 |  | 2567.5 |  | 10 | 50 |  |  |  | 2567.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n77 |  | 3460 |  | 10 | 50 |  |  |  | 3460 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 782.5 | 3.0 |  |  |  |  | IMD5 |  |  |
| DC_28A-41A_n78A |  | 28 |  | 738 |  | 5 | 25 |  |  |  | 793 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3380 |  | 10 | 50 |  |  |  | 3380 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2642 | 29.5 |  |  |  |  | IMD2 |  |  |
| DC_28A-41A_n78A |  | 41 |  | 2642 |  | 5 | 25 |  |  |  | 2642 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3440 |  | 10 | 50 |  |  |  | 3440 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 798 | 30.8 |  |  |  |  | IMD2 |  |  |
| DC_28A-41A_n79A |  | 28 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4739 |  | 40 | 216 |  |  |  | 4739 | N/A |  |  |  |  | N/A |  |  |
|  |  | 41 |  | N/A |  | 5 | N/A |  |  |  | 2510 | 8.6 |  |  |  |  | IMD4 |  |  |
| DC_28A-41A_n79A |  | 41 |  | 2650 |  | 5 | 25 |  |  |  | 2650 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4502 |  | 40 | 216 |  |  |  | 4502 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 798 | 15.9 |  |  |  |  | IMD3 |  |  |
| DC_28A-42A_n79A |  | 28 |  | 730 |  | 5 | 25 |  |  |  | 785 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-42A_n79C |  | 42 |  | N/A |  | 5 | N/A |  |  |  | 3420 | 15.3 |  |  |  |  | IMD3 |  |  |
| DC_28A-42C_n79A |  | n79 |  | 4880 |  | 40 | 216 |  |  |  | 4880 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-42C_n79C |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 800 | 16.2 |  |  |  |  | IMD2 |  |  |
|  |  | 42 |  | 3597.5 |  | 5 | 25 |  |  |  | 3597.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4420 |  | 40 | 216 |  |  |  | 4420 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-66A_n7A |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 790 | 27.6 |  |  |  |  | IMD2 |  |  |
|  |  | 66 |  | 1715 |  | 5 | 25 |  |  |  | 2115 | N/A |  |  |  |  | N/A |  |  |
|  |  | n7 |  | 2505 |  | 5 | 50 |  |  |  | 2625 | N/A |  |  |  |  | N/A |  |  |
| DC_28A-66A_n66A |  | 28 |  | 710.5 |  | 5 | 25 |  |  |  | 765.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 66 |  | N/A |  | 5 | N/A |  |  |  | 2129 | 11.0 |  |  |  |  | IMD4 |  |  |
|  |  | n66 |  | 1775 |  | 5 | 25 |  |  |  | 2175 | N/A |  |  |  |  | N/A |  |  |
| DC_19A_n78A-n79A |  | 19 |  | 835 |  | 5 | 25 |  |  |  | 880 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3680 |  | 10 | 50 |  |  |  | 3680 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | N/A |  | 40 | N/A |  |  |  | 4515 | 29.3 |  |  |  |  | IMD2 |  |  |
|  |  | 19 |  | 835 |  | 5 | 25 |  |  |  | 880 | N/A |  |  |  |  | N/A |  |  |
|  |  | n79 |  | 4550 |  | 40 | 216 |  |  |  | 4550 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3715 | 28.8 |  |  |  |  | IMD2 |  |  |
| DC_20A-28A_n3A |  | 20 |  | 845 |  | 5 | 25 |  |  |  | 804 | N/A |  |  |  |  | N/A |  |  |
|  |  | 28 |  | N/A |  | 5 | N/A |  |  |  | 785 | 9.4 |  |  |  |  | IMD4 |  |  |
|  |  | n3 |  | 1750 |  | 5 | 25 |  |  |  | 1845 |  | N/A |  |  | N/A |  |  |  |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A |  | 20 |  | 857 |  | 5 | 25 |  |  |  | 816 |  | N/A |  |  | N/A |  |  |  |
|  |  | n28, n83 |  | 743 |  | 5 | 25 |  |  |  | 798 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | N/A |  | 10 | N/A |  |  |  | 3314 | 8.7 |  |  |  |  | IMD4 |  |  |
|  |  | 20 |  | 837 |  | 5 | 25 |  |  |  | 796 | N/A |  |  |  |  | N/A |  |  |
|  |  | n78 |  | 3310 |  | 10 | 50 |  |  |  | 3310 | N/A |  |  |  |  | N/A |  |  |
|  |  | n28 |  | N/A |  | 5 | N/A |  |  |  | 799 | 9.4 |  |  |  |  | IMD4 |  |  |
| DC_20A-32A_n1A |  | n1 |  | 1950.5 |  | 5 | 50 |  |  |  | 2140.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 20 |  | 852.5 |  | 5 | 25 |  |  |  | 811.5 | N/A |  |  |  |  | N/A |  |  |
|  |  | 32 |  | N/A |  | 5 | N/A |  |  |  | 1459.5 | 4.0 |  |  |  |  | IMD5 |  |  |
| DC_20A-38A_n3A |  | 20 |  | 850 |  | 5 | 25 |  |  |  | 809 | N/A |  |  |  |  | N/A |  |  |
|  |  | 38 |  | N/A |  | 5 | N/A |  |  |  | 2610 | 28.4 |  |  |  |  | IMD21 |  |  |
|  |  | n3 |  | 1760 |  | 5 | 25 |  |  |  | 1855 | N/A |  |  |  |  | N/A |  |  |
| DC_20A-40A_n1A DC_20A-40C_n1A |  | 20 |  | N/A |  | 5 | N/A |  |  |  | 800 | 8.0 |  |  |  |  | IMD4 |  |  |
|  |  | 40 |  | 2330 |  | 5 | 25 |  |  |  | 2330 | N/A |  |  |  |  | N/A |  |  |
|  |  | n1 |  | 1930 |  | 5 | 25 |  |  |  | 2120 |  | N/A |  |  | N/A |  |  |  |
| DC_20A-40A_n78A DC_20A-40C_n78A DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | 20 |  | N/A |  | 5 |  |  | N/A | 815 |  |  |  |  | 19.8 |  |  |  | IMD3 |  |
|  | 40 |  | 2302.5 |  | 5 |  |  | 25 | 2302.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3790 |  | 10 |  |  | 50 | 3790 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_21A_n78A-n79A | 21 |  | 1453 |  | 5 |  |  | 25 | 1501 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3420 |  | 10 |  |  | 50 | 3420 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4873 |  |  |  |  | 30.1 |  |  |  | IMD2 |  |
|  | 21 |  | 1453 |  | 5 |  |  | 25 | 1501 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | 4940 |  | 40 |  |  | 216 | 4940 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3487 |  |  |  |  | 29.8 |  |  |  | IMD2 |  |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | 25 |  | N/A |  | 5 |  |  | N/A | 1992.5 |  |  |  |  | 8.5 |  |  |  | IMD7 |  |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | 41 |  | 2502.5 |  | 5 |  |  | 1 (RBstart=0) | 2502.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | n41 |  | 2670 |  | 5 |  |  | 1 (RBstart=9) | 2670 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 |  | 1855 |  | 5 |  |  | 25 | 1935 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2115 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | n77 |  | 3970 |  | 10 |  |  | 25 | 3970 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | M/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 10.4 |  |  |  | IMD4 |  |
|  | n77 |  | 3500 |  | 10 |  |  | 25 | 3500 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | 1885 |  | 5 |  |  | 25 | 1965 |  |  |  |  | M/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2175 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
|  | n77 |  | 3915 |  | 10 |  |  | 25 | 3915 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
|  | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3720 |  | 10 |  |  | 25 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 9.1 |  |  |  | IMD411 |  |
|  | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3385 |  | 10 |  |  | 25 | 3385 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
|  | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3540 |  | 10 |  |  | 25 | 3540 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | M/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2160 |  |  |  |  | 10.4 |  |  |  | IMD4 |  |
|  | n78 |  | 3480 |  | 10 |  |  | 50 | 3480 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
|  | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3700 |  | 10 |  |  | 50 | 3700 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 25 |  | N/A |  | 5 |  |  | N/A | 1980 |  |  |  |  | 4.2 |  |  |  | IMD5 |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3645 |  | 10 |  |  | 25 | 3645 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_28A_n8A-n78A | 28 |  | 728 |  | 5 |  |  | 25 | 783 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | 910 |  | 5 |  |  | 25 | 955 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3458 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | 28 |  | 713 |  | 5 |  |  | 25 | 768 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n8 |  | N/A |  | 5 |  |  | N/A | 935 |  |  |  |  | 4.3 |  |  |  | IMD5 |  |
|  | n78 |  | 3787 |  | 10 |  |  | 50 | 3787 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 |  | N/A |  | 5 |  |  | N/A | 800.5 |  |  |  |  | 11 |  |  |  | IMD3 |  |
|  | 40 |  | 2302.5 |  | 5 |  |  | 25 | 2302.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3795 |  | 10 |  |  | 50 | 3795 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 28 |  | 715 |  | 5 |  |  | 25 | 770 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 40 |  | N/A |  | 5 |  |  | N/A | 2320 |  |  |  |  | 15.7 |  |  |  | IMD3 |  |
|  | n78 |  | 3750 |  | 10 |  |  | 50 | 3750 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_29A-30A_n66A | 29 |  | N/A |  | 5 |  |  | N/A | 719.5 |  |  |  |  | 4.5 |  |  |  | IMD5 |  |
|  | 30 |  | 2307.5 |  | 5 |  |  | 25 | 2352.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | 1777.5 |  | 5 |  |  | 25 | 2177.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_29A-30A_n77A | 29 |  | N/A |  | 5 |  |  | N/A | 722 |  |  |  |  | 15.2 |  |  |  | IMD34 |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3898 |  | 10 |  |  | 50 | 3898 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_29A-66A_n77A | 29 |  | N/A |  | 5 |  |  | N/A | 722 |  |  |  |  | 15.2 |  |  |  | IMD311 |  |
| DC_29A-66A-66A_n77A | 66 |  | 1734 |  | 5 |  |  | 25 | 2134 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4190 |  | 10 |  |  | 50 | 4190 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2130 |  |  |  |  | 2.5 |  |  |  | IMD5 |  |
|  | n5 |  | 830 |  | 5 |  |  | 25 | 875 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_30A-66A_n77A DC_30A-66A_n77(2A) | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | 29.2 |  |  |  | IMD211 |  |
| DC_30A-66A-66A_n77A | 66 |  | 1745 |  | 5 |  |  | 25 | 2145 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4100 |  | 10 |  |  | 50 | 4100 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  | N/A |  | 5 |  |  | N/A | 2355 |  |  |  |  | 3.4 |  |  |  | IMD5 |  |
|  | 66 |  | 1735 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3780 |  | 10 |  |  | 50 | 3780 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 30 |  | 2310 |  | 5 |  |  | 25 | 2355 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2160 |  |  |  |  | 8.7 |  |  |  | IMD411 |  |
|  | n77 |  | 3390 |  | 10 |  |  | 50 | 3390 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_39A_n40A-n79A | 39 |  | 1917.5 |  | 5 |  |  | 25 | 1917.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n40 |  | 2302.5 |  | 5 |  |  | 25 | 2302.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4980 |  |  |  |  | 5.8 |  |  |  | IMD4 |  |
| DC_39A_n41A-n79A | 39 |  | 1900 |  | 5 |  |  | 25 | 1900 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2620 |  | 10 |  |  | 50 | 2620 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n79 |  | N/A |  | 40 |  |  | N/A | 4520 |  |  |  |  | 29.8 |  |  |  | IMD24 |  |
|  | 39 |  | 1900 |  | 5 |  |  | 25 | 1900 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 10 |  |  | N/A | 2620 |  |  |  |  | 30.2 |  |  |  | IMD24 |  |
|  | n79 |  | 4520 |  | 40 |  |  | 216 | 4520 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_40A_n1A-n78A | 40 |  | 2340 |  | 5 |  |  | 25 | 2340 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | 1930 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3450 |  |  |  |  | 9.8 |  |  |  | IMD4 |  |
|  | 40 |  | 2360 |  | 5 |  |  | 25 | 2360 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2140 |  |  |  |  | 9.1 |  |  |  | IMD4 |  |
|  | n78 |  | 3430 |  | 10 |  |  | 50 | 3430 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_41A_n1A-n77A | 41 |  | 2650 |  | 5 |  |  | 25 | 2650 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_41C_n1A-n77A | n1 |  | 1970 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3330 |  |  |  |  | 19.6 |  |  |  | IMD39 |  |
|  | 41 |  | 2510 |  | 5 |  |  | 25 | 2510 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 4150 |  | 10 |  |  | 50 | 4150 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n1 |  | N/A |  | 5 |  |  | N/A | 2120 |  |  |  |  | 11.0 |  |  |  | IMD4 |  |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 |  | 2620 |  | 5 |  |  | 25 | 2620 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | N/A |  | 5 |  |  | N/A | 1840 |  |  |  |  | 16.4 |  |  |  | IMD3 |  |
|  | n77/n78 |  | 3400 |  | 10 |  |  | 50 | 3400 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 41 |  | 2580 |  | 5 |  |  | 25 | 2580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n3 |  | 1720 |  | 5 |  |  | 25 | 1815 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77/n78 |  | N/A |  | 10 |  |  | N/A | 3440 |  |  |  |  | 16.8 |  |  |  | IMD34 |  |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 |  | 2580 |  | 5 |  |  | 25 | 2580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | 743 |  | 5 |  |  | 25 | 798 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77/n78 |  | N/A |  | 10 |  |  | N/A | 3323 |  |  |  |  | 28.2 |  |  |  | IMD21 |  |
|  | 41 |  | 2642 |  | 5 |  |  | 25 | 2642 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n28 |  | N/A |  | 5 |  |  | N/A | 798 |  |  |  |  | 30.8 |  |  |  | IMD21 |  |
|  | n77/n78 |  | 3440 |  | 10 |  |  | 50 | 3440 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD2, IMD3 |  |
|  | 48 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD2, IMD3 |  |
|  | 48 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_46A-66A_n5A | 46 |  | N/A |  | 10 |  |  | N/A | 5163 |  |  |  |  | 9.0 |  |  |  | IMD4 |  |
| DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 847 |  | 5 |  |  | 25 | 892 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 |  | N/A |  | 10 |  |  | N/A | 5505 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | 46 |  | N/A |  | 10 |  |  | N/A | 5505 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |
|  | n25 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 46 |  | N/A |  | 10 |  |  | N/A | 5505 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2112.5 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | n25 |  | 1912.5 |  | 5 |  |  | 25 | 1992.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_46A-66A_n77A5 DC_46A-46A-66A_n77A5 | 46 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | IMD2, IMD3 |  |
|  | 66 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | N/A |  |  | N/A | N/A |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48A-(n)12AA | 48 |  | 3557.5 |  | 10 |  |  | 50 | 3557.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 12 |  | N/A |  | 5 |  |  | N/A | 740.5 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |
|  | n12 |  | 705.5 |  | 5 |  |  | 25 | 735.5 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | 3560 |  | 5 |  |  | 25 | 3560 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2155 |  |  |  |  | 12.1 |  |  |  | IMD4 |  |
|  | n2 |  | 1905 |  | 5 |  |  | 25 | 1985 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48A-66A_n12A | 48 |  | 3580 |  | 5 |  |  | 25 | 3580 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2160 |  |  |  |  | 17.1 |  |  |  | IMD3 |  |
|  | n12 |  | 710 |  | 5 |  |  | 25 | 740 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 |  | 3630 |  | 20 |  |  | 100 | 3630 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2130 |  |  |  |  | 8.3 |  |  |  | IMD4 |  |
|  | n25 |  | 1883.3 |  | 5 |  |  | 25 | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48A-66A_n66A DC_48C-66A_n66A | 48 |  | 3660 |  | 20 |  |  | 100 | 3660 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48D-66A_n66A | 66 |  | N/A |  | 5 |  |  | N/A | 2175 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
| DC_48E-66A_n66A | n66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_48A-66A_n71A | 48 |  | 3560 |  | 5 |  |  | 25 | 3560 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | N/A |  | 5 |  |  | N/A | 2174 |  |  |  |  | 15.8 |  |  |  | IMD3 |  |
|  | n71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 48 |  | N/A |  | 5 |  |  | N/A | 3697.5 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
|  | 66 |  | 1712.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 665.5 |  | 5 |  |  | 25 | 619.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n2A-n66A | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |
|  | n66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1720 |  | 5 |  |  | 25 | 2120 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1870 |  | 5 |  |  | 25 | 1950 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2170 |  |  |  |  | 4.0 |  |  |  | IMD5 |  |
| DC_66A_n2A-n77A | n2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
|  | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | 3720 |  | 10 |  |  | 50 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66_n2-n78 | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 32.1 |  |  |  | IMD2 |  |
|  | n78 |  | 3720 |  | 10 |  |  | 50 | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 34.9 |  |  |  | IMD2 |  |
|  | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3340 |  |  |  |  | 20.9 |  |  |  | IMD4 |  |
|  | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 21.1 |  |  |  | IMD4 |  |
|  | n78 |  | 3350 |  | 10 |  |  | 50 | 3350 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 2.1 |  |  |  | IMD5 |  |
|  | n78 |  | 3620 |  | 10 |  |  | 50 | 3620 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A-(n)5AA | 66 |  | 1721 |  | 5 |  |  | 25 | 2121 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 5 |  | N/A |  | 5 |  |  | N/A | 878 |  |  |  |  | 25 |  |  |  | IMD2 |  |
|  | n5 |  | 838 |  | 5 |  |  | 25 | 883 |  |  |  |  | 30 |  |  |  | IMD2 |  |
| DC_66A_n5A-n48A | 66 |  | 1750 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 834 |  | 5 |  |  | 25 | 879 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n48 |  | N/A |  | 5 |  |  | N/A | 3582 |  |  |  |  | 3.3 |  |  |  | IMD5 |  |
| DC_66A_n5A-n77A | 66 |  | 1770 |  | 5 |  |  | 25 | 2170 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n5 |  | 845 |  | 5 |  |  | 25 | 890 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n77 |  | N/A |  | 10 |  |  | N/A | 3460 |  |  |  |  | 16.6 |  |  |  | IMD39 |  |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 |  | 1730 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n7 |  | 2560 |  | 5 |  |  | 25 | 2680 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3390 |  |  |  |  | 16.1 |  |  |  | IMD3 |  |
| DC_66A_n25A-n41A | 66 |  | 1715 |  | 5 |  |  | 25 | 2115 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | 2685 |  | 10 |  |  | 50 | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1940 |  |  |  |  | 5 |  |  |  | 11.0 |  |
| DC_66A_n25A-n48A | 66 |  | 1740 |  | 5 |  |  | 25 | 2140 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | 1880 |  | 5 |  |  | 25 | 1960 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n48 |  | N/A |  | 10 |  |  | N/A | 3620 |  |  |  |  | 29.4 |  |  |  | IMD2 |  |
|  | 66 |  | 1735 |  | 5 |  |  | 25 | 2135 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | N/A |  | 5 |  |  | N/A | 1960 |  |  |  |  | 28.3 |  |  |  | IMD2 |  |
|  | n48 |  | 3695 |  | 5 |  |  | 25 | 3695 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n25A-n66A | 66 |  | 1712.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | 1912.5 |  | 5 |  |  | 25 | 1992.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2117.5 |  |  |  |  | 23 |  |  |  | IMD3 |  |
|  | 66 |  | 1750 |  | 5 |  |  | 25 | 2150 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n25 |  | 1873 |  | 5 |  |  | 25 | 1953 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2119 |  |  |  |  | 4 |  |  |  | IMD5 |  |
| DC_66A_n38A-n78A | 66 |  | 1760 |  | 5 |  |  | 25 | 2160 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | 2610 |  | 10 |  |  | 50 | 2610 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3460 |  |  |  |  | 15.0 |  |  |  | IMD3 |  |
| DC_66A_n66A-n71A | 66 |  | 1752 |  | 5 |  |  | 25 | 2152 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2118 |  |  |  |  | 5.0 |  |  |  | IMD4 |  |
|  | n71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n66A-n77A | 66 |  | 1730 |  | 5 |  |  | 25 | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2170 |  |  |  |  | 31 |  |  |  | IMD2 |  |
|  | n77 |  | 3900 |  | 10 |  |  | 50 | 3900 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n66A-n78A | 66 |  | 1775 |  | 5 |  |  | 25 | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2125 |  |  |  |  | 2.8 |  |  |  | IMD5 |  |
|  | n78 |  | 3725 |  | 10 |  |  | 50 | 3725 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_66A_n71A-n78A | 66 |  | 1712.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n71 |  | 665.5 |  | 5 |  |  | 25 | 619.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3709 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
| DC_71A_n2A-n41A | n2 |  | 1900 |  | 5 |  |  | 25 | 1980 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n41 |  | N/A |  | 5 |  |  | N/A | 2586 |  |  |  |  | 29.2 |  |  |  | IMD2 |  |
|  | 71 |  | 686 |  | 5 |  |  | 50 | 640 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1942 |  |  |  |  | 26 |  |  |  | IMD2 |  |
|  | n41 |  | 2610 |  | 5 |  |  | 25 | 2610 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  | 668 |  | 5 |  |  | 25 | 622 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_71A_n2A-n78A | n2 |  | 1907.5 |  | 5 |  |  | 25 | 1987.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | 71 |  | 695.5 |  | 5 |  |  | 25 | 649.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3305 |  |  |  |  | 8.0 |  |  |  | IMD3 |  |
|  | n2 |  | N/A |  | 5 |  |  | N/A | 1954 |  |  |  |  | 16.5 |  |  |  | IMD3 |  |
|  | 71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3340 |  | 10 |  |  | 50 | 3340 |  |  |  |  | N/A |  |  |  | N/A |  |
| DC_71A_n38A-n78A | 71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | 2615 |  | 10 |  |  | 50 | 2615 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3308 |  |  |  |  | 29.1 |  |  |  | IMD2 |  |
|  | 71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3308 |  | 10 |  |  | 50 | 3308 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n38 |  | N/A |  | 10 |  |  | N/A | 2615 |  |  |  |  | 28.7 |  |  |  | IMD2 |  |
| DC_71A_n66A-n78A | 71 |  | 693 |  | 5 |  |  | 25 | 647 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | 3546 |  | 10 |  |  | 50 | 3546 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n66 |  | N/A |  | 5 |  |  | N/A | 2160 |  |  |  |  | 15.5 |  |  |  | IMD3 |  |
|  | 71 |  | 665.5 |  | 5 |  |  | 25 | 619.5 |  |  |  |  | N/A |  |  |  | N/A |  |
|  | n78 |  | N/A |  | 10 |  |  | N/A | 3697.5 |  |  |  |  | 13.0 |  |  |  | IMD4 |  |
|  | n66 |  | 1712.5 |  | 5 |  |  | 25 | 2112.5 |  |  |  |  | N/A |  |  |  | N/A |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.NOTE 12: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 13: Void NOTE 14: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 15: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 16: This band is subject to 1st order triple-beat IMD3 where MSD is not specified when the UL configuration includes intra-band uplink CCs. |

### << End of change >>