3GPP TSG-RAN4 Meeting #109	 R4-2320544

# Chicago, USA, November 13 –17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | TBD | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for spectrum flatness for NR coverage enhancement Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The transparent scheme to improve the uplink coverage is introduced in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new RF requriement on spectrum flatness, ACLR when UE support power boosting with transparent scheme. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uplink coverage enhancement is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

##### 6.4.2.4.2	Requirements for Rel-18 power boosting for pi/2 BPSK and QPSK

These requirements apply if the IE [powerBoostPi2BPSKQPSK-REL-18] is set to 1 for power class 2 capable UE and UE indicates support for UE capability [powerBoostingPiBPSKQPSK-Rel-18] and [80 %] or less slots in certain evaluation period in radio frame are used for UL transmission.

The EVM equalizer coefficients across the allocated uplink block shall be modified to fit inside the mask specified in Table 6.4.2.4.2-1 for normal conditions, prior to the calculation of EVM. The limiting mask shall be placed to minimize the change in equalizer coefficients in a sum of squares sense.

Table 6.4.2.4.2-1: Mask for EVM equalizer coefficients for QPSK, normal conditions

| Frequency range | Parameter | Maximum ripple (dB) |
| --- | --- | --- |
| ¦FUL_Meas – Fcenter¦ ≤ X MHz (Range 1) | X1 | [6] (p-p) |
| ¦FUL_Meas – Fcenter¦ > X MHz (Range 2) | X2 | [8] (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: Fcenter refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to [40%] of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.4.1-1 for description of X1, X2 NOTE 5: Requirement applies for inner RB allocation. |  |  |

Figure 6.4.2.4.2-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation

------------------------------------------------------------- END CHANGE ------------------------------------------------------