# 3GPP TSG-RAN WG4 Meeting #109	R4-2320554

# Chicago, USA, 13 – 17 November, 2023

# Source:	ZTE Corporation

# Title:	Discussion on general issues

# Agenda item:	8.21.1

# Document for:	Approval

- Introduction

In 3GPP RAN#94e meeting, the latest SID has been approved. According to the SID, the study will focus on the general issues, evaluations for three different use cases and other aspects relate to specification impacts.

Three use cases which confirmed by RAN1:

- Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

The RAN4 scope in this SID:

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we will discuss the RAN4 related specific issues

- Discussion

2.1 Requirements for data collection

For the discussion of data collection, the first thing to be clear is that data collection for training does not study the online situation, as the priority of online training has been lowered in the RAN4 discussion, and at the current stage, online training is not discussed. And for the offline training data, it is up to the implementation and it shall not have any requirements.

In the last meeting, RAN2 has already proposed the framework of AI/ML as below:

Figure 1 AI/ML framework

There are several differences among three kinds of data which will cause the different requirements:

For training data, the training data is the input of model training function and used based on the TS38.901, e.g. the Dense Urban dataset which is applied to the UMa scenario which is divided into three parts for model training, model validation and model testing the three sub-processes which belong to the model training function.

For inference data, the inference data is the input for model inference function and a type of  “unknown” real-time data, the only one principle for inference data is that it shall be different from the training data.

For monitoring data,  it is the input for management function which is kind of logical node for the whole framework. The monitoring data is type of the “ideal data” which is used for judging the model performance with the inference output in the above figure.

Based on RAN1’s assumption, there are two ways for data collection: one is over the 3GPP air interface, such as UE reports the real channel and CSI compression to gNB over the air interface and another one is over none 3GPP air interface, such as UE collects the real channel and transmits them to the OTT server on the UE side. The former one shall be discussed per use case and the latter one is out of 3GPP scope.

From RAN2 perspective, it is agreed that:

- For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection.

Based on RAN2’s agreements we deem that :

For model training, RAN1/2/4 have already deprioritized the online training, so the training is only the offline training in R18 and there is no latency requirements for data collection.

For model inference, when required data comes from other entities, there is latency requirement for data collection (the NW-sided model).

For model monitoring, the same situation as the model inference data which the latency requirements shall be studied for data collection.

Observation 1: RAN2 assumes that there is no latency requirements for data collection for all types of offline training. However, the latency requirements shall be studied for data collection for inference data and monitoring data.

From RAN4 perspective, we shall analyze the concrete latency requirements for data collection and I just illustrate it as below (model inference as an example):

For CSI compression:

Figure 2  the process of data collection from different entities

Figure 3 latency requirements (CSI compression)

Based on the RAN2’s agreement, for one-sided model the latency requirements mainly for NW-sided model since the gNB sends the RS to UE and UE measures the RS and then UE reports the result to gNB which an AI/ML Model whose inference is performed entirely at the network as figure 2 shown.

Since the inference data is type of a real-time data, so it is sensitive to the timing and latency requirements and the samples shall be processed one by one over the air interface. Based on what I just mentioned before, the total considering latency for one sample shall be as shown in figure 3:

where,

t1 denotes the propagation delay from gNB to send the related RS to UE;

t2 denotes the measurement time for RS;

- t2-1: the processing time for raw input data

- t2-2: the measurement time for processed input data

t3 denotes the uncertainty time;

t4 denotes the propagation delay from UE to report the measurement result to gNB;

t5 denotes the processing time for results;

N denotes the number of  selected samples.

In theory, t1=t4 the propagation delay shall be the same which equals the distance (d) between gNB and UE divides the speed of light (c):

t2 and t3 shall also be considered in RAN4.

Observation 2: For model inference, the MAX total latency values will be .

Proposal 1: RAN4 shall define the latency requirements based on RAN2’s agreements and the MAX total latency requirements can be:

where,

t1 denotes the propagation delay from gNB to send the related RS to UE;

t2 denotes the measurement time for RS;

- t2-1: the processing time for raw input data

- t2-2: the measurement time for processed input data

t3 denotes the uncertainty time;

t4 denotes the propagation delay from UE to report the measurement result to gNB;

t5 denotes the processing time for results;

N denotes the number of  selected samples.

In theory, t1=t4 the propagation delay shall be the same which equals the distance (d) between gNB and UE divides the speed of light (c):

Proposal 2: The latency requirements shall be defined per use case.

It should be clarified that data collection is decoupled from training procedure, which is part of LCM. It should be noted that not all data collection are related to model training, also for model monitoring and model inference based on RAN1’s assumption. The focus of our research should be on data quality. For example, when UE collects training data sent by NW, or when NW collect training data sent by UE via air interface, this process is visible to the air interface, and the quality of data transmission is also affected by the protocol. For direct AI/ML positioning, the necessity of dataset quality shall be considered since in the current stage the high precision positioning is needed, the unknown error will be generated if the dataset quality can not be guaranteed. So the requirements for data collection shall be focused on the quality of the data. From our perspective, the ground truth information shall be considered to be mapped to the model input accurately. For example the AI/ML-based positioning, the ground truth information is provided by the PRU which is positioning reference unit that we know its position, if the gNB sends PRS to UE and UE has the terrible measurements since the SNR is bad for channel and there is no NLOS path. This type of data is “bad data” and then gNB sends the PRS to PRU in order to provide the ground truth which can be compared to the “bad data” (measurement results) and drop it  (the similarity). This progress is kind of screening process.

However, there is a new concern for us how to trade off the data quality and the model generalization. For example, there are two UEs (UE 1 at the center of serving cell and UE 2 at the edge of the serving cell), we use the ground truth to select the “good data” (UE 1 since the SNR is good) and drop the “bad data” (UE 2 since the SNR is bad), there is no doubt that the model training can have a better performance. But if model is sent to the field which the radio conditions and other obstacles (NLOS) are totally different from and complicated than the test environment, the model performance will be degraded and also the model generalization is not beneficial to RAN4 testing goals.

So based on above RAN4 shall consider not only the data quality but the model generalization.

Observation 3: The ground truth shall be mapped to the input and compared to measurement results to guarantee the data quality.

Observation 4: If model is sent to the field which the radio conditions and other obstacles ( e.g. NLOS) are totally different and complicated than the test environment, the model performance will be degradation and also the model generalization is not beneficial to RAN4 testing goals.

2.2 Consideration of  Generalization

Generalization

Generalization refers to the model's ability to adapt properly to new, previously unseen data, drawn from the same distribution as the one used to create the model. In other words, generalization examines how well a model can digest new data (mostly corresponding to new environment/scenario) and make correct predictions (for unseen/new environment or scenario) after getting trained on a training set.

In the last meeting, we have several issues to be discussed as below:

Issue 1-2: Generalization goals Agreement: - Verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations - FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - FFS on what the minimum level performance for each identified scenario and/or configuration is - FFS on what the significant degradation for other scenarios and/or configurations is

For RAN4, we main define the worst requirement, and discussions suggest that the same applies to the validation of generalization. Firstly, we know that the real environment is quite complex. After defining a model in the testing environment, we usually deploy it in the real environment since the radio conditions will definitely change, and more noise or interference factors will greatly reduce the performance of the model, resulting in poor generalization of the model. Therefore, in the testing environment, whether we need to identify specific scenarios and/or configurations to verify whether the performance of the model will increase or maintain, or define a minimum level of performance, is the main key to our research.

From my perspective, for the scenario of UE capability report, which is AI authentication, it can be identified. Otherwise, the remaining scenarios can be regarded as the other scenarios. E.g. we would like to verify the AI model performance under TDL-C which is the identified scenario (supported by the UE capability) and the TDL-A is the other scenario. However, this doesn’t mean that the performance under TDL-A which is not supported by the UE capability will have the significantly degradation, on the contrary, we shall also confirm that is the performance the same under TDL A.

Proposal 3: The scenario of UE capability report, which is AI authentication, it can be identified. Otherwise, the remaining scenarios can be regarded as the other scenarios for test.

Another issue shall be considered, what the minimum level of performance for each identified scenario is. I deem that when we study the minimum level of performance, it shall be studied per use case, that is, different cases have different KPIs to verify the performance of the related AI models even if under the same identified scenarios. How can we define the minimum level of performance? As is well known, the purpose of studying AI-enabled method is to improve the performance other than the complexity and overhead than legacy method. So the legacy performance can be as the baseline when we defining the minimum performance of  AI-based method for each identified scenario.

Proposal 4: The minimum level of performance shall be studied per use case.

Proposal 5: The legacy performance under different use cases can be as the baseline to judge the minimum performance of  AI-based method for each identified scenario.

Above the agreements, we shall also consider the performance under other scenarios. We just mentioned that, RAN4 can verify the performance or minimum level of  performance for identified scenarios, but the significant degradation shall not be occurred. We have not discussed what is the boundary especially the lower bound for judging the performance is degraded or not so far. What the significant degradation for other scenarios and/or configurations is. First of all, I deem that the margin will be needed for judging the performance has the significant degradation or not. We just verify the performance gain of the AI model under the identified scenarios to confirm the generalization of the AI model. The margin shall be added based on the known performance gain for identified scenario and then we have the rules to judge and guarantee the significant degradation will not occur and take the corresponding measures even if it happens.

Proposal 6: The margin shall be added based on the known performance gain for identified scenario to judge what is the significant degradation for other scenarios and try to guarantee that it will not occur.

- Conclusions

In this contribution, we got following proposals

Observation 1: RAN2 assumes that there is no latency requirements for data collection for all types of offline training. However, the latency requirements shall be studied for data collection for inference data and monitoring data.

Observation 2: For model inference, the MAX total latency values will be .

Proposal 1: RAN4 shall define the latency requirements based on RAN2’s agreements and the MAX total latency requirements can be:

where,

t1 denotes the propagation delay from gNB to send the related RS to UE;

t2 denotes the measurement time for RS;

- t2-1: the processing time for raw input data

- t2-2: the measurement time for input data

t3 denotes the uncertainty time;

t4 denotes the propagation delay from UE to report the measurement result to gNB;

t5 denotes the processing time for results ;

N denotes the number of  selected samples.

In theory, t1=t4 the propagation delay shall be the same which equals the distance (d) between gNB and UE divides the speed of light (c):

Proposal 2: The latency requirements shall be defined per use case.

Observation 3: The ground truth shall be mapped to the input and compared to measurement results to guarantee the data quality.

Observation 4: If model is sent to the field which the radio conditions and other obstacles ( e.g. NLOS) are totally different and complicated than the test environment, the model performance will be degradation and also the model generalization is not beneficial to RAN4 testing goals.

Proposal 3: The scenario of UE capability report, which is AI authentication, it can be identified. Otherwise, the remaining scenarios can be regarded as the other scenarios for test.

Proposal 4: The minimum level of performance shall be studied per use case.

Proposal 5: The legacy performance under different use cases can be as the baseline to judge the minimum performance of  AI-based method for each identified scenario.

Proposal 6: The margin shall be added based on the known performance gain for identified scenario to judge what is the significant degradation for other scenarios and try to guarantee that it will not occur.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, Dec. 2021

- R4-2317631, WF on RAN4 requirements for NR AI/ML, Qualcomm.