# 3GPP TSG-RAN WG4 Meeting #109	R4-2320555

# Chicago, USA, 13 – 17 November, 2023

Source:	ZTE Corporation

Title:	Discussion on the Interoperability and testability aspects of AI/ML

Agenda Item:	8.21.3

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some initial views on the interoperability and testability of AI/ML from the general test framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - Following use cases and sub-use cases will be handled in RAN4: - CSI feedback enhancement - time domain CSI prediction - spatial-frequency domain CSI compression - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction - Positioning accuracy enhancements - direct AI/ML positioning - AI/ML assisted positioning

In addition, the framework of AI/ML is illustrated in Fig 1 during RAN2 discussion.

Fig 1: AI/ML framework

To facilitate discussion, RAN2 has agreed the following terminologies :

- As seen in Figure 4.4-1, the general framework consists of:

- -Data Collection is a function that provides input data to the Model Training, Management, and Inference functions.

- oTraining Data: Data needed as input for the AI/ML Model Training function.

- oMonitoring Data: Data needed as input for the Management of AI/ML Models or AI/ML 	 functionalities.

- oInference Data: Data needed as input for the AI/ML Inference function.

- -The Model Training function performs the AI/ML model training, validation, and testing which may generate model performance metrics which can be used as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

- oTrained/Updated Model: In case of having a Model Storage function, this is used to deliver trained, 	validated, and tested AI/ML models to the Model Storage function, or to deliver an updated version 	of a model to the Model Storage function.

- -Management is a function that oversees the operation (e.g., selection/(de)activation/switching/fallback) and monitoring of AI/ML models or AI/ML functionalities. This function is also responsible for making decisions to ensure the proper inference operation based on data received from the Data Collection function and the Inference function.

- oSelection/(de)activation/switching/fallback: Information needed as input to manage the Inference 		function. Concerning information may include selection/(de)activation/switching of AI/ML models 		or AI/ML-based functionalities, fallback to non-AI/ML operation (i.e., not relying on inference 	process), etc…

- oModel Transfer/Delivery Request: Used to request model(s) to the Model Storage function.

- oPerformance feedback/ Retraining request: Information needed as input for the Model Training 	function, e.g., for model (re)training or updating purposes.

- -Inference is a function that provides outputs from the process of applying AI/ML models or AI/ML functionalities to new data (i.e., Inference Data). The Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

- oInference Output: Data used by the Management function to monitor the performance of AI/ML 	models or AI/ML functionalities.

- -Model Storage is a function responsible for storing trained/updated models that can be used to perform the inference process.

- -Model Transfer/Delivery: Used to deliver an AI/ML model to the Inference function. From RAN4 perspective, the discussion should be held around the core requirements related to the functionality/model and the performance requirements so as to guarantee the availability of all the above essential components during the LCM.

- 2.1 The interoperability of AI/ML for NR air interface

- From RAN4 perspective, the interoperability of AI/ML for NR air interface is a kind of a new aspect. We would like to analyse the interoperability based on RAN1 progress and outcomes as below:

- RAN1 109e meeting: Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels 1. Level x: No collaboration 2. Level y: Signaling-based collaboration without model transfer 3. Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary - RAN1 110bis-e meeting: Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signaling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. - RAN1 112 meeting: Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary

And in the previous meeting RAN4 also has the suggestions on interoperability aspect as below:

2.3.6 Interoperability aspects RAN4 will further study interoperability aspects and how they are related to RAN4 aspects. The table in R4-2305199 can be taken as reference. Interested companies are invited to bring analysis on different issues such as relationship/need to consider interoperability and different network-UE collaboration levels.

- a、Collaboration level-x

- In RAN1 110bis-e and 109-e meeting we listed above, level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x, so we shall not consider the interoperability for level x.

- Observation 1: Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x based on RAN1 outcomes.

- Proposal 1: RAN4 shall not study the interoperability aspect for level x based on previous meetings in RAN1.

- b、Collaboration level-y

- Based on RAN1 outcomes in 109-e and 112 meeting, for the level y collaboration, it is clarified as the signaling-based collaboration without model transfer, and the signaling-based collaboration exists between the UE and over-the-top which is outside 3GPP network. Compared to level x, level y is signaling-based NW-UE collaboration, we deem that more RAN2 progress is need which the 3GPP signaling for use case to enable or disable the AI/ML model function or feature.

- Observation 2: For the level y collaboration, it is clarified as the signaling-based collaboration without model transfer.

- Proposal 2: RAN4 needs to consider the interoperability for collaboration level y based on more RAN2 progress since it is the signaling-based collaboration.

- c、Collaboration level-z

- Based on RAN1 progress, two separate model format categories were identified, i.e. Proprietary-format models and Open-format models, The descriptions for these two modes are:

| Proprietary-format models | - ML models of vendor-/device-specific proprietary format, from 3GPP perspective - NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective |

- In terms of the above description, it can be assumed that the Proprietary-format models are not mutually recognizable across vendors, the model designs are vendor-specific, and usually it is invisible between UE/network vendors. So from the specification perspective, it is hard to realize inter-operation between different different vendors due to different models would be used for different vendors, which would cause incompatible problem. Even though there is not any specific definition of interoperability yet, it can be determined that due to lack of inter-operation between different vendors, it is hard to standardize based on the unified specification identification. So in our view, we need to study the feasibility and necessity of the Proprietary-format models for RAN4 core part and performance part discussion.

- Observation 3: Two categories of models including Proprietary-format models and Open-format models were proposed by RAN1. For the Proprietary-format models, due to the lack of inter-operation and recognition between vendors, it is hard to standardize based on the unified specification identification.

- While regarding the Open-format models, the models can be mutually recognizable across different vendors, and it can be visible between vendors when the model information is shared. So from the perspective of 3GPP, it is feasible to define the unified interface to guarantee the functionality. Of course from RAN4 perspective, the related core part and performance part should be discussed. However, the unified interaction format between vendors should be studied.

- Observation 4: Regarding to the Open-format models, the interoperability is feasible.

- Based on the above analysis, it seems that RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models Since it may involve other working groups (RAN2, SA) and other proprietary information sharinf between different vendors. And we shall also consider what the structure of the open-format is and between different vendors shall have the common understanding.

Proposal 3: RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models.

Proposal 4: RAN4 shall study the basic structure of the open-format and consider how the common understanding defined between different vendors.

- 2.2 The testability of AI/ML for NR air interface

When defining the core part and performance part requirements, the testability should also be considered. For AI/ML, since it is a module based enhancement which has a bit difference from legacy RRM procedure, and the details within the module would be algorithm implementations, so maybe some new types of test should be considered. We provide some analysis based on the general AI/ML framework below.

RAN1 has achieved some progress referring to the model/functionality identification as follows:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

No matter what the functionality-based LCM procedure or the model-ID-based LCM procedure is, the basic work procedure is still based on the UE capability reporting and NW signaling indication. We believe the differences are how to package the process/method, to package into model or functionality. The latter is more straightforward.  From the perspective of test, both functionality test and performance test should be considered.

Proposal 5: From the perspective of test, both functionality test and performance test should be considered.

- For the functionality test, whether the module can realize the predicted function should be tested. Beside the functionality test, the performance test can verify the performance gain caused by the AI/ML framework compared with the traditional operation.

Model inference

Model inference is the core component of AI/ML. It is the process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. For the test of model inference, the following aspects should be verified:

- The outputs are the results from the AI/ML inference model rather than the traditional solution.

- The accuracy of outputs meet the requirement.

- For the 1st aspect, maybe some new test setups and procedures are needed, which is different from the traditional test case design. For the 2nd aspect, the legacy requirement of accuracy can be referred, depend on the exact use case or scenario to be tested.

- Observation 5: model inference is the core component of AI/ML. Two aspects should be considered to verify: 1) The outputs are the results from the AI/ML inference model rather than the traditional solution; 2) The accuracy of outputs meet the requirement.

Model monitoring

For model monitoring, it is the logical node in the whole framework. In the previous discussion paper we just studied the latency requirements for monitoring data which is highly related to the model monitoring function. And based on the framework the model monitoring has the relationship with the model training, model storage and model inference so it is necessary for RAN4 to analyse the requirements for model monitoring function.

- The latency requirements for monitoring procedure

- We know that the monitoring data is discontinuity monitoring compared to the inference operation. The inference output will be the part of input for model monitoring, and with the monitoring data the monitoring operation will judge the KPI (such as SGCS etc) between the monitoring data and the inference output in order to judge the performance of the model which can impact the post-process such as model retraining or model transfer/delivery request. So RAN4 shall study the latency of “discontinuity monitoring” and also the testability of the KPI since the criterion of the outcome (such as SGCS) will also impact the feedback behaviour which is included in the model monitoring function. The model transfer/delivery request also has the latency consideration, but from my perspective the latency requirements mainly related to the “discontinuity monitoring”.

- The KPI (e.g., positioning accuracy for AI positioning or SGCS for AI CSI) and the threshold to define the model performance

- As we mentioned before, the model monitoring will monitor the model inference operation in order to guarantee the model performance. However, what we are considering is that the standardization of the threshold to distinguish the model performance (such as the SGCS is larger than 0.8 means the better performance). If RAN4 defines the threshold with the tolerant margin to judge the KPI or threshold to the model performance the clear performance per use case can be understand.

- Observation 6: the latency requirements and the KPI and the threshold for judging the model performance shall be considered in RAN4.

Encoder/decoder for 2-sided model - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec.

Figure 2 the encoder and the reference decoder

In the last meeting, we discussed the encoder/decoder for 2-sided model, the pros and cons on each option:

For option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

Pros:

- Mitigate the impact of the model mismatch in tests.

- Allow UE vendor to update the reference decoder in the future.

Cons:

- UE can obviously pass the tests.

- TE has to support all the reference decoders provided by different UE vendors.

- Impact of the model mismatch may happen in the field since network vendor would not use reference decoder.

- It is not clear how to define RAN4 requirement for models provided by different UE vendors.

For option 2: reference decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained

Pros:

- UE may not obviously pass the tests.

Cons:

- The encoder(s) of UE vendor has to match all the reference decoders provided by different network vendors.

- TE has to support all the reference decoders provided by different network vendors.

- It is not clear how UE vendors train their encoders.

- It is hard to align RAN4 requirement for different models provided by different network vendors.

For option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Pros:

- Easy to align the test requirement and easy to implement for TE vendors.

- If the reference decoder(s) is generated from the well-designed datasets, RAN4 test result can ensure the AI/ML model performance in the field.

Cons:

- The encoder passed the test may not work for the decoder in the field since the fully specified reference decoder may be mismatch with the decoder in the field. UE may have to implement an additional encoder only for the RAN4 test.

- Possible lengthy RAN4 discussion to agree on one (or more) fully specified reference decoder.

For option 4: The reference decoder(s) are partially specified and captured in RAN4 spec.

Pros:

- The issues regarding different models having different performance can be improved.

Cons:

- Different performance may be achieved by different reference decoders implemented by TE vendors.

For option 1 and option 2, they are similar to each other, the former one is the UE vendors for CSI compression use case and the latter one is gNB vendors for CSI compression use case. Both of them have boundedness and absoluteness. For option 1, the UE will obviously pass the test since the reference decoder is totally known to the vendor of the encoder, it has no benefit for the generalization, that is, it will perform the bad performance and the model mismatch may happen in the field since network vendor would not use reference decoder in the field, so the option 1 is not proper for us. For option 2, although the UE may not pass the test easily, the encoder(s) of UE vendor has to match all the reference decoders provided by different network vendors, this will cause the mismatch problems which will impact the performance not only in the test but in the field. For option 3 and option 4, the former one has less flexibility and may not perform well in the field, the latter one is lack of details when captured in RAN4 spec. Both of them will not perform well. For option 3, the reference decoder(s) are totally specified and captured in RAN4 spec, the encoder passed the test may not work for the decoder in the field since the fully specified reference decoder may be mismatch with the decoder in the field since it loses the

flexibility and too absoluteness. For option 4, it is not clear that how the reference decoder is partially specified and captured in RAN4 spec. If RAN4 would like to chose anyone of them, not only between the UE, network and TE but in the field the bad performance will happen and it is not satisfy the request in the latest SID and also the limitation of TU is the key factor for our discussion.

Based on the above analysis, I deem that we shall study the one-sided model firstly in this stage also for for the limitation of TU since the performance for all options will not perform well not only in test but in the field.

Observation 7: The test environment is quite different from the field, so the assumptions is a little limitations and the generalization will be degraded.

Observation 8: The options may not work well, and the requirements based on these assumptions may not have significance.

Observation 9: The TU is limited in RAN4 for AI/ML.

Proposal 6: RAN4 shall study one-sided model firstly and the discussion of two-sided model shall be deprioritized since the two-sided model is only for CSI case.

We can have the table below:

|  | Option 1: DUT provides decoder | Option 2: Decoder not from DUT and Spec | Option 3: Full decoder specification in standard | Option 4: partially specified decoder |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | Options: - RAN4 specifications FFS whether alignment with UE/gNB vendors is required, - Up to decoder implementer (TE vendor) FFS whether alignment with UE/gNB vendors is required - Combination of Option 1 and 2 |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type between DUT and decoder provider (source of training data should be consistent with the collaboration type). | No need to consider training collaboration; Type1/2/3 | No need to consider training collaboration; Type1/2/3 | No need to consider training collaboration; Type1/2/3 | No need to consider training collaboration; Type1/2/3 |
| Test decoder performance verification procedure at TE and/or DUT | Not needed (UE has full knowledge) | Needed (it needs to be proved that the decoder works such that test failure is because of DUT) | Not needed (fully specified in spec.) | Not needed (Test validation would follow the typical RAN5/testing procedures) Needed |
| Feasibility of test decoder verification procedure | NA | Feasible if network vendor provides a test encoder and passes the test Other options? | NA | Feasible if TE vendors shares test decoder |
| Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration) | [One] | [Option A: One Option B: More than one Option C: RAN4 doesn’t need to make decision] | [One] |  |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Low - There could be large performance mismatch with field performance due to mismatch between test decoder and field decoder implemented by gNB - Depends on the training data set | High Model which is similar with the test model could be used in the actual deployment by gNBs | Low There could be large performance mismatch with field performance due to mismatch between test decoder and field decoder implemented by gNB | Medium/Low - The test decoder may have a large mismatch with the decoders deployed in the field, and UE may easily pass the test since UE could train the model based on the specified decoder - May partially reflect the performance in real deployment based on specified parts of test decoder - Depends on the data sets used for training |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | - TE will need to support a wide range of architectures/interfaces/algorithms (at least one per UE vendor). TE Computational resources requirements should be defined. No additional training required by TE vendor. - Medium. TE needs to cope with multiple decoders from multiple UE vendors | - TE will need to support a considerable range of architectures/interfaces/algorithms (at least one per infra vendor). TE Computational resources requirements should be defined. No additional training required by TE vendor. - Medium. TE need to implement multiple decoders from different BS vendors - Hgh | Low Least complexity required on the TE side | Medium If TE is responsible for the training of test decoder, the requirements for TE is high. If UE or gNB provide the test decoder, the requirements for TE is the same as option 1 and 2. |
| Specification Effort (e.g. test decoder) | Low Some conditions on the test decoder might be needed to ensure it can be implemented by TE | Low Some conditions on the test decoder might be needed to ensure it can be implemented by TE | High Long/complicated discussion expected in RAN4 to derive the test decoder | Medium to high Less heavy workload than Option 3 Long/complicated discussion expected in RAN4 to agree on the test decoder to be specified in RAN4 |
| Confidentiality/ IP issues | FFS Need to be considered DUT vendor might have to expose some aspects of the design to the TE vendor Depending on means used to share test decoder, TE vendors might require integrating source code from third party, which could even require licensing | FFS Need to be considered Decoder vendor might have to expose some aspects of the design to the TE vendor Depending on means used to share test decoder, TE vendors might require integrating source code from third party, which could even require licensing | None Decoder is fully captured in the specifications | - None - Depending on the source of data used for training the model to be specified, there might be confidentiality issues in this option. |
| Applicability to different scenarios/conditions/ configurations | Maybe - pending on how to design the test to guarantee the generalization - The model should be applicable for the scenarios/ configurations tested for in RAN4 | Maybe - pending on how to design the test to guarantee the generalization - The model should be applicable for the scenarios/ configurations tested for in RAN4 | Maybe - pending on how to design the test to guarantee the generalization - The model should be applicable for the scenarios/ configurations tested for in RAN4 | Maybe - pending on how to design the test to guarantee the generalization - The model should be applicable for the scenarios/ configurations tested for in RAN4 |
| Complexity of actual testing procedure for the ecosystem | Medium/High Potentially, for each DUT, TE vendor will need to integrate its test decoder (if not leveraging from a previous design) before enabling test. When executing test, DUT vendor will need to make a manufacturer declaration indicating the test decoder(s) they want to be tested against and for which scenarios (only one or more than one if the DUT is using different AI/ML models for different scenarios). | High DUT will need to be tested against one or multiple test decoders provided by different NW vendors (manufacturer declaration?) Potentially, for each NW test decoder (or even test decoder update?), TE vendor will need to integrate its test decoder (if not leveraging from a previous design) before enabling test. | Low No additional TE integration required once initial implementation of the test system is completed. | Low/Medium All DUT are supposed be tested against equivalent TE vendor implementation of the test decoder (only one). No additional TE integration required once initial implementation of the test system is completed. |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Yes | Yes | No | No/Maybe |
| Relationship with reference decoder/encoder for defining requirement | [Alt 1: same as reference decoder May not be possible to define requirements as there could be larger performance gap among companies. The results may not be able to be calibrated. Alt 2: different from reference decoder UE may not pass the tests due to different test decoders are used for defining requirements and tests.] | [Alt 1: same as reference decoder May not be possible to define requirements as there could be larger performance gap among companies. The results may not be able to be calibrated. Alt 2: different from reference decoder UE may not pass the tests due to different test decoders are used for defining requirements and tests.] | [Alt 1: same as reference decoder Possible to define requirements and be able to calibrate results from companies. Alt 2: different from reference decoder There is no reason to specify test decoder different from that is used for defining requirements. ] | [Alt 1: same as reference decoder There is good chance that the results among companies can be calibrated as the performance of the model could largely be decided by the specified part. Possible to define requirements Alt 2: different from reference decoder There is no reason to specify different test decoder than that is used for defining requirements.] |
| Whether model transfer/delivery is needed during the test procedure | [Yes] | [Yes] | [No] | [No] |

The next issue is the test dataset generation, RAN4 has three options as below:

Test dataset generation should be studied. Different generating methods can be used for different tests. The following candidate methods are to be considered or down-selected: - Option a: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc. - “Legacy approach” refers legacy test in which a channel model is used - Option b: Field dataset (data collected directly from field measurements) - Option c: TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data) - Other methods are not precluded

- For option a,

- Currently most of simulation results of AI/ML models in RAN1 are provided based on the dataset 	generated by 3GPP channel models. 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models. In RAN1, the simulation is also based on the dataset in TS 38.901 (e.g. Dense Urban data is 	simulated for CSI use case).

- For option b,

- The problem of Option-b for field dataset (data collected directly from field measurements), because 	of the difficulty/cost to get field dataset which can be fully recognized by 3GPP. Unless a fully 	trustable 3rd party want to spend efforts to build up the field dataset, it is hard for 3GPP to follow 	this Option b for test data generation. And the next question how to guarantee the proposed field 	dataset is representative enough.

- For option c,

- Compared to the option a, there is a few difference that the test dataset could be very large by 	considering all possible random variables which may be presented for the rule/function to generate 	data.

- Based on the stable performance and convenience, the option a can be the starting point to be 	studied in RAN4.

Observation 10: 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models.

Proposal 7: In order to guarantee the stable performance and convenience, RAN4 shall study and use the dataset based on TR 38.901 firstly.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML :

- Observation 1: Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x based on RAN1 outcomes.

- Proposal 1: RAN4 shall not study the interoperability aspect for level x based on previous meetings in RAN1.

- Observation 2: For the level y collaboration, it is clarified as the signaling-based collaboration without model transfer.

- Proposal 2: RAN4 needs to consider the interoperability for collaboration level y based on more RAN2 progress since it is the signaling-based collaboration.

Observation 3: Two categories of models including Proprietary-format models and Open-format models were proposed by RAN1. For the Proprietary-format models, due to the lack of inter-operation and recognition between vendors, it is hard to standardize based on the unified specification identification.

- Observation 4: Regarding to the Open-format models, the interoperability is feasible.

Proposal 3: RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models.

Proposal 4: RAN4 shall study the basic structure of the open-format and consider how the common understanding defined between different vendors.

Proposal 5: From the perspective of test, both functionality test and performance test should be considered.

- Observation 5: model inference is the core component of AI/ML. Two aspects should be considered to verify: 1) The outputs are the results from the AI/ML inference model rather than the traditional solution; 2) The accuracy of outputs meet the requirement.

- Observation 6: the latency requirements and the KPI and the threshold for judging the model performance shall be considered in RAN4.

Observation 7: The test environment is quite different from the field, so the assumptions is a little limitations and the generalization will be degraded.

Observation 8: The options may not work well, and the requirements based on these assumptions may not have significance.

Observation 9: The TU is limited in RAN4 for AI/ML.

Proposal 6: RAN4 shall study one-sided model firstly and the discussion of two-sided model shall be deprioritized since the two-sided model is only for CSI case.

Observation 10: 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models.

Proposal 7: In order to guarantee the stable performance and convenience, RAN4 shall study and use the dataset based on TR 38.901 firstly.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- [2] R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated