3GPP TSG-RAN WG4 Meeting #109 	R4-2320577

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	NCR Demodulation Performance Requirements

Agenda item:	8.28.6

Document for:	Discussion

Introduction

In RAN4#108 bis, the discussion on Network-Controlled Repeater (NCR)-Mobile Termination (MT) demodulation performance requirements has taken place, and the way forward (WF) is documented in [1]. The remaining open issues carried from RAN4#108 in [2] have been agreed, while one new issue concerning the FRC has been brought up for further discussion in RAN4#109. The agreements and open issues from RAN4#108bis are summarized below:

Issue 1-1-1: Whether define new PDCCH requirements for DCI format 5_0/2_8 - Agreement: - No need to define new DCI format 5_0/2_8 requirements. Reuse same DCI type from IAB-MT and legacy UE requirements for NCR-MT PDCCH requirements. Following table used for simulation purpose on the requirements with legacy DCI. Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix. 10; 48; 1; 8; TDLA30-10; 2x4 Low. Issue 1-2-1: Whether to define PMI requirements - Agreement: - No need to define PMI requirements for NCR-MT. Issue 1-3-1: Which FRC can be reused for NCR-MT - Agreement: - FRCs need to be based on IAB-MT style testing. - For test cases in which the corresponding FRCs are available in IAB-MT specification, reuse the FRC from IAB-MT specification. - For the remaining test cases, companies are encouraged to provide FRCs.

Discussion

As in the summary above, RAN4 has agreed in RAN4#108bis to not define new requirements for PDCCH DCI type 2_8 and PMI reporting. Concerning PDCCH requirements, RAN4 will still define requirements for NCR-MT by reusing IAB-MT and UE requirements with one additional required test case as agreed. Hence, summarizing from RAN4 agreements in related previous meetings, the requirements for NCR-MT will mainly be defined for PDSCH, PDCCH and CQI reporting.

Concerning the open issue from RAN4#108bis, the corresponding FRCs for PDSCH and PDCCH requirements are still needed to be defined and agreed in RAN4, as stated below:

Issue 1-3-1: Which FRC can be reused for NCR-MT - Agreement: - FRCs need to be based on IAB-MT style testing. - For test cases in which the corresponding FRCs are available in IAB-MT specification, reuse the FRC from IAB-MT specification. - For the remaining test cases, companies are encouraged to provide FRCs.

Based on the agreed test cases, below we provide related FRCs from related specifications, namely, IAB specification [3] and UE specification [4]:

Table 1. Test Cases and FRCs for PDSCH FR1

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Test metric | FRC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER | R.PDSCH.1-1.1 FDD |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER | R.PDSCH.1-1.1 FDD |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER | R.PDSCH.1-2.1 FDD |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER | R.PDSCH.1-2.1 FDD |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER | R.PDSCH.2-1.1 TDD |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER | R.PDSCH.2-1.1 TDD |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER | M-FR1-A.3.1-1 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER | M-FR1-A.3.1-1 |

Table 2. Test Case and FRC for PDSCH FR2

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric | FRC |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER | R.PDSCH.5-1.1 TDD |

Table 3. Test Case and FRC for PDCCH FR1 10 MHz

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | FRC |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | R.PDCCH.1-1.3 FDD |

While we provide FRCs for the considered test cases for PDSCH and PDCCH as above, we noted here that there is a proposal for FRCs in [5], which was agreed in offline email discussions among companies to be used for simulation for RAN4#109 discussions.

Concerning the simulation activities, we provide our simulation results on NCR-MT PDSCH and PDCCH in [6] as reference to be discussed in RAN4#109 in the simulation alignment process.

Test metrics and test cases for NCR-MT PDSCH have been agreed by interested companies in RAN4#108.

Simulation results for NCR-MT PDSCH from interested companies are available in RAN4#108, RAN4#108bis and RAN4#109 meetings.

There are already proposals on FRCs for the agreed test cases.

RAN4 to finalize the FRCs for NCR-MT specification.

RAN4 to discuss the PDSCH simulation results, do the alignment and decide on the final requirements on PDSCH for NCR-MT.

Conclusion

In this paper, we provide our views on the remaining open issues on NCR demodulation performance requirements. They are summarized in the following Observations and Proposals:

Observation 1: Test metrics and test cases for NCR-MT PDSCH have been agreed by interested companies in RAN4#108.

Observation 2: Simulation results for NCR-MT PDSCH from interested companies are available in RAN4#108, RAN4#108bis and RAN4#109 meetings.

Observation 3: There are already proposals on FRCs for the agreed test cases.

Proposal 1: RAN4 to finalize the FRCs for NCR-MT specification.

Proposal 2: RAN4 to discuss the PDSCH simulation results, do the alignment and decide on the final requirements on PDSCH for NCR-MT.

- References

- R4-2316923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108bis, Xiamen, China, October 09 – 13, 2023

- R4-2313923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.7.0, 12-2022

- R4-23xxxxx, Discussion on demodulation performance requirements for NR network-controlled repeaters, Huawei, HiSilicon, 3GPP RAN4#109, Chicago, USA, November 13 - 17, 2023

- R4-2320578, Simulation Results on NCR-MT PDSCH and PDCCH Demodulation Requirements, Nokia, Nokia Shanghai Bell, 3GPP RAN4#109, Chicago, USA, November 13-17, 2023