3GPP TSG-RAN WG4 Meeting #109 	R4-2320578

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on NCR PDSCH and PDCCH Demodulation Requirements

Agenda item:	8.28.6

Document for:	Information

Introduction

In [1], the way forward (WF) from the discussions on NCR (Network-Controlled Repeaters) demodulation in RAN4#108bis is documented. The agreements in the WF related to simulations for NCR-Mobile Termination (MT) for the requirements are highlighted below:

Issue 1-1-1: Whether define new PDCCH requirements for DCI format 5_0/2_8 - Agreement: - No need to define new DCI format 5_0/2_8 requirements. Reuse same DCI type from IAB-MT and legacy UE requirements for NCR-MT PDCCH requirements. Following table used for simulation purpose on the requirements with legacy DCI. Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix. 10; 48; 1; 8; TDLA30-10; 2x4 Low. Issue 1-3-1: Which FRC can be reused for NCR-MT - Agreement: - FRCs need to be based on IAB-MT style testing. - For test cases in which the corresponding FRCs are available in IAB-MT specification, reuse the FRC from IAB-MT specification. - For the remaining test cases, companies are encouraged to provide FRCs.

Simulation Results on PDSCH Requirements

In this paper, we provide simulation results on PDSCH requirements for the test metrics and test cases agreed in RAN4#108 [2]. Our observations and proposals in details on the remaining open issues on NCR-MT demodulation can be found in [3].

While there are already agreements on the test cases in RAN4#108, nonetheless, there are no details on the simulation parameters discussed by the companies in RAN4#108. As FRCs are open issues to be discussed in RAN4#109, we provide our proposal on FRCs for NCR-MT in [3]. They are using related simulation parameters of UE and IAB-MT PDSCH requirements as in [4] and [5].

We note here that there is an offline discussion among companies via email, in which companies agreed to run simulation for the discussion in RAN4#109 based on the proposed FRCs in [6]. Another note here is that, in the simulation results, for 1% BLER we consider it from the 1st transmission without HARQ retransmission, while for 70% Throughput from the HARQ retransmission with number of retransmissions 4.

PDSCH Simulation in FR1

In the following, we provide our simulation results on NCR PDSCH requirements in FR1, i.e., using FR1 FRCs. The corresponding FRCs are based on the proposed FRCs in [6]. Our simulation results on PDSCH in FR1 are tabulated in Table 1 below.

Table 1. Simulation Results on PDSCH in FR1

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70 | -2.45 | 1 | 0.65 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70 | -5.21 | 1 | -2.56 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70 | 4.32 | 1 | 8.26 |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70 | 1.44 | 1 | 4.61 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70 | -2.89 | 1 | 0.016 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70 | TBA | 1 | TBA |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70 | 4.09 | 1 | 7.19 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70 | 1.72 | 1 | 4.09 |
| Note: For the highlighted test cases, the current values are based on FRCs in [3]. At the time of the writing of this contribution, the updated simulation based on the FRCs in [6] is still ongoing. Updated simulation results of those test cases will be provided during the review week or on the first day of the meeting. |  |  |  |  |  |  |  |  |  |

PDSCH Simulation in FR2

In the following, we provide our simulation results of NCR PDSCH requirements in FR2, i.e., using FR2 FRCs. The simulation results on PDSCH in FR2 is tabulated in Table 2 below.

Table 2. Simulation Results on PDSCH in FR2

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Reference Values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70 | 3.86 | 1 | 5.27 |
| Note: For the highlighted test case, the current value is based on FRC in [3]. At the time of the writing of this contribution, the updated simulation based on the FRCs in [6] is still ongoing. Updated simulation result will be provided during the review week or on the first day of the meeting. |  |  |  |  |  |  |  |  |

Simulation Results on PDCCH Requirements

As stated in the WF, there is only one additional test case in which companies are needed to provide the simulation result, namely, for 10 MHz, aggregation level 8 and antenna configuration of 2x4 low. Below is the simulation result for the considered test case presented in Table 3:

Table 3. Simulation Results on PDCCH in FR1 FDD 10 MHz, 2x4, Aggregation Level 8

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | 1 | -4.3 |

It was previously agreed that RAN4 will reuse the IAB-MT and UE specifications for PDCCH requirements. For reference, we also provide in Appendix, our complete set of simulation results on PDCCH based on the previously agreed parameters.

Conclusion

In this paper, we provide our simulation results on PDSCH and PDCCH for NCR-MT demodulation requirements. The PDSCH simulation results take into account the new agreed metrics, namely, BLER 1%. More on our observations and proposal can be found in [3].

- References

- R4-2316923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108bis, Xiamen, China, October 09 – 13, 2023

- R4-2313923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023

- R4-2320577, NCR Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell, 3GPP RAN4#109, Chicago, USA, November 13 – 17, 2023

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.7.0, 12-2022

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- R4-23xxxxx, Discussion on demodulation performance requirements for NR network-controlled repeaters, Huawei, HiSilicon, 3GPP RAN4#109, Chicago, USA, November 13 - 17, 2023

- Appendix

In the following, for reference, we provide our complete simulation results on PDCCH in tables:

Table 4. Simulation Results on PDCCH FR1 TDD

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | 1 | 3.4 |
| 40 | 102 | 1 | 4 | TDLA30-10 | 1x2 Low | 1 | 1.0 |
| 40 | 90 | 1 | 8 | TDLA30-10 | 2x2 Low | 1 | -1.4 |
| 40 | 102 | 1 | 2 | TDLA30-10 | 1x4 Low | 1 | -1.6 |
| 40 | 102 | 1 | 4 | TDLA30-10 | 1x4 Low | 1 | -3.7 |
| 40 | 90 | 1 | 8 | TDLA30-10 | 2x4 Low | 1 | -6.0 |

Table 5. Simulation Results on PDCCH in FR1 FDD

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 Low | 1 | 7.3 |
| 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 Low | 1 | 4.7 |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 Low | 1 | 0.7 |
| 10 | 24 | 2 | 2 | TDLA30-10 | 1x4 Low | 1 | -0.2 |
| 10 | 48 | 2 | 4 | TDLA30-10 | 1x4 Low | 1 | -2.0 |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | 1 | -4.3 |

Table 6. Simulation Results on PDCCH in FR2

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 Low | 1 | 3.0 |
| 100 | 60 | 1 | 4 | TDLA30-75 | 1x2 Low | 1 | 0.6 |
| 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 Low | 1 | -2.1 |