3GPP TSG-RAN WG4 Meeting #109 	R4-2320580

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on HST FR2 Enhanced with Carrier Aggregation

Agenda item:	8.12.3.2

Document for:	Information

Introduction

In RAN4#107, RAN4#108 and RAN4#108bis some companies including Nokia have provided simulation results on HST FR2 UE Demodulation PDSCH with Carrier Aggregation (CA). The simulation collection from RAN4#107 is documented in [1], in which the simulation results were without considering any impairments. The simulation collections from RAN4#108 and RAN4#108bis are documented in [2] and [3], respectively, in which companies provided simulation results for the two cases, namely, with and without impairments.

The WF from RAN4#108bis on HST FR2 PDSCH with CA is documented in [4], which is also including the FRCs for HST FR2 with CA.

Discussion

The simulation results on HST FR2 UE Demodulation PDSCH with CA without impairments are tabulated in Table 1 below:

Table 1. Simulation Results on HST FR2 PDSCH with CA without Impairments

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,4 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,5 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,4 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10,5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,4 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,5 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,4 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10,5 |

The simulation results on HST FR2 PDSCH with CA with impairments are tabulated in Table 2 below:

Table 2. Simulation Results for Different Test Cases for HST FR2 with CA with impairments

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 13.1 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 13.1 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |

As expected, the simulation results with impairments require higher SNR to achieve the same target performance of 70% throughput compared to without impairments. We provide our observations and proposals related to the simulation results above in [5].

Conclusion

In this paper, we provide our simulation results on HST FR2 UE Demodulation PDSCH with CA, in which the simulation results are considering both with and without impairments. As expected, the simulation results with impairments require higher SNR than without impairments to achieve the same target performance of 70% throughput.

- References

- R4-2308834, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4#107, Incheon, South Korea, May 22 - 26, 2023.

- R4-2312211, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2316148, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4 #108bis, Xiamen, China, October 09 – 13, 2023.

- R4-2317007, WF for FR2 HST demodulation requirements, Samsung, RAN4#108bis, Xiamen, China, October 09 - 13, 2023.

- R4-2320579, HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation, Nokia, Nokia Shanghai Bell, RAN4 #109, Chicago, USA, November 13 – 17, 2023.