3GPP TSG-RAN WG4 Meeting #109 	R4-2320582

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception

Agenda item:	8.12.3.3

Document for:	Information

Introduction

The way forward (WF) from the discussions on HST FR2 Enhanced PDSCH with multi-RX in RAN4#108bis is documented in [1]. Several issues related to simulation on PDSCH with multi-RX are summarized below:

Issue 3-1-0: Test requirement to be defined Way forward: - Test cases to be introduced - Option 1: one case with RTD larger than CP - Option 2: two cases based on UE declaration on supported baseband processing with RTD larger than CP or not - Case 1: RTD = 1.0 CP - Case 2: RTD larger than CP Issue 3-1-1: UE processing assumption for the FFT window Issue 3-1-2: Necessity to introduce RTD into channel model for CPE FFT processing test Issue 3-2-1: Number of MCS for PDSCH requirement for multi-Rx reception Issue 3-2-2: Number of SNR for PDSCH requirement for multi-Rx reception Way forward: - Introduce RTD in the FR2 HST PDSCH requirement between the different RX panels. Discuss RTD value based on evaluation. - Note RTD: Timing offset of TRP2 from TRP1 - Define one fixed MCS value per each Panel for PDSCH requirement with multi-Rx reception with fixed FRC - Encourage companies to evaluate the performance difference under assumption on FFT (single FFT across Rx chains, and separate FFT per RF chain) with the following test setup and test metric, make a decision in RAN4#109 meeting - RTD for evaluation with the following priority order - Option 1: (2CP) 1.2 us - Option 2: (1 CP): 0.57us - Option 3a (1.2CP): 0.7us and Option 3b: (2.5CP) 1.5us - MCS - Set MCS 19 for TRP1 and Set MCS Y for TRP2 for FR2 HST simultaneous multi-Rx scenario. - Candidate MCS Y = {MCS 11, MCS 13} - Other feasible MCS are not precluded. - Expected power imbalance value and RTD value according to the deployment model (depending on UE Location) can be considered to derive suitable MCS pair (MCS 19, MCS Y). For example - Step 1: Derive the UE location based on given RTD value; - Step 2: Derive the expected power imbalance value x dB based on the UE location - Step 3: Run simulation to find SNR pair (SNR1, SNR2) for MCS pair (MCS 19, MCS Y) derived based on test metric 70% of TP for each PDSCH. - Choose the highest MCS Y according to the following condition: - SNR1(MCS19) – SNR2(MCS Y) =< x dB - To be decided whether SNR based on averaged submitted simulation results or individual SNR values reported. - Test metric for SNR derivation - Option 1: 70% Tput for each PDSCH as baseline. - Option 2: 70% Tput across all PDSCH - Step 1: Run simulation to find one SNR = y dB so that SNR pair {y dB, y-x dB} derived based on MCS pair (MCS 19, MCS Y) is fulfilled the following test metric - (Tput1 from TRP1 (MCS19) + Tput2 from TRP2 (MCS Y)) > 70% * (Tput1_max from TRP1 + Tput2_max from TRP2) Issue 3-2-3: Power scaling for two served RRH Agreement: - Do not consider the power scaling factor 1/sqrt(2) to transmitted PDSCH signal from each TRxP

In RAN4#108, beside the WF in [2], another outcome is the updated simulation assumptions and parameters in [4]. The simulation assumptions and parameters in [4] can be used by companies to provide their simulation results. For a direct reference, we provide the simulation parameters for HST FR2 PDSCH with multi-RX as in [4] in the Appendix.

Discussion

In this paper, we provide our simulation results based on the simulation assumptions and parameters in [4]. Furthermore, as agreed in [1], our simulation results do not consider the power scaling factor.

Simulation Results without RTD and Power Difference

In the following, we provide our simulation results for the agreed case, i.e., mDCI with 2+2 layer combination and independent processing, based on the simulation assumption in [4] in Table 1:

Table 1. Simulation Results on HST FR2 PDSCH with multi-RX with mDCI and independent processing with independent FFT per panel without RTD

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (17,13) | 2 | 2x2 Low | 70 | (11.4, 7.5) |
| 200 | 120 | 30 | (17,19) | 2 | 2x2 Low | 70 | (11.4, 12.6) |

Simulation Results with RTD and Power Difference

There are still some open issues as described in the WF above concerning the use of single FFT versus independent FFT in HST FR2 multi-RX and the related parameters which might impact the performance of HST FR2 with multi-RX. Such open issues need to be finalized before running simulation intended to define the requirements.

As mentioned in our contribution for discussion in RAN4#109 in [3], we assume independent FFT to be applied in HST FR2 with multi-RX. In the following, we provide simulation results for the case of independent FFT which can also be considered in the discussions with other companies on finalizing the assumption and related parameters for simulation on HST FR2 multi-RX.

For the evaluation on the impact of RTD to the performance of HST FR2 with multi-RX and mDCI scheme, the companies are now considering MCS 19 for the first TRP, and the MCS for the second TRP can be either MCS 11 or MCS 13 or any other MCS considered by the companies.

Below are the simulation results for two different RTD cases, namely, 0.57 µs (1 CP) and 1.2 µs (2 CP) with power difference of 4 dB and 8.8 dB, respectively:

Table 2. Simulation results for different values of RTD and the corresponding power difference

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | [RTD (µs), Power Difference (dB)] | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | [0.57, 4] | 70 | (12.6, 9.3) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | [0.57, 4] | 70 | (12.6, 11.3) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | [1.2, 8.8] | 70 | (12.7, 14.1) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | [1.2, 8.8] | 70 | (12.7, 16.2) |

More on our observations and proposals on HST FR2 multi-RX can be found in [3].

Conclusion

In this paper, we provide simulation results with the agreed simulation assumptions and parameters in [4]. Our simulation is considering only the case of independent FFT. More on our observations and proposals related to HST FR2 with multi-RX can be found in [3].

- References

- R4-2317007, WF for FR2 HST demodulation requirements, Samsung, RAN4#108bis, Xiamen, China, October 09 - 13, 2023.

- R4-2313919, WF for FR2 HST demodulation requirements, Samsung, RAN4#108, Toulouse, France, August 21 - 25, 2023.

- R4-2320581, HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4 #109, Chicago, USA, November 13-17, 2023.

- R4-2313920, Simulation assumption for FR2 HST demodulation, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- Appendix

Table A. Simulation assumption for PDSCH demodulation requirements with multi-Rx simultaneous reception [4]

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| UL-DL configuration |  |  |  | FR2.120-1 |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 13 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 2 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | full-overlapping |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| - Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRxP #1 | TRxP #2-1 | TRxP #2-2 |  |
| Channel bandwidth | MHz | 200 | 200 | 200 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 132 | 132 | 132 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 13 | 19 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.48 | 0.50 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 36 | 36 | 36 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 36 | 36 | 36 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 51216 | 33816 | 53288 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 77896 | 57376 | 92200 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 5 | 7 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 10 | 7 | 11 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 125136 | 72864 | 109296 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 182160 | 121440 | 182160 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 441.601 | 319.213 | 511.326 |  |
| - Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms - Note 2: Slot i is slot index per 2 frames - Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. - Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |