3GPP TSG-RAN WG4 Meeting #109	R4-2320585

Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements

Agenda item:	8.36.2

Document for:	Discussion

Introduction

In [1], the Way Forward (WF) of Enhancement of NR Dynamic Spectrum Sharing (eDSS) is documented. It is agreed that RAN4 will define requirements only for PDCCH and will not define requirements for PDSCH. Concerning the PDCCH requirements, there are more detail agreements as follows:

Whether to define PDCCH demodulation requirements for Rel-18 eDSS feature Agreement: - Define PDCCH demodulation requirements for eDSS Evaluation scenario Agreement: - 2-symbol CORESET - Basic assumptions - Rate matching pattern - Option 1: Single non-overlapping CRS rate matching patterns for PDCCH demodulation. - Other options not precluded - 4 CRS ports for LTE - Symbol #0: LTE PDCCH/PCFICH - Symbol #1: NR PDCCH overlapping with LTE CRS - Symbol #2: NR PDCCH gNB assumption for PDCCH symbols overlapped with LTE CRS Agreement: - PDCCH data and DMRS REs overlapped with LTE CRS are punctured. UE receiver assumption (e.g., channel estimation) Agreement: - PDCCH channel estimation is assumed to use only the clean PDCCH symbol.

In this contribution, we will address the remaining open issues from RAN4#108bis.

Discussion

In RAN4#108bis, there is one open issue that needs to be addressed in RAN4#109 meeting, namely:

Simulation assumption for evaluation Way forward: - FFS on the parameter configuration for the PDCCH - Option 1 - Parameters; Values. Carrier frequency; 2 GHz. SCS; 15 kHz. Bandwidth; 10 MHz. Channel model; TDLC300-100. Antenna configuration; 1x2, 1x4; 2x2, 2x4. CRS; 4 port CRS without power boosted. DCI payload (excluding CRC); 60 bits. Interleaving; Non-interleaved. Precoding; Precoder cycling per REG bundle. REG bundle size; 6 REGs. Test number; Bandwidth (MHz); Symbols with PDCCH; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 2; 4; TDLA30-10; 1x2, 1x4 Low; 1. 2; 10; 2; 8; TDLC300-100; 2x2, 2x4 Low; 1. - Option 2 - Cover all PDCCH Aggregation levels for evaluation purpose. (Not for requirements definition purpose) Parameters; Values. Carrier frequency; 2 GHz. SCS; 15 kHz. Bandwidth; 20MHz. LTE Bandwidth; 20MHz. Channel model; TDL-C 300-100. Correlation; Low. Number of BS antennas; 4Tx. Number of UE antennas; 2 Rx. DCI payload (excluding CRC); 60 bits. Interleaving; Non-Interleaved. Precoding; Precoder cycling per REG bundle. REG bundle size; 6 PRBs. CRS; single 4 port CRS pattern. Aggregation level; 1,2,4,8,16. CORESET; Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports. Channel estimation; Scenario a): Use clean symbol Scenario b): Use legacy channel estimation. Transmitter; DMRS REs and data REs overlapped with CRS are punctured. Receiver; Puncture the CRS REs. Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE; 3dB.

Concerning NR PDCCH reception in symbols with LTE CRS REs, in [2] some studies have been conducted on the BLER performance of PDCCH with puncturing versus superpositioning, which show tradeoffs in performance depending on the aggregation level and the power ratio of CRS and PDCCH.

The PDCCH performance of enhanced DSS depends on the power difference between the power ratio of LTE-CRS RE and NR PDCCH DMRS RE as well as the Aggregation level.

RAN4 to consider all aggregation level for PDCCH requirements, namely, 1, 2, 4, 8 and 16.

As it is already agreed to use only clean symbol and the PDCCH data and DMRS REs overlapped with LTE CRS are punctured, RAN4 shall consider the case of without power boosting, in which the power ratio can be set to 0 dB.

RAN4 to consider possible antenna configurations of 1x2, 1x4, 2x2, 2x4.

Conclusion

In this contribution, we provide our views on the remaining open issues on UE demodulation performance requirements of NR eDSS. The following Observations and Proposals were made:

Observation 1: The PDCCH performance of enhanced DSS depends on the power difference between the power ratio of LTE-CRS RE and NR PDCCH DMRS RE as well as the Aggregation level.

Proposal 1: RAN4 to consider all aggregation level for PDCCH requirements, namely, 1, 2, 4, 8 and 16.

Proposal 2: As it is already agreed to use only clean symbol and the PDCCH data and DMRS REs overlapped with LTE CRS are punctured, RAN4 shall consider the case of without power boosting, in which the power ratio can be set to 0 dB.

Proposal 3: RAN4 to consider possible antenna configurations of 1x2, 1x4, 2x2, 2x4.

- References

- R4-2317000 WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, 3GPP RAN4#108bis, Ericsson, Xiamen, China, Oct. 9 ‒ Oct. 13, 2023

- R1-2204823, DSS – NR PDCCH overlapping with LTE CRS, Nokia, Nokia Shanghai Bell, RAN1#109, e-Meeting, 9 – 20 May, 2022