3GPP TSG-RAN WG4 Meeting # 109	R4-2320597
Chicago, US, November 13 – 17, 2023

Agenda item:	5.2.5

Source:	CAICT

Title:	On MIMO OTA Doppler validation pass fail limits

Document for:	Approval

1	Introduction

In this contribution, we share our views on Doppler validation pass/fail limits for FR1 and FR2 channel mode validations.

2	Discussion

After reviewing the FR1 Doppler validation measurement results from 7 labs in [1] and [2], and FR2 Doppler validation measurement results from 6 labs in [3], we found that the measured doppler commonly matched well with the target ones. The current pass/fail limits look over loose. Thus, we propose to tighten the Doppler pass/fail limit appropriately.

Proposal 1: The Doppler pass/fail limits for FR1 and FR2 channel model validation should be tightened appropriately.

In [4], Huawei proposed two options for tightening the Doppler pass/fail limits as below:

Proposal 4a in [4]: The pass/fail limits for temporal correlation are formed as bands of ±10% of correlation capped at 100% from the target defined in clause D.3.3. Additionally, when the upper bound reaches 20%, the limit stays at 20% and the lower limit drops to 0%. The values defined in Table 2 are introduced into the spec. Proposal 4b in [4]: The pass/fail limits for temporal correlation are formed as bands of ±10% of correlation capped at 100% from the target defined in clause D.3.3, and the lower limit drops to 0%. The values defined in Table 3 are introduced into the spec. Figure 2 Pass/fail, target and measured curve of temporal correlation [4]

We think Proposal 4a in [4] is more appropriate.

Proposal 2: The pass/fail limits for temporal correlation are formed as bands of ±10% of correlation capped at 100% from the target. Additionally, when the upper bound reaches 20%, the limit stays at 20% and the lower limit drops to 0%. The pass/fail limits apply for both FR1 and FR2.

3	Conclusion

Proposal 1: The Doppler pass/fail limits for FR1 and FR2 channel model validation should be tightened appropriately.

Proposal 2: The pass/fail limits for temporal correlation are formed as bands of ±10% of correlation capped at 100% from the target. Additionally, when the upper bound reaches 20%, the limit stays at 20% and the lower limit drops to 0%. The pass/fail limits apply for both FR1 and FR2.

References

- R4-2316309, TP to TR 38.761 on channel model validation for n41 and n78, CAICT

- R4-2320061, TP to TR 38.761 on channel model validation for n28, CAICT

- R4-2320062, TP to TR 38.761 on FR2 channel model validation, CAICT

- R4-2213190, Discussion on FR2 MIMO OTA channel model validation, Huawei, HiSilicon