3GPP TSG-RAN WG4 Meeting #109                                                                R4-2320610

Chicago, US, November 13th – 17th, 2023

Source:	Samsung

Title:	General aspects for AI/ML air interface

Agenda item:			8.21.1

Document for:	Discussion

# 1. Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest SID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

In RAN4#106-bis-e and #107, the WFs [3][5] were approved, in which the agreements on how to perform the RAN4 study on general issues, specific issue related to use cases for AI/ML and interoperability/testing aspects were provided. In RAN4#108, WF [8] has not been approved while the relevant agreements are captured in the ad-hoc minutes [7] and chair notes. In RAN4#108bis, WF [10] captured the agreements. In this contribution, we would like to provide our viewpoints on the general aspects for AI/ML for NR air interface.

# 2. Discussion on General Aspects

## 2.1 Handling of generalization

The performance of generalization is of importance for a successful AI/ML feature deployment. For generalization verification aspects, RAN4 agreed to “study the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML”.

#### 2.1.1 Generalization goals

Based on RAN4#108bis, RAN4 achieved the following agreement on the generalization goals:

Issue 1-2: Generalization goals Agreement: - Verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations - FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - FFS on what the minimum level performance for each identified scenario and/or configuration is - FFS on what the significant degradation for other scenarios and/or configurations is

For the discussion on “FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality”, we think this is closely related to the RAN1 agreement for AI/ML functionality and AI/ML model, which are proposed for AI/ML functionality/model identification respectively, as provided in the following content from agreed RAN1 part of TR 38.843:

<RAN1 Agreement, captured from TR38.843 v1.0.0> For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. ... For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG) as determined/identified between UE-side and NW-side. - It doesn’t imply that additional conditions are necessarily specified - Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. - Note: whether specification impact is needed is separate discussion

Accordingly, the following proposal is provided:

Proposal 1: Further understanding on generalization goals is provided/highlighted:

- Verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations

  - FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality

    - The identified scenarios and/or configurations for test

      - shall include: Configuration(s) of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG, for AI/ML model/functionality identification and model-ID-based/functionality-based LCM of UE-side models and/or UE-part of two-sided models;

      - shall NOT include: Additional conditions (e.g., any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG) as determined/identified between UE-side and NW-side

For the other two bullets of FFS, i.e., “FFS on what the minimum level performance for each identified scenario and/or configuration is” and “FFS on what the significant degradation for other scenarios and/or configurations is”, obviously these shall be further studied in normative phase by focusing on the detailed use case. Therefore, there is no need for further discussion on these two bullets in this SI.

Proposal 2: The following two bullets shall be discussed in normative phase by focusing on the particular use case, and no further discussion is needed in SI:

- FFS on what the minimum level performance for each identified scenario and/or configuration is

- FFS on what the significant degradation for other scenarios and/or configurations is

#### 2.1.2 Handling of generalization in tests

Based on RAN4#108bis, RAN4 achieved the following agreement on the handling of generalization in tests:

Issue 1-3: Handling of generalization in tests Agreement: - Take the modified Option 1 as the baseline - Modified Option 1: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify it’s generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined. - FFS on Option 2 - In Option 2, change the same model ID to “the same specified UE configuration, which includes functionality and/or model ID if defined

By comparing the modified Option 1 and Option 2, there are two differences shall be highlighted: (1) the signaling based LCM procedures and performance monitoring are explicitly excluded in the test for generalization; (2) different propagation conditions used in Option 2, which is more specific than the condition used in Option 1. Based on our understanding on “the identified scenarios and/or configurations for test” in proposal 1, we think prefer the modified Option 1, in which the concept of conditions includes broader concept than the propagation condition.

Proposal 3: For handling of generalization in tests, RAN4 adopt modified Option 1.

## 2.2 Requirement for data collection

#### 2.2.1 Data collection for training purpose

In agreed WF [3], RAN4 expect “FFS if requirements for data collection (in particular for training) could/need be defined”. RAN4 already agreed that “RAN4 does not need to study requirements/tests for training, unless other WG defines the training procedure.

From RAN2 perspective, it is agreed that:

##### Agreement from RAN2#121bis-e Related to data collection, the following was agreed: - R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

Furthermore, in RAN2#122, it has been further agreed that there is not latency requirement for data collection for all types of offline training.

##### Agreement from RAN2#122 Related to data collection, the following was agreed: - R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

Furthermore, in RAN2#122, it has been further agreed that there is not latency requirement for data collection

##### Agreement from RAN2#121bis-e Related to data collection, the following was agreed: ... - For the latency requirement of data collection, RAN2 assumes: - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - for model inference, when required data comes from other entities, there is a latency requirement for data collection - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection.

Furthermore, considering the procedure discussion for both on-line/real-time training has already been deprioritized in the whole SI in RAN2, and there is no clear definition of the data collection procedure for on-line/real-time till now, RAN4 shall skip further discussion the relative requirement in this SI.

Based on RAN1’s reply LS (R1-2310681) on data collection requirements and assumptions, the typical latency requirement for data collection for training purpose (for all three use cases) are marked as “Relaxed”, therefore we don’t expect any latency requirement shall be specified in RAN4.

Proposal 4: For data collection for training purpose:

- Latency requirement shall not be specified in RAN4;

- The importance of defining accuracy requirement is use case-specific: Until a clear data collection procedure is defined in RAN1/2 (, i.e., probably in work item stage), there is no necessity for RAN4 to further study on accuracy requirement for data collection for training purpose.

#### 2.2.2 Data collection for inference purpose

The data collection for inference purpose is certainly of necessity, however, it may not need the explicit requirement, e.g., for the AI/ML inference at BS side, the requirement on UE report as one method of data collection shall be considered (e.g., UE report accuracy test similar as measurement accuracy requirement), but explicit requirement for data collection may not be needed for AI/ML inference at UE side in which the data collection is proceeded inside the UE, and the performance can be guaranteed by the inference performance which is targeted for the overall performance. The latter case is also provided as one example in RAN2 agreement:

##### Agreement from RAN2#121bis-e Related to data collection, the following was agreed: - RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - For model inference of UE-sided model, input data for model inference is available inside the UE. - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

On the other hand, RAN2 also assume latency requirement of data collection could be needed if the required data comes from other entities.

##### Agreement from RAN2#121bis-e Related to data collection, the following was agreed: ... - For the latency requirement of data collection, RAN2 assumes: - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - for model inference, when required data comes from other entities, there is a latency requirement for data collection - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection.

Based on RAN1’s reply LS (R1-2310681) on data collection requirements and assumptions, the typical latency requirement for data collection for inference purpose (for all three use cases) are marked as “Time-critical”, therefore we expect RRM requirement shall be specified for data collection delay requirement in RAN4 if the procedure is defined in RAN1/2.

Proposal 5: For data collection for inference purpose,

- RAN4 requirement shall be specified only if the required data comes from other entities and the procedure of data collection is agreed in RAN1/2.

- The necessity of latency and/or accuracy requirement should be discussed for the particular use case for normative work.

#### 2.2.3 Data collection for monitoring purpose

Firstly, as agreed in RAN2, for model monitoring, the latency requirement of data collection is only required if the required monitoring data (e.g., performance metric) comes from the other entities. For accuracy requirement, we assume the requirement shall be similar to existing RAN4 measurement accuracy requirement.

Based on RAN1’s reply LS (R1-2310681) on data collection requirements and assumptions, the typical latency requirement for data collection for monitoring purpose (for all three use cases) are marked as “Near-real-time”, therefore we expect RRM requirement shall be specified for data collection delay requirement in RAN4 if the procedure is defined in RAN1/2.

Proposal 6: the data collection for monitoring purpose,

- RAN4 requirement shall be specified only if the required data comes from other entities and the procedure of data collection is agreed in RAN1/2.

- The necessity of latency and/or accuracy requirement should be discussed for the particular use case for normative work.

## 2.3 Requirements for LCM

As agreed in RAN4#107, RAN4 should study how/whether RAN4 core requirements could be defined for model monitoring in LCM. For the model monitoring (in which the performance of AI/ML model inference and/or the other environment conditions are under monitoring), it is similar to radio link monitoring (RLM, in which the downlink radio link quality on the RLM-RS resources).

Take the CSI compression sub-use case for example: model monitoring can be performed in UE side, in which the CSI and other conditions can be more easily perceived for model monitoring purpose, with monitoring metrics derived including model inference accuracy, system performance, data distribution etc. Furthermore, it is also possible that the reference Encoder/Decoder can be used to derived the metrics to be monitored, though at the expense of UE complexity. These kinds of model monitoring schemes are all listed as possible performance monitoring methods as captured in TR38.843 [RP-231766]:

Performance monitoring The following metrics/methods for AI/ML model monitoring in lifecycle management per use case are considered: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least the following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

Fig. 1: Illustration of model monitoring for CSI compression sub-use case: Two possible model monitoring implementations

Model monitoring is the key module to guarantee the performance of AI/ML model inference, because the generality is widely regarded as the issue for AI/ML operation. However, we also

Proposal 7: RAN4 shall define RAN4 core requirement for performance monitoring tests based on RAN1/2 defined monitoring metrics/methods for particular (sub-)use case for normative work.

- No need further RAN4 study until clear monitoring metrics/methods specified in other WGs.

## 2.4 RAN4 performance testing goals

As discussed in previous RAN4 meetings, there are different understanding on RAN4 test goal for AI/ML-related function, i.e.,

- Option 1: The testing goal is to verify whether a specific AI/ML model can be conducted in a proper way.

  - FFS how to define the specific AI/ML model (e.g., a model captured in RAN4 spec as baseline)

  - FFS how to define that the model is properly conducted (e.g., by defining AI/ML dedicated performance/core requirements associated with model outputs)

- Option 2: The testing goal is to verify whether the performance gain of AI/ML model can be achieved for a static scenario/configuration.

  - FFS how to define a static scenario/configuration (e.g., by defining a related testing dataset based on channel models in TR 38.901)

  - FFS whether to define non-static specific scenarios/configurations

- Option 3: Option 1 or 2 depending on test

Our understanding is Option 3, i.e, both Option 1 and 2 are needed, which could be used to guarantee the LCM procedure work properly as Option 1 and performance gain by AI/ML model over baseline performance as Option 2.

Proposal 8: For RAN4 performance testing goal, Option 3 (Option 1 or 2 depending on test) is preferred to guarantee LCM procedure and performance gain in different use cases, both of which shall be considered in RAN4 in case-by-case manner.

# 3. Conclusion

In this contribution, we provided our viewpoints on the general aspects for AI/ML air interface, accordingly the following observations and proposals are obtained.

Handling of generalization

Proposal 1: Further understanding on generalization goals is provided/highlighted:

- Verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations

  - FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality

    - The identified scenarios and/or configurations for test

      - shall include: Configuration(s) of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG, for AI/ML model/functionality identification and model-ID-based/functionality-based LCM of UE-side models and/or UE-part of two-sided models;

      - shall NOT include: Additional conditions (e.g., any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG) as determined/identified between UE-side and NW-side

Proposal 2: The following two bullets shall be discussed in normative phase by focusing on the particular use case, and no further discussion is needed in SI:

- FFS on what the minimum level performance for each identified scenario and/or configuration is

- FFS on what the significant degradation for other scenarios and/or configurations is

Proposal 3: For handling of generalization in tests, RAN4 adopt modified Option 1.

Requirement for data collection

Proposal 4: For data collection for training purpose:

- Latency requirement shall not be specified in RAN4;

- The importance of defining accuracy requirement is use case-specific: Until a clear data collection procedure is defined in RAN1/2 (, i.e., probably in work item stage), there is no necessity for RAN4 to further study on accuracy requirement for data collection for training purpose.

Proposal 5: For data collection for inference purpose,

- RAN4 requirement shall be specified only if the required data comes from other entities and the procedure of data collection is agreed in RAN1/2.

- The necessity of latency and/or accuracy requirement should be discussed for the particular use case for normative work.

Proposal 6: the data collection for monitoring purpose,

- RAN4 requirement shall be specified only if the required data comes from other entities and the procedure of data collection is agreed in RAN1/2.

- The necessity of latency and/or accuracy requirement should be discussed for the particular use case for normative work.

Requirements for LCM

Proposal 7: RAN4 shall define RAN4 core requirement for performance monitoring tests based on RAN1/2 defined monitoring metrics/methods for particular (sub-)use case for normative work.

- No need further RAN4 study until clear monitoring metrics/methods specified in other WGs.

RAN4 performance testing goals

Proposal 8: For RAN4 performance testing goal, Option 3 (Option 1 or 2 depending on test) is preferred to guarantee LCM procedure and performance gain in different use cases, both of which shall be considered in RAN4 in case-by-case manner.

# 4. Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#96.

[3] R4-2306299, “WF on AI/ML RAN4 studies”, Qualcomm, RAN4#106bis-e.

[4] R4-2305199, “Interoperability and testability aspect for AI/ML air interface”, Samsung, RAN4#106bis-e.

[5] R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm, RAN4#107.

[6] R4-2309317, “Interoperability and testability aspect for AI/ML air interface”, Samsung, RAN4#107.

[7] R4-2314907, “Ad hoc minutes for FS_NR_AIML_air”, Qualcomm, RAN4#108.

[8] R4-2314740	, “WF on FS_NR_AIML_air”, Qualcomm, RAN4#108 (not approved).

[9] R4-2313800, “General aspects for AI/ML air interface”, Samsung, RAN4#108.

[10] R4-2317631, “WF on RAN4 requirements for NR AI/ML”, Qualcomm, RAN4#108bis.

[11] R4-2316597, “General aspects for AI/ML air interface”, Samsung, RAN4#108bis.

# 5. Appendix: RAN4 Agreements on General Aspects

In RAN4#106-bis-e WF [3], the following agreements have been provided for the general aspects:

### RAN4 Scope and baseline performance Agreement: - General aspects - RAN4 will study how to define requirements and tests for inference - RAN4 does not need to study requirements/tests for training - If other WG defines the training procedure, RAN4 may need study to define the requirements for it. - RAN4 could evaluate feasibility of requirements/tests for LCM - Progress of the discussion will depend on RAN1/2 progress on these procedures - FFS if requirements for data collection (in particular for training) could/need be defined - Defining AI/ML requirements - For the cases with the existing legacy performance - Take the legacy performance as baseline for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - FFS how to define “legacy performance” (whether on meeting/exceeding existing RAN4 requirements, or a wider criterion taking into account generalization) - New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - For the cases without the existing legacy performance - New or enhanced performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods - Generalization verification aspects - Study the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML - One sided and 2-sided models - RAN4 to consider both models, discussion can continue in parallel. ### Training dataset definition Agreement: - Dataset to be used for the device model training is left to implementation - If a specific test for training is defined, RAN4 might have to introduce some conditions and/or accuracy requirements for the training dataset or training data generation ### High level testing framework Agreement: - RAN4 should design the tests such that performance is guaranteed and to avoid that a UE can easily pass the test but perform poorly in the field. - This framework is not directly enforceable but should be considered for all the tests to be introduced - This also applies to LCM tests, if they are defined.

In RAN4#107 WF [5], the following agreements have been reached which are related to the scope discussion:

### Agreements in ad-hoc session: Issue 1-6: Performance monitoring tests Option 3: RAN4 should study how/whether RAN4 core requirements could be defined for model monitoring in LCM

In RAN4#108 [7], the following agreements have been reached which are related to the scope discussion:

Issue 1-4: AI/ML model complexity Agreement: - The practical processing capability and implementation complexity for device under test should be assumed when specifying RAN4 requirements. - The UE capability may be needed to handle different complexity for one side and two-side models. - The complexity of UE should also be studied when making assumption on BS side model, and vice versa.

In RAN4#108bis [10], the following agreements have been reached:

Issue 1-2: Generalization goals Agreement: - Verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations - FFS on details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - FFS on what the minimum level performance for each identified scenario and/or configuration is - FFS on what the significant degradation for other scenarios and/or configurations is Issue 1-3: Handling of generalization in tests Agreement: - Take the modified Option 1 as the baseline - Modified Option 1: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify it’s generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined. - FFS on Option 2 - In Option 2, change the same model ID to “the same specified UE configuration, which includes functionality and/or model ID if defined