3GPP TSG-RAN WG4 Meeting #109	R4-2320639

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	FRCs for 5 MHz channel bandwidth in 30 KHz SCS

Agenda item:	4.1

Document for:	Discussion

Introduction

In the Annex of 38.101-1 [1], there are fixed reference channels defined which are applicable for measurement of the receiver characteristics. There are multiple tables for different modulation schemes, multiplexing scheme, and sub-carrier spacing. Each table includes multiple columns corresponding to different bandwidths. However, it seems that there are some redundant columns in the table. We have brought CRs [2—5] in order to rectify the situation, and this discussion paper details our motivation for the same.

Discussion

UE channel bandwidths for NR bands

Table 5.3.5-1 [1] specifies the applicable channel bandwidths for all NR bands. It can be seen from the table that no NR band supports channel bandwidth of 5MHz in case of 30 KHz SCS. This is also true for V2X bands (Table 5.3E.1-1). This theme continues in both receiver and transmitter specifications, with missing SEM, reference sensitivity, etc, for 5 MHz channel bandwidth with 30 KHz SCS. However, the FRCs seems to be defined for this particular case, which seems unnecessary. Further, Table 5.3.2-1 does allow 30 KHz SCS for 5 MHz channel bandwidth, but given no band actually allows it, again makes it confusing.

None of the bands defined, uses 30 KHz SCS for 5 MHz channel bandwidth.

Agree to delete the 5 MHz channel bandwidth column from the 30 KHz SCS in FRC tables.

Agree to the companion CRs [2—5].

Encourage other companies to debate if Table 5.3.2-1 should be corrected too.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: None of the bands defined, uses 30 KHz SCS for 5 MHz channel bandwidth.

Proposal 1: Agree to delete the 5 MHz channel bandwidth column from the 30 KHz SCS in FRC tables.

Proposal 2: Agree to the companion CRs [2—5].

Proposal 3: Encourage other companies to debate if Table 5.3.2-1 should be corrected too.

- References

- TS 38.101-1, User Equipment (UE) radio transmission and reception, v15.23.0.

- R4-2320632, [NR_newRAT] CR to 38.101-1 on FRC deletion for 5MHz 30 KHz, Nokia, Nokia Shanghai Bell, RAN4#109, Chicago, United States, November 13 – November 17, 2023.

- R4-2320633, [NR_newRAT] CR to 38.101-1 on FRC deletion for 5MHz 30 KHz, Nokia, Nokia Shanghai Bell, RAN4#109, Chicago, United States, November 13 – November 17, 2023.

- R4-2320634, [NR_newRAT] CR to 38.101-1 on FRC deletion for 5MHz 30 KHz, Nokia, Nokia Shanghai Bell, RAN4#109, Chicago, United States, November 13 – November 17, 2023.

- R4-2320635, [NR_newRAT] CR to 38.101-1 on FRC deletion for 5MHz 30 KHz, Nokia, Nokia Shanghai Bell, RAN4#109, Chicago, United States, November 13 – November 17, 2023.