3GPP TSG-RAN WG4 Meeting # 109	R4-2320640

Chicago, US, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell, Charter Communications, Spark NZ Ltd.

Title:	TP to TR 38.858 Section 11: Adjacent channel co-existence evaluation results

Agenda item:	8.19.2.1

Document for:	Approval

Introduction

In 2022, a new study item work on evolution of duplex operation [1] was started, with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation.

The detailed objectives specified in [1] are as follows:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution we discuss the coexistence conclusions agreed on RAN4#108bis and propose changes to the conclusions in a TP.

Discussion

The latest draft of TR 38.858 including all the accepted TPs in RAN4#108bis is found in [2]. Here, we propose a TP for Annex E, with the following changes:

- Adding a note in Table E.2.1-1 and Table E.2.1-2 to clarify how the indoor distance is calculated for the UE-to-UE penetration loss calculations. This is in line with the agreements reached in RAN4#108 (R4-2113872)

- In Table E.2.1-1 (micro deployment column), removing a sentence that indicates that free space model should be applicable for 2D distances lower than 10m. The outdoor UE – outdoor UE path loss model in TR 38.828 is applicable to distances lower than 10 m and therefore the application of FSPL is not needed.

- Adding notes in Table E.2.1-3 to clarify the cluster-based UE dropping. This is in line with the agreements reached in RAN4#108 (R4-2113872)

- Adding subsections E.2.3.1 and E.2.3.2 to improve the structure of the Annex. Subsection E.2.3.1 describes the BS receiver blocking model. Subsection E.2.3.2 describes the impact on ACLR and ACS values due to the in-balance in channel bandwidth between aggressor and victim networks. This new subsection covers the agreements reached in RAN4#106bis (R4-2305923)

- Editorial changes:

  - Improve readability.

  - Correction of typos.

# 3 References

- RP-223041, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN #98-e, Electronic Meeting, December 12-16, 2022.

- R4-2317010, Draft TR 38.858, RAN4#108 bis, CMCC

<Start of TP to TR 38.858>

Annex <E>:
Adjacent channel co-existence evaluation

# E.1	RAN4 co-existence simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table E.1-1.

Table E.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | High |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Dense | Urban Dense | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |
| Note 1: Scenario 7 has been down selected. |  |  |  |  |

The Urban Hotspot reuses most parameter assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE as described in Annex E, in Table E.2.1-1 and Table E.2.1-2 and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact on SBFD network due to the legacy TDD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration.

Table E.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |  |

# E.2	RAN4 co-existence simulation assumptions

## E.2.1	Deployment

Simulation assumptions related to network layout is captured for FR1 in Table E.2.1-1 and for FR2-1 in Table E.2.1-2.

Simulation assumptions related to cluster-based UE dropping are captured in Table E.2.1-3.

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) | Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Single layer with 19 hexagonal cells with wrap around. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 500 m | 289 m | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | Baseline: 100% The victim network (UMi) in the center, the aggressor network is moved by the grid offset. 100% is relative to the micro ISD. 2nd priority: Other values | N/A |
| Carrier frequency | 4 GHz | 4 GHz | 4 GHz |
| Path-loss model | BS-to-UE: UMa see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803 (Note 2). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-UE: UMi see TR 38.803 BS-to-BS: for the UMa-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. For LoS probability for Macro-to-Micro case: Reuse the same model as in TR 38.828 with h_UT = 10 m for the UMi-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 (Note 2). | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m | 1.5 m |
| UE distribution | Uniform dropping Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | Uniformly UE distribution in the cell 20% indoor and 80% outdoor | 100% indoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model | 80% low-loss model and 20% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 5 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 6 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based UE dropping. See Table E.2.1-3 for cluster-based dropping. Note 2: For UE-UE penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D-in =Uniform(0,min⁡{25m,d2D2}) |  |  |  |

Table E.2.1-2: FR2-1 deployment parameters

| Parameter | Urban Macro (Note 1) | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Fixed cluster circle within a macro cell. Number of micro BSs per macro cell is 3. Radius of UE dropping within a micro cell is < 28.9 m. Minimum distance between micro BSs in different operator is 10 m. Shadowing correlation is 0.5. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 200 m | 10 m Minimum distance between micro BSs in different operator | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | N/A | N/A |
| Carrier frequency | 30 GHz | 30 GHz | 30 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-UE: UMi + penetration loss BS-to-BS: UMi UE-to-UE: UMi (h_BS=1.5m~22.5m) + penetration loss, see TR 38.803 (Note 2). UMi model is not applicable when 2D distance is less than 10 m, instead free space path loss model is applicable. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m ≦ hUT ≦ 22.5 m | 1.5 m |
| UE distribution | Indoor UE ratio: 0% Uniform UE distribution | Indoor UE ratio: 80 % Uniform UE distribution | 100% indoor |
| O-to-I penetration loss model | N/A | 50% low-loss model and 50% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 3 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 10 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes cluster-based dropping. See Table E.2.1-3 for cluster-based dropping. Note 2: For UE-UE penetration loss, the following is used to generate d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O-to-I building penetration loss: - d2D-in =Uniform(0,min⁡{25m,d2D2}) |  |  |  |

Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 (Note 1) |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor (Note 2) |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro BS – cluster center: 60 m (Note 3) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Cells fully or partly overlapping between the 2 networks share the same cluster. Note 2: O-to-I penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area. For pairs of indoor UEs in different cluster areas, a single penetration loss component is calculated. Note 3: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro BS to cluster centre should be respected also for BSs belonging to the other operator |  |

## E.2.2	Traffic

Assumptions related to network traffic load configuration for FR1 is captured in Table E.2.2-1 and for FR2-1 in Table E.2.2-2. In each scheduling interval, a single UE is served per link direction.

Table E.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro | Micro | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 100 MHz | 100 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

Table E.2.2-2: FR2-1 network traffic parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 200 MHz | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For {DU}: 40 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

## E.2.3	BS RF characteristics

The SBFD BS is defined for two different antenna configurations:

- SBFD configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

As baseline the array antenna model, for single element configuration, is described in TR 38.803, subclause 5.2.3. As an option the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

As an option, power boosting of 3 dB can be considered to compensate for fewer transmitter branches in the case of SBFD config 1.

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table E.2.3-1 and for FR2-1 in Table E.2.3-2.

Table E.2.3-1: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) | Micro (Medium Range) | Indoor (Local Area) |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1: TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 46 dBm SBFD: 46 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (q3dB,j3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) SBFD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ, (q3dB,j3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2) SBFD config 1: (Mg,Ng,M,N,P) =(1,1,2,4,2) SBFD config 2: (Mg,Ng,M,N,P) =(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ, (q3dB,j3dB)=(90,90) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi |
| Receiver noise figure model parameters | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 | P1=-38 dBm P2=-20 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-35 dBm P2=-17 dBm F1=13 dB F2=22 dB See Figure E.2.3-1 |
| ACLR | 45 dBc | 45 dBc | 45 dBc |
| ACS | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

Table E.2.3-2: FR2-1 RF BS parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | 1st priority: TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm 2nd priority: TDD: 40 dBm SBFD config 1: 37 dBm SBFD config 2: 40 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi Optional: (q3dB, j3dB)=(90,90) o, SLAv=30 dB, Am=30 dB, GE,max=5.5 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,4,8,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,2,8,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(90,90) o, SLAv=30 dB Am=30 dB, GE,max=3 dBi |
| Receiver noise figure parameters | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 |
| ACLR | 28 dBc | 28 dBc | 28 dBc |
| ACS | 24 dBc | 24 dBc | 24 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

### E.2.3.1 BS receiver blocking model

To capture receiver blocking aspects, the receiver noise figure (Fblock) will be characterised as a piece-wise linear function of input receiver power (Pin) as shown in Figure E.2.3-1.

Figure E.2.3-1: Receiver blocking model

Relevant values for F, F2, P1 and P2 for different BS classes are listed in Table E.2.3-1 and Table E.2.3-2. If the total received power is larger than P2 the receiver will be blocked, which correspond to zero throughput.

The input power to the blocking model is defined as average total power in dBm and the output is the noise figure in dB including noise rise due to receiver blocking aspects, such as e.g., receiver intermodulation.

The receiver input total power can be expressed as a sum of wanted signal and all interferer signals in linear scale as:

Pin=Pw+Iintra-SB, inter-UE+Iinter-SB,inter-gNB+IACI+Iself+Ico-site,inter-sector

, where the signal and interferer sources are defined as:

Pw is the wanted signal received from scheduled UE. The wanted signal level is calculated as:
Pw = PTX,UECLUE→serving gNB

Iintra-SB,inter-UE is the interference from UEs transmitting within the UL sub-band in the victim network. The interference level is calculated as:

Iintra-SB,inter-UE = PTX,UECLUE→serving BS

Iinter-SB,inter-BS is the interference from BS transmitting on the adjacent DL sub-band in the victim network. The interference level is calculated as:

Iinter-SB,inter-BS=PTX,BSCLown network→serving BS

IACI is the adjacent channel interference from BS transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTX,BSCLadjacent network gNB→serving BS

Iself is the victim self-interference power. The interference level is calculated as:

Iself=kTBSB FMself

Ico-site,inter-sector is the interference from neighboring sectors in a 3-sector configuration. The interference level is calculated as:

Ico-site,inter-sector=kTBSB FMco-site

, where

PTX,UE is the configured UE transmitter power (see transmission power control in Annex E.3.5).

PTX,BS is the BS transmitter power.

BSB is the allocated SBFD slot UL receiver bandwidth.

Mself is the noise-floor-to-self interference ratio.

Mco-site is the noise-floor-to-aggregated co-site inter sector interference ratio.

kT is the thermal noise (equal to -174 dBm/Hz in logarithmical scale).

F is the receiver noise figure.

Note that the receiver input total power does not include the ACLR and ACS effects.

The total interference power can be expressed in linear scale as:

Itot=Iintra-SB,inter-UE+Iinter-SB,inter-BSASBIRinter-SB,inter-BS+IACIACIR+Iself+Ico-site,inter-sector

, where additional variables are defined as:

ASBIRinter-SB,inter-BS is the in-channel Adjacent Sub-Band Interference Ratio, which is derived by applicable BS ACLR and BS ACS.

ACIR is derived by applicable BS ACLR and BS ACS.

The SINR at the victim receiver including total noise and total interference can be expressed in linear scale as:

SINR=PwItot+kTBSBFblock

The receiver noise figure for the receiver exposed for blocking Fblock is produced by the blocking model described in Figure E.2.3-1. The receiver blocking model is applicable to both legacy TDD and SBFD gNBs.

In simulation, the UE power control scheme is only used to compensate path loss and doesn’t consider noise figure increase due to BS receiver blocking. Therefore, the final SINR for UL is less than target SINR.

Note that additional, power scaling of all interference contributions should be considered with respect to aggressor bandwidth and victim bandwidth when ACIR is applied.

### E.2.3.2 BS ACLR and ACS bandwidth dependency

In RAN4, ACLR and ACS requirements are defined assuming equal bandwidth for the aggressor’s and victim’s channel bandwidth. This means that the power in the adjacent channel can be easily associated to the wanted carrier power using ACLR and similar for ACS when receiver performance is evaluated. However, in this adjacent channel co-existence study, the bandwidths of aggressor and victim’s networks are different. To compensate for the difference in bandwidth, the following methodology is adopted:

- For cases where the aggressor’s bandwidth is narrower that the victim’s bandwidth, e.g., in Case 2 in Table E.1-2:

  - Both the ACLR and ACS are kept to the values defined in Table E.2.3-1 and Table E.2.3-2.

- For cases where the aggressor’s bandwidth is wider than the victim’s bandwidth, i.e., in Case 3 in Table E.1-2:

  - A bandwidth compensation factor is applied to the ACLR and ACS values defined in Table E.2.3-1 and Table E.2.3-2. The factor is calculated as:

BWfactor=10log10BWaggressorBWvictim,

For instance, for a TDD DL 100 MHz BW aggressor network interfering a SBFD UL 20 MHz BW victim, the ACLR is equal to: 45dBc + 10log10(100/20) = 52 dBc

## E.2.4	UE RF characteristics

Assumptions relevant for modelling the UE RF characteristics are captured in Table E.2.4-1.

Table E.2.4-1: UE RF parameters

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | 23 dBm | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |
| Antenna configuration | 0 dBi | (Mg,Ng,M,N,P) = (1,1,2,2,2) (dH,dV)=(0.5,0.5)λ (q3dB, j3dB)=(90,90) o, SLAv=25 dB Am=25 dB, GE,max=5.5 dBi |
| Receiver noise figure | 9 dB | 10 dB |
| ACLR | 30 dBc The ACLR is modelled as 30 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB. | 24 dBc The ACLR is modelled as 24 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB. |
| ACS | 33 dBc | 23 dBc |

# E.3	RAN4 co-existence simulation methodology

## E.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNO ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the inter-cell interference.

- Throughput is computed considering ACI as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation caused by ACI as:

LossACI=1-ThputACI/ThputSINGLE

The simulation results should be in the form of the throughput with adjacent network  and the relative difference between the two can be compared at 50% and 5% points.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

| S(SNIR) | Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz |
| --- | --- |
|  | Attenuation factor, representing implementation losses |
| SNIRMIN | Minimum SNIR of the code set, dB |
| SNIRMAX | Maximum SNIR of the code set, dB |

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table E.3.1-1.

Table E.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| a, attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

## E.3.2	Received signal power model

The following model is applied:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

, where

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

## E.3.3	Network grid shift

In cases with grid shift >0%, the victim network base stations should be placed as shown in Figure E.3.3-1. The victim network grid is shifted along the line between BS and its closest 100% grid-shift BS, where the distance from any BS in victim network to its second and third closest BS in the aggressor network is the same.

Figure E.3.3-1: Network grid shift definition

## E.3.4	Coupling-loss

The Coupling Loss (CL) is defined as the loss in signal between BS-to-UE, UE-to-UE and BS-to-BS. CL is defined as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors.

## E.3.5	Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2-1 is 15 dB.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target value according to power control scheme in TS 38.213.

<End of TP to TR 38.858>