3GPP TSG-RAN4 Meeting #109	       R4-2320649 Chicago, USA, 13 – 17 November 2023

Source: 	Qualcomm Inc.

Title: 	TP to TR 38.741: A-MPR for LS-band

Agenda Item:	7.31.2

Document for:	Approval

# Introduction

In this contribution A-MPR simulation results are provided to quantify required A-MPR to meet additional emission requirements in LS-band. The results are included in a text proposal to TR 38.741 to capture the A-MPR simulation data as background for the final A-MPR values.

# Discussion

A number of additional emission requirements have been considered for LS-band, and these additional requirements may results in need for A-MPR to meet the emission limits. A simulation study was done to evaluate the magnitude of A-MPR. The simulations assumptions were:

- PA calibration point: ACLR of 30 dBc is met with DFT-s-OFDM waveform with StartRB = 0, L_CRB = 100

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, Additional out of band bands emissions from ETSI EN 301 441 (see table 1 and table 2)

Table 1: Additional out of band emission requirements considered in the simulations

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 2: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers

| Frequency offset (kHz) (Note 1) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) (Note 2) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from edge of nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. frequency offset |  |  |  |

The simulation results based on these assumptions have been captured in the text proposal to avoid duplication of multiple figures inside the Tdoc. Following proposals are made

Proposal 1: Capture the A-MPR simulations results in the technical report 38.741

Proposal 2: Take the simulation results into account when defining the A-MPR specification.

# Conclusions

In this contribution A-MPR simulation study for LS-band was presented. Following proposals are made

Proposal 1: Capture the A-MPR simulations results in the technical report 38.741

Proposal 2: Take the simulation results into account when defining the A-MPR specification.

# References

# Text proposal to TR 38.741 v.0.1.0

###### 6.2.1.2.2.2	Power back-off results for ETSI

###### 6.2.1.2.2.2.1	Company 1 results

6.2.1.2.2.2.1.1	5MHz channel

Figure 6.2.1.2.2.2.1.1-1: Power back-off values for the 5MHz channel at Fc=1612.5MHz.

Figure 6.2.1.2.2.2.1.1-2: Power back-off values for the 5MHz channel at Fc=1615.5MHz.

Figure 6.2.1.2.2.2.1.1-3: Power back-off values for the 5MHz channel at Fc=1620.5MHz.

Figure 6.2.1.2.2.2.1.1-4: Power back-off values for the 5MHz channel at Fc=1624MHz.

6.2.1.2.2.2.1.2	10MHz channel

Figure 6.2.1.2.2.2.1.2-1: Power back-off values for the 10MHz channel at Fc=1615MHz.

Figure 6.2.1.2.2.2.1.2-2: Power back-off values for the 10MHz channel at Fc=1621.5MHz.

6.2.1.2.2.2.1.3	15MHz channel

Figure 6.2.1.2.2.2.1.3-1: Power back-off values for the 15MHz channel at Fc=1617.5MHz.

Figure 6.2.1.2.2.2.1.3-2: Power back-off values for the 15MHz channel at Fc=1619MHz.

###### 6.2.1.2.2.2.2	Company 2 results

A simulation study was done to evaluate the magnitude of A-MPR to meet additional emission requirements in LS-band. The simulations assumptions were:

- PA calibration point: ACLR of 30 dBc is met with DFT-s-OFDM waveform with StartRB = 0, L_CRB = 100

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, Additional out of band bands emissions from ETSI EN 301 441 (see Table 6.2.1.2.2.2.2-1 and Table 6.2.1.2.2.2.2-2)

The results consider only PA nonlinearity with the above impairments, while other transceiver noise sources have not been taken into account. Further increase in backoff may be needed due to other emission sources.

Table 6.2.1.2.2.2.2-1: Additional out of band emission requirements considered in the simulations

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.2.1.2.2.2.2-2: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers

| Frequency offset (kHz) (Note 1) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) (Note 2) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from edge of nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. frequency offset |  |  |  |

6.2.1.2.2.2.2.1	5MHz channel

Figure 6.2.1.2.2.2.3.1-1: Power back-off values for the 5MHz channel at Fc=1612.5MHz.

Figure 6.2.1.2.2.2.3.1-2: Power back-off values for the 5MHz channel at Fc=1615.5MHz.

Figure 6.2.1.2.2.2.3.1-3: Power back-off values for the 5MHz channel at Fc=1620.5MHz.

Figure 6.2.1.2.2.2.3.1-4: Power back-off values for the 5MHz channel at Fc=1624MHz.

6.2.1.2.2.2.2.2	10MHz channel

Figure 6.2.1.2.2.2.2.2-1: Power back-off values for the 5MHz channel at Fc=1615MHz.

Figure 6.2.1.2.2.2.2.2-2: Power back-off values for the 5MHz channel at Fc=1621.5MHz.

6.2.1.2.2.2.2.3	15MHz channel

Figure 6.2.1.2.2.2.2.3-1: Power back-off values for the 5MHz channel at Fc=1617.5MHz.

Figure 6.2.1.2.2.2.2.3-2: Power back-off values for the 5MHz channel at Fc=1619MHz.

###### 6.2.1.2.2.3		Power back-off results for FCC