3GPP TSG-RAN4 Meeting #109	        R4-2320652 Chicago, USA, 13 – 17 November 2023

Source: 	Qualcomm Inc.

Title: 	PC2 A-MPR for bands n7, n13 and n28

Agenda Item:	7.22.2

Document for:	Approval

# Introduction

In this contribution A-MPR with PC2 is discussed and proposals are made for bands n7, n13 and n28.

# Discussion

## A-MPR for n7 PC2

Operating band n7 is subject to additional emission requirement captured under NS_46. This requirement is re-produced below from TS 38.101-1

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

The corresponding A-MPR has been specified for channel bandwidths of 25 MHz and wider. For 15 and 20 MHz channel bandwidths an RB restriction to 54RB is applicable. A-MPR for PC3 is reproduced below

Table 6.2.3.17-1: A-MPR regions for NS_46

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.17-2: A-MPR for NS_46

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

A-MPR was evaluated for PC2 using simulations and results for some individual simulated RB allocations were also validated by measurements. Simulation assumptions were as follows:

- PA calibration point: ACLR of 31 dBc is met with DFT-s-OFDM StartRB = 0, L_CRB = 100 waveform

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, NS_46 additional requirements.

Based on the simulations the A-MPR requirements in Table 1 and Table 2 are proposed for n7 PC2 to meet NS_46 requirements.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2550 ≤ FC ≤ 2565 | ≥ 7.92 | > 0 | A9 |
|  |  | ≥ 7.92 | < 5.4 | A10 |
| 15 MHz | 2545 ≤ FC ≤ 2562.5 | ≥ 11.16 | > 0 | A9 |
|  |  | ≥ 11.16 | < 5.4 | A10 |
| 20 MHz | 2540 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | A9 |
|  |  | ≥ 14.4 | < 5.4 | A10 |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

Proposal 1: Adopt the A-MPR regions and A-MPR values in Table 1 and Table 2 for NS_46 for PC2 operation

One additional detail in n7 operation is that PC3 does not have any A-MPR for 10-20 MHz channel bandwidths, but instead 15 and 20 MHz channel bandwidths are restricted to 54RB transmission bandwidth. It is possible that this results in a case that PC2 UE needs to fallback to PC3 while it is transmitting more than 54RB. The UE behaviour in such case is currently unclear. If the UE keeps transmitting with full output power of 23 dBm, it risks violating the emission limit. On the other hand it is detrimental for network performance and user experience if the UE stops transmitting.

Therefore, it would be beneficial to allow UE to keep transmitting, but at the same time allow UE to reduce its output power to ensure that emission limits are met. Based on our evaluation, UE should be allowed to reduce its output power to 23 dBm – max(MPR,4) dB in case it falls back to PC3 operation and is transmitting more than 54 RB. This should be considered at least for PC2 capable UEs, as changing the PC3 A-MPR needs further consideration.

Proposal 2: Allow PC2 capable UEs to reduce their output power 23 dBm – max(MPR,4) dB in case PC2 needs to fall back to PC3 operation and transmit more than 54 RB. Apply this for both SC-FDMA and CP-OFDM and all modulation orders.

## A-MPR for n28 PC2

As agreed in RAN4#108bis, we have analyzed A-MPR for n28 assuming full band duplexer. In the analysis we have assumed filter rejection to be

- 0 dB towards NS_18 requirements

- 0 dB towards NS_17 requirements

- 15 dB at 694 MHz towards UE-to-UE coex requirements

UE-to-UE coex requirements above the Tx frequencies do not require additional analysis as the frequencies overlap n28 Rx frequencies where the full band duplexer provides significant rejection.

In the simulation campaign following

- PA calibration point: ACLR of 31 dBc is met with DFT-s-OFDM StartRB = 0, L_CRB = 100 waveform

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, SEM, Spurious emissions, EVM, NS_17 and NS_18 additional requirements, general co-existence requirements as highlighted in the below table

| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76, NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, 73 NR Band n79, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

Furthermore, going through the requirements we noticed that the current specification does not include 25 MHz channel bandwidth to the NS_18 additional requirement table. This is an error as A-MPR was analysed and specified for NS_18 with 25 MHz channel bandwidth for PC3. We have therefore also analysed 25 MHz channel bandwidth

Observation 1: Current specification has an error and NS_18 emission requirement does not capture 25 MHz bandwidth.

Based on the simulations for NS_17, 3 and 5 MHz channel bandwidths do not require A-MPR. A-MPR is proposed for NS_17 for 10 MHz channel bandwidth as shown in tables 3 and 4. The evaluations consider only APT operating mode and further work is needed to verify the values also for ET.

Table 3: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

Table 4: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [4] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [4] |
|  | 64 QAM | ≤ [4] | ≤ [4.5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

Proposal 3: Take A-MPR values in Tables 3 and 4 as baseline for NS_17 for n28 PC2, verify the values further to cover all operating modes.

For NS_18, there a baseline A-MPR was already suggested in [3]. We are suggesting minor changes to the baseline is shown in tables 5 and 6. Changes are with red font. It was also observed that 3 MHz channel bandwidth would not need A-MPR for NS_18, though A1 A-MPR is specified for 3 MHz for PC3. As specified in TS 38.101-1 Table 6.2.3.1-1, 3 and 5 MHz channel bandwidths are allowed A1 A-MPR, 10 to 20 MHz channel bandwidths are allowed A2 A-MPR, and finally for 25 and 30 MHz currently regions A3, A4 and A5 are defined.

Similarly as for NS_17, the evaluations done so far consider only APT operation and further work is needed to verify them for other operating modes.

Table 5: A-MPR regions for NS_18 for PC2

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 6: A-MPR for NS_18 for PC2

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [3.5] | N/A | ≤ [7] | ≤ [3.5] | ≤ [9] | ≤ [4] | ≤ [2] |
|  | QPSK | ≤ [3.5] |  | ≤ [7] | ≤ [3.5] | ≤ [9] | ≤ [4] | ≤ [2] |
|  | 16 QAM | ≤ [4] |  | ≤ [8] | ≤ [3.5] | ≤ [9] | ≤ [5] | ≤ [2] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [9] | ≤ [3.5] | ≤ [9] | ≤ [5.5] | ≤ [2.5] |
|  | 256 QAM | ≤ [6.5] |  | ≤ [10] | ≤ [3.5] | ≤ [9] | ≤ [5.5] |  |
| CP-OFDM | QPSK | ≤ [5.5] |  | ≤ [9.5] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 16 QAM | ≤ [5.5] |  | ≤ [10] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 64 QAM | ≤ [6] |  | ≤ [10] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 256 QAM | ≤ [9] |  | ≤ [11.5] | ≤ [5] | ≤ [10.5] | ≤ [7.5] |  |

Proposal 4: Take A-MPR values in Tables 5 and 6 as baseline for NS_17 for n28 PC2, verify the values further to cover all operating modes.

Fulfilling general UE-to-UE coex requirements was also evaluated. It was observed that especially the -42 dBm with 8 MHz MBW reaching up to 694 MHz is sensitive on assumed filter attenuation. Further evaluations are still needed to see whether this requirement can be met without A-MPR.

If necessary, RAN4 can specify a new NS for this case, while keeping the general requirements as is for PC3 operation.

Observation 2: If necessary, a new NS can be added for -42 dBm/8MHz UE-to-UE coex requirement

## A-MPR for n13 PC2

A-MPR was evaluated for PC2 using simulations and results for some individual simulated RB allocations were also validated by measurements. Simulation assumptions were as follows:

- PA calibration point: ACLR of 31 dBc is met with DFT-s-OFDM StartRB = 0, L_CRB = 100 waveform

- IQ-image: -28 dBc

- LO-leakage: -28 dBc

- CIM3: -60 dBc

- Requirements considered: NR ACLR, Spurious emissions, EVM, NS_07 additional SEM and Spurious requirements requirements.

Based on the simulations the A-MPR requirements in Table 7 and Table 8 are proposed for n13 PC2 to meet NS_07 requirements.

Table 7: A-MPR regions for NS_07 for PC2

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 8: A-MPR for NS_07 for PC2

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 8 | 6 |
| DFT-s-OFDM QPSK | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 8 | 6 |
| CP-OFDM QPSK | 14 | 10 | 9 | 6 |
| CP-OFDM 16 QAM | 14 | 10 | 9 | 6 |
| CP-OFDM 64 QAM | 14 | 10 | 9 | 6 |
| CP-OFDM 256 QAM | 14 | 10 | 9 | 6 |

Proposal 5: Adopt the A-MPR regions and A-MPR values in Table 7 and Table 8 for NS_07 for PC2 operation

# Conclusions

In this contribution A-MPR with PC2 was discussed and following proposals were made for bands n7, n13 and n28.

Proposal 1: Adopt the A-MPR regions and A-MPR values in Table 1 and Table 2 for NS_46 for PC2 operation

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2550 ≤ FC ≤ 2565 | ≥ 7.92 | > 0 | A9 |
|  |  | ≥ 7.92 | < 5.4 | A10 |
| 15 MHz | 2545 ≤ FC ≤ 2562.5 | ≥ 11.16 | > 0 | A9 |
|  |  | ≥ 11.16 | < 5.4 | A10 |
| 20 MHz | 2540 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | A9 |
|  |  | ≥ 14.4 | < 5.4 | A10 |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

Proposal 2: Allow PC2 capable UEs to reduce their output power 23 dBm – max(MPR,4) dB in case PC2 needs to fall back to PC3 operation and transmit more than 54 RB. Apply this for both SC-FDMA and CP-OFDM and all modulation orders.

Observation 1: Current specification has an error and NS_18 emission requirement does not capture 25 MHz bandwidth.

Proposal 3: Take A-MPR values in Tables 3 and 4 as baseline for NS_17 for n28 PC2, verify the values further to cover all operating modes.

Table 3: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

Table 4: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [4] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [4] |
|  | 64 QAM | ≤ [4] | ≤ [4.5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

Proposal 4: Take A-MPR values in Tables 5 and 6 as baseline for NS_17 for n28 PC2, verify the values further to cover all operating modes.

Table 5: A-MPR regions for NS_18 for PC2

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

Table 6: A-MPR for NS_18 for PC2

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [3.5] | N/A | ≤ [7] | ≤ [3.5] | ≤ [9] | ≤ [4] | ≤ [2] |
|  | QPSK | ≤ [3.5] |  | ≤ [7] | ≤ [3.5] | ≤ [9] | ≤ [4] | ≤ [2] |
|  | 16 QAM | ≤ [4] |  | ≤ [8] | ≤ [3.5] | ≤ [9] | ≤ [5] | ≤ [2] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [9] | ≤ [3.5] | ≤ [9] | ≤ [5.5] | ≤ [2.5] |
|  | 256 QAM | ≤ [6.5] |  | ≤ [10] | ≤ [3.5] | ≤ [9] | ≤ [5.5] |  |
| CP-OFDM | QPSK | ≤ [5.5] |  | ≤ [9.5] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 16 QAM | ≤ [5.5] |  | ≤ [10] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 64 QAM | ≤ [6] |  | ≤ [10] | ≤ [5] | ≤ [10.5] | ≤ [6] | ≤ [4] |
|  | 256 QAM | ≤ [9] |  | ≤ [11.5] | ≤ [5] | ≤ [10.5] | ≤ [7.5] |  |

Observation 2: If necessary, a new NS can be added for -42 dBm/8MHz UE-to-UE coex requirement

Proposal 5: Adopt the A-MPR regions and A-MPR values in Table 7 and Table 8 for NS_07 for PC2 operation

Table 7: A-MPR regions for NS_07 for PC2

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 8: A-MPR for NS_07 for PC2

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 8 | 6 |
| DFT-s-OFDM QPSK | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 8 | 6 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 8 | 6 |
| CP-OFDM QPSK | 14 | 10 | 9 | 6 |
| CP-OFDM 16 QAM | 14 | 10 | 9 | 6 |
| CP-OFDM 64 QAM | 14 | 10 | 9 | 6 |
| CP-OFDM 256 QAM | 14 | 10 | 9 | 6 |

# References

[1]	R4-2313548, PC2 A-MPR and RSD for bands n7 and n70, Qualcomm Inc.

[2]	R4-2317586, WF on HPUE for FDD bands, Qualcomm Inc.

[3]	R4-2316477, DraftCR for Adding PC2 requirements for band n8 and n28, Huawei, HiSilicon, China Unicom