3GPP TSG RAN WG4 Meeting #109	R4-2320675

Chicago, USA, November 13th – 17th, 2023

Title:	[Draft] Reply LS on power class indication in lower MSD capability

Response to:	R2-2311586 LS on power class indication in lower MSD capability

Release:	Rel-18

Work Item:	NR_ENDC_RF_FR1_enh2

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name: Jin Wang

E-mail Address: jinwang@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 respectfully thanks RAN2 for the LS on power class indication in lower MSD capability in R2-2311586.

RAN4 discussed the following questions raised in the LS, and have the following answers based on the group’s common understanding on this topic:

Question 1)

It is not completely clear to RAN2 whether the power class that is supposed to signalled in the new MSD capability signalling is the power class of aggressor band(s) and/or victim band.

Answer 1: RAN4 confirms that the power class used in the new MSD capability signalling is the power class of the band combination consisting of the victim band and the aggressor band(s) with 1 or 2 bands in the UL. The aggressor can be the one UL band for MSD type of harmonic, harmonic mixing and cross-band isolation, or the 2 UL bands for MSD type of IMD for lower-MSD reporting in Rel-18.

Question 2)

RAN2 would like to point out that under the current UE capability signalling, the UE reports a power class per frequency band, per band combination and per band per band combination respectively (see the table below).

| UE capability parameter | Applicability |
| --- | --- |
| ue-PowerClass ue-PowerClass-v1610 ue-PowerClass-v1700 | per frequency band |
| powerClass powerClass-v1610 | Per band combination |
| ue-PowerClassPerBandPerBC-r17 | Per band per band combination |

It was not clear to RAN2 which of the above power class types is relevant in the MSD capability signalling, and whether the choice of power class type can be dependent on the MSD type (e.g. whether the aggressor is a single band or two bands).

Answer 2: The power class used in the new MSD capability signaling shall be determined by the power class of the underlying 2/3-band combination. More explicitly,

- For MSD type of harmonic, harmonic mixing and cross-band isolation, the power class is that for the band combination with the following DL/UL component(s):

  - DL_bandA(victim)-bandB, UL_bandB(aggressor).

- For MSD type of IMD, the power class is that for the band combination with the following DL/UL components:

  - DL_bandA(victim)-bandB, UL_bandA(aggressor)-bandB(aggressor) or

  - DL_bandA(victim)-bandB-bandC, UL_bandB(aggressor)-bandC(aggressor).

Such band combinations may not be signaled via BandCombination IE by the UE. To avoid potential ambiguity, a UE shall include the necessary power class information in the lower-MSD capability reporting, which is in line with the essential information for lower-MSD capability as previously communicated to RAN2.

Question 3)

RAN2 would also like to point out that under the current UE capability signalling, the UE reports only a single power class per frequency band, per band combination and per band per band combination respectively. RAN2 therefore needs a clarification from RAN4 regarding the RAN4 text, what the “highest supported power class” and “other power classes” refer to.

Answer 3: RAN4 clarifies that both “highest supported power class” and “other power classes” are the power classes which a UE can support for the band combinations as explained in answer#2. Relative to the “highest supported power class”, “other power classes” are intended for the lower power class(es) that the UE may fall back to. For example, if a UE supports PC2 for a band combination, “other power classes” means PC3.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take the above RAN4 responses into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #110	26 Feb – 01 Mar, 2024		Athens, Greece

TSG-RAN WG4 Meeting #110-bis	15 Apr – 19 Apr, 2024		TBC, China