| 3GPP TR 38.899 V0.7.0 (2023-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	13

2	References	13

3	Definitions of terms, symbols and abbreviations	14

3.1	Terms	14

3.2	Symbols	14

3.3	Abbreviations	14

4	Background	14

4.1	TR Maintenance	15

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL	15

5.x	CA_nX-nY	15

5.x.1	Configurations	15

5.x.2	Maximum output power	15

5.x.3	REFSENS requirements	15

5.x.4	∆TIB and ∆RIB values	16

5.1	CA_n25-n41C	16

5.1.1	Configurations	16

5.1.2	Maximum output power	16

5.1.3	REFSENS requirements	16

5.1.4	∆TIB and ∆RIB values	17

5.2	CA_n41C-n66	17

5.2.1	Configurations	17

5.2.2	Maximum output power	17

5.2.3	REFSENS requirements	18

5.2.4	∆TIB and ∆RIB values	18

5.3	CA_n41C-n71	18

5.3.1	Configurations	18

5.3.2	Maximum output power	19

5.3.3	REFSENS requirements	19

5.3.4	∆TIB and ∆RIB values	19

5.4	CA_n41C-n77A	19

5.4.1	Configurations	19

5.4.2	Maximum output power	20

5.4.3	REFSENS requirements	20

5.4.4	∆TIB and ∆RIB values	20

5.5	CA_n77-n79	21

5.5.1	Configurations	21

5.5.2	Maximum output power	21

5.5.3	REFSENS requirements	21

5.5.4	∆TIB and ∆RIB values	22

5.6	CA_n78-n79	22

5.6.1	Configurations	22

5.6.2	Maximum output power	23

5.6.3	REFSENS requirements	23

5.6.4	∆TIB and ∆RIB values	25

5.7	CA_n3-n41	25

5.7.1	Configurations	25

5.7.2	Maximum output power	25

5.7.3	REFSENS requirements	25

5.7.4	∆TIB and ∆RIB values	26

5.8	CA_n3-n77	26

5.8.1	Configurations	26

5.8.2	Maximum output power	26

5.8.3	REFSENS requirements	26

5.8.4	∆TIB and ∆RIB values	28

5.9	CA_n8-n78	28

5.9.1	Configurations	28

5.9.2	Maximum output power	28

5.9.3	REFSENS requirements	29

5.9.4	∆TIB and ∆RIB values	30

5.10	CA_n40-n77	30

5.10.1	Configurations	30

5.10.2	Maximum output power	30

5.10.3	REFSENS requirements	30

5.10.4	∆TIB and ∆RIB values	31

5.11	CA_n28-n77	31

5.11.1	Configurations	31

5.11.2	Maximum output power	32

5.11.3	REFSENS requirements	32

5.11.4	∆TIB and ∆RIB values	33

5.12	CA_n1-n41	33

5.12.1	Configurations	33

5.12.2	Maximum output power	34

5.12.3	REFSENS requirements	34

5.12.4	∆TIB and ∆RIB values	34

5.13	CA_n1-n77	35

5.13.1	Configurations	35

5.13.2	Maximum output power	35

5.13.3	REFSENS requirements	35

5.14	CA_n25-n78	37

5.14.1	Configurations	37

5.14.2	Maximum output power	37

5.14.3	REFSENS requirements	37

5.14.4	∆TIB and ∆RIB values	38

5.15	CA_n66-n78	39

5.15.1	Configurations	39

5.15.2	Maximum output power	39

5.15.3	REFSENS requirements	39

5.15.4	∆TIB and ∆RIB values	40

5.16	CA_n7-n77	41

5.16.1	Configurations	41

5.16.2	Maximum output power	41

5.16.3	REFSENS requirements	41

5.16.4	∆TIB and ∆RIB values	43

5.17	CA_n1-n79	43

5.17.1	Configurations	43

5.17.2	Maximum output power	44

5.17.3	REFSENS requirements	44

5.17.4	∆TIB and ∆RIB values	44

5.18	CA_n8-n79	45

5.18.1	Configurations	45

5.18.2	Maximum output power	45

5.18.3	REFSENS requirements	45

5.18.4	∆TIB and ∆RIB values	46

5.19	CA_n13-n77	47

5.19.1	Configurations	47

5.19.2	Maximum output power	47

5.19.3	REFSENS requirements	47

5.19.4	∆TIB and ∆RIB values	47

5.20	CA_n5-n77	48

5.20.1	Configurations	48

5.20.2	Maximum output power	48

5.20.3	REFSENS requirements	48

5.21	CA_n66-n77	49

5.21.1	Configurations	49

5.21.2	Maximum output power	49

5.21.3	REFSENS requirements	50

5.22	CA_n25-n77	50

5.22.1	Configurations	50

5.22.2	Maximum output power	51

5.22.3	REFSENS requirements	51

5.23	CA_n71-n78	52

5.23.1	Configurations	52

5.23.2	Maximum output power	52

5.23.3	REFSENS requirements	52

5.23.4	∆TIB and ∆RIB values	53

5.24	CA_n5-n78	53

5.24.1	Configurations	53

5.24.2	Maximum output power	53

5.24.3	REFSENS reqsuirements	53

5.25	DL CA_n1-n78, UL n78A	54

5.25.1	Configurations	54

5.25.2	Maximum output power	54

5.25.3	REFSENS requirements	55

5.25.4	∆TIB and ∆RIB values	55

5.26	DL CA_n3-n78, UL n78A	55

5.26.1	Configurations	55

5.26.2	Maximum output power	56

5.26.3	REFSENS requirements	56

5.26.4	∆TIB and ∆RIB values	56

5.27	DL CA_n28-n78, UL n78A	57

5.27.1	Configurations	57

5.27.2	Maximum output power	57

5.27.3	REFSENS requirements	57

5.27.4	∆TIB and ∆RIB values	58

5.28	CA_n8-n77	58

5.28.1	Configurations	58

5.28.2	Maximum output power	58

5.9.3	REFSENS requirements	58

5.28.4	∆TIB and ∆RIB values	59

5.29	CA_n3-n79	60

5.29.1	Configurations	60

5.29.2	Maximum output power	60

5.29.3	REFSENS requirements	60

5.29.4	∆TIB and ∆RIB values	60

5.30	CA_n28-n79	60

5.30.1	Configurations	60

5.30.2	Maximum output power	61

5.30.3	REFSENS requirements	61

5.30.4	∆TIB and ∆RIB values	61

5.31	CA_n41-n79	62

5.31.1	Configurations	62

5.31.2	Maximum output power	62

5.31.3	REFSENS requirements	62

5.31.4	∆TIB and ∆RIB values	63

5.32	CA_n18-n41	63

5.32.1	Configurations	63

5.32.2	Maximum output power	63

5.32.3	REFSENS requirements	63

5.32.4	∆TIB and ∆RIB values	64

5.33	CA_n18-n77	64

5.33.1	Configurations	64

5.33.2	Maximum output power	65

5.33.3	REFSENS requirements	65

6	High Power UE CA of 3 bands DL and 1 or 2 bands UL	65

6.x	CA_nX-nY-nZ	65

6.x.1	Configurations	65

6.x.2	Maximum output power	66

6.x.3	REFSENS requirements	66

6.x.4	∆TIB and ∆RIB values	66

6.1 	DL CA_n25-n41-n66, UL CA_n25A-n41A	66

6.1.1	Configurations	66

6.1.2	Maximum output power	67

6.1.3	REFSENS requirements	67

6.1.4	Void	67

6.2	DL CA_n25-n41-n66, UL CA_n41A-n66A	67

6.2.1	Configurations	67

6.2.2	Maximum output power	68

6.2.3	REFSENS requirements	68

6.2.4	Void	69

6.3	DL CA_n25-n41-n71, UL CA_n25A-n41A	70

6.3.1	Configurations	70

6.3.2	Maximum output power	70

6.3.3	REFSENS requirements	70

6.3.4	Void	70

6.4	DL CA_n25-n41-n71, UL CA_n41A-n71A	71

6.4.1	Configurations	71

6.4.2	Maximum output power	71

6.4.3	REFSENS requirements	71

6.4.4	Void	72

6.5	DL CA_n25-n41-n71, UL CA_n41C	72

6.5.1	Configurations	72

6.5.2	Maximum output power	72

6.5.3	REFSENS requirements	72

6.5.4	Void	72

6.6	DL CA_n25-n41-n77, UL CA_n25A-n41A	72

6.6.1	Configurations	72

6.6.2	Maximum output power	73

6.6.3	REFSENS requirements	73

6.6.4	∆TIB and ∆RIB values	74

6.7	DL CA_n25-n41-n77, UL CA_n25A-n77A	74

6.7.1	Configurations	74

6.7.2	Maximum output power	75

6.7.3	REFSENS requirements	75

6.7.4	∆TIB and ∆RIB values	76

6.8	DL CA_n25-n41-n77, UL CA_n41A-n77A	76

6.8.1	Configurations	76

6.8.2	Maximum output power	76

6.8.3	REFSENS requirements	76

6.8.4	∆TIB and ∆RIB values	77

6.9	DL CA_n25-n66-n77, UL CA_n25A-n77A	78

6.9.1	Configurations	78

6.9.2	Maximum output power	78

6.9.3	REFSENS requirements	78

6.9.4	∆TIB and ∆RIB values	79

6.10	DL CA_n25-n66-n77, UL CA_n66A-n77A	79

6.10.1	Configurations	79

6.10.2	Maximum output power	80

6.10.3	REFSENS requirements	80

6.10.4	∆TIB and ∆RIB values	81

6.11	DL CA_n25-n71-n77, UL CA_n25A-n77A	81

6.11.1	Configurations	81

6.11.2	Maximum output power	82

6.11.3	REFSENS requirements	82

6.11.4	∆TIB and ∆RIB values	82

6.12	DL CA_n25-n71-n77, UL CA_n71A-n77A	82

6.12.1	Configurations	82

6.12.2	Maximum output power	82

6.12.3	REFSENS requirements	83

6.12.4	∆TIB and ∆RIB values	84

6.13	DL CA_n41-n66-n77, UL CA_n41A-n66A	84

6.13.1	Configurations	84

6.13.2	Maximum output power	84

6.13.3	REFSENS requirements	84

6.13.4	∆TIB and ∆RIB values	85

6.14	DL CA_n41-n66-n77, UL CA_n41A-n77A	85

6.14.1	Configurations	85

6.14.2	Maximum output power	86

6.14.3	REFSENS requirements	86

6.14.4	∆TIB and ∆RIB values	87

6.15	DL CA_n41-n66-n77, UL CA_n66A-n77A	87

6.15.1	Configurations	87

6.15.2	Maximum output power	87

6.15.3	REFSENS requirements	88

6.15.4	∆TIB and ∆RIB values	88

6.16	DL CA_n41-n71-n77, UL CA_n41A-n71A	89

6.16.1	Configurations	89

6.16.2	Maximum output power	89

6.16.3	REFSENS requirements	89

6.16.4	∆TIB and ∆RIB values	91

6.17	DL CA_n41-n71-n77, UL CA_n41A-n77A	91

6.17.1	Configurations	91

6.17.2	Maximum output power	91

6.17.3	REFSENS requirements	92

6.17.4	∆TIB and ∆RIB values	93

6.18	DL CA_n41-n71-n77, UL CA_n71A-n77A	93

6.18.1	Configurations	93

6.18.2	Maximum output power	93

6.18.3	REFSENS requirements	93

6.18.4	∆TIB and ∆RIB values	94

6.19	DL CA_n66-n71-n77, UL CA_n66A-n77A	95

6.19.1	Configurations	95

6.19.2	Maximum output power	95

6.19.3	REFSENS requirements	95

6.19.4	∆TIB and ∆RIB values	96

6.20	DL CA_n66-n71-n77, UL CA_n71A-n77A	96

6.20.1	Configurations	96

6.20.2	Maximum output power	97

6.20.3	REFSENS requirements	97

6.20.4	∆TIB and ∆RIB values	98

6.21	DL CA_n41-n66-n71, UL CA_n41A-n71A	98

6.21.1	Configurations	98

6.21.2	Maximum output power	98

6.21.3	REFSENS requirements	99

6.21.4	Void	99

6.22	DL CA_n41-n66-n71, UL CA_n41A-n71A	99

6.22.1	Configurations	99

6.22.2	Maximum output power	99

6.22.3	REFSENS requirements	100

6.22.4	Void	100

6.23	CA_n3-n28-n41	100

6.23.1	Configurations	100

6.23.2	Maximum output power	100

6.23.3	REFSENS requirements	100

6.24	CA_n3-n41-n77	101

6.24.1	Configurations	101

6.24.2	Maximum output power	101

6.24.3	REFSENS requirements	101

6.24.4	∆TIB and ∆RIB values	102

6.25	CA_n3-n28-n77	102

6.25.1	Configurations	102

6.25.2	Maximum output power	103

6.25.3	REFSENS requirements	103

6.25.4	∆TIB and ∆RIB values	103

6.26	CA_n28-n41-n77	103

6.26.1	Configurations	103

6.26.2	Maximum output power	104

6.26.3	REFSENS requirements	104

6.26.4	∆TIB and ∆RIB values	105

6.27	CA_n2-n48-n77	105

6.27.1	Configurations	105

6.27.2	Maximum output power	105

6.27.3	REFSENS requirements	105

6.27.4	Void	106

6.28	CA_n5-n48-n77	106

6.28.1	Configurations	106

6.28.2	Maximum output power	106

6.28.3	REFSENS requirements	106

6.28.4	Void	106

6.29	CA_n48-n66-n77	106

6.29.1	Configurations	106

6.29.2	Maximum output power	107

6.29.3	REFSENS requirements	107

6.29.4	Void	107

6.30	DL CA_n2-n5-n77, UL_ CA_n2A-n77A	107

6.30.1	Configurations	107

6.30.2	Maximum output power	108

6.30.3	REFSENS requirements	108

6.30.4	∆TIB and ∆RIB values	108

6.31	DL CA_n2-n5-n77, UL_CA_n5A-n77A	109

6.31.1	Configurations	109

6.31.2	Maximum output power	109

6.31.3	REFSENS requirements	109

6.31.4	∆TIB and ∆RIB values	109

6.32	CA_n1-n3-n41	109

6.32.1	Configurations	109

6.32.2	Maximum output power	110

6.32.3	REFSENS requirements	110

6.32.4	∆TIB and ∆RIB values	110

6.33	CA_n7-n66-n78	110

6.33.1	Configurations	110

6.33.2	Maximum output power	110

6.33.3	REFSENS requirements	111

6.33.4	∆TIB and ∆RIB values	111

6.34	CA_n25-n66-n78	111

6.34.1	Configurations	111

6.34.2	Maximum output power	112

6.34.3	REFSENS requirements	112

6.34.4	∆TIB and ∆RIB values	112

6.35	CA_n5-n7-n77	112

6.35.1	Configurations	112

6.35.2	Maximum output power	113

6.35.3	REFSENS requirements	113

6.35.4	∆TIB and ∆RIB values	114

6.36	CA_n7-n25-n77	115

6.36.1	Configurations	115

6.36.2	Maximum output power	116

6.36.3	REFSENS requirements	116

6.36.4	∆TIB and ∆RIB values	116

6.37	CA_n5-n25-n77	117

6.37.1	Configurations	117

6.37.2	Maximum output power	117

6.37.3	REFSENS requirements	117

6.37.4	∆TIB and ∆RIB values	118

6.38	CA_n5-n25-n78	118

6.38.1	Configurations	118

6.38.2	Maximum output power	118

6.38.3	REFSENS requirements	119

6.38.4	∆TIB and ∆RIB values	119

6.39	CA_n7-n66-n77	120

6.39.1	Configurations	120

6.39.2	Maximum output power	122

6.39.3	REFSENS requirements	122

6.39.4	∆TIB and ∆RIB values	122

6.40	CA_n7-n71-n77	123

6.40.1	Configurations	123

6.40.2	Maximum output power	123

6.40.3	REFSENS requirements	123

6.40.4	∆TIB and ∆RIB values	124

6.41	CA_n13-n25-n77	124

6.41.1	Configurations	124

6.41.2	Maximum output power	124

6.41.3	REFSENS requirements	125

6.41.4	∆TIB and ∆RIB values	125

6.42	CA_n1-n3-n77	125

6.42.1	Configurations	125

6.42.2	Maximum output power	126

6.42.3	REFSENS requirements	126

6.42.4	∆TIB and ∆RIB values	126

6.43	DL CA_n3-n77-n79, UL CA_n3A-n77A	126

6.43.1	Configurations	126

6.43.2	Maximum output power	127

6.43.3	REFSENS requirements	127

6.43.4	Void	127

6.44	DL CA_n3-n77-n79, UL CA_n3A-n79A	127

6.44.1	Configurations	127

6.44.2	Maximum output power	128

6.44.3	REFSENS requirements	128

6.44.4	Void	128

6.45	DL CA_n3-n77-n79, UL CA_n77A-n79A	128

6.45.1	Configurations	128

6.45.2	Maximum output power	128

6.45.3	REFSENS requirements	129

6.45.4	∆TIB and ∆RIB values	130

6.46	DL CA_n28-n77-n79, UL CA_n28A-n77A	130

6.46.1	Configurations	130

6.46.2	Maximum output power	130

6.46.3	REFSENS requirements	130

6.46.4	Void	131

6.47	DL CA_n28-n77-n79, UL CA_n28A-n79A	131

6.47.1	Configurations	131

6.47.2	Maximum output power	131

6.47.3	REFSENS requirements	131

6.47.4	Void	132

6.48	DL CA_n28-n77-n79, UL CA_n77A-n79A	132

6.48.1	Configurations	132

6.48.2	Maximum output power	132

6.48.3	REFSENS requirements	132

6.48.4	∆TIB and ∆RIB values	133

6.49	DL CA_n3-n28-n79, UL CA_n3A-n79A	134

6.49.1	Configurations	134

6.49.2	Maximum output power	134

6.49.3	REFSENS requirements	134

6.49.4	∆TIB and ∆RIB values	135

6.50	DL CA_n3-n28-n79, UL CA_n28A-n79A	135

6.50.1	Configurations	135

6.50.2	Maximum output power	136

6.50.3	REFSENS requirements	136

6.50.4	∆TIB and ∆RIB values	137

6.51	CA_n1-n28-n41	137

6.51.1	Configurations	137

6.51.2	Maximum output power	137

6.51.3	REFSENS requirements	137

6.51.4	∆TIB and ∆RIB values	138

6.52	CA_n18-n28-n77	138

6.52.1	Configurations	138

6.52.2	Maximum output power	138

6.52.3	REFSENS requirements	138

6.52.4	∆TIB and ∆RIB values	139

6.53	CA_n3-n18-n77	139

6.53.1	Configurations	139

6.53.2	Maximum output power	139

6.53.3	REFSENS requirements	140

6.53.4	∆TIB and ∆RIB values	140

7	Power class 2 CA with SUL	140

7.x	CA_nW-nX_SUL_nY-nZ	140

7.x.1	Configurations	140

7.x.2	Maximum output power	141

7.x.3	REFSENS requirements	141

7.x.4	∆TIB and ∆RIB values	141

Annex <A> (informative): Change history	142

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for high power UE for NR inter-band Carrier Aggregation or Dual Connection and NR SUL band combinations with y (1<y<=6) bands downlink and x (x=1, 2) bands uplink under Rel-18 time-frame. The purpose is to gather the relevant background information and studies in order to complete the band-combination specific requirements for the newly requested band combinations for power class 2 and power class 1.5 UE. Table 1-1 lists the three types of band combinations covered by this TR.

Table 1-1 High power UE inter-band CA/DC and SUL band combinations

| # | Band combination | Power class for uplink |
| --- | --- | --- |
| 1 | Power class 2 NR Inter-band CA/DC for y bands DL with x bands UL (x=1, 2) | 1UL(TDD): PC2 on TDD band 2UL (FDD+TDD, TDD+FDD, TDD+TDD): PC3 on FDD band, PC2 or PC3 on TDD band |
| 2 | Power class 1.5 NR Inter-band CA for y bands DL with 1 band UL | 1UL (TDD): PC1.5 on TDD band |
| 3 | Power class 2 SUL band combinations with or without CA for y bands DL with 2 bands UL | SUL: PC3 on SUL band NUL(TDD): PC2 on TDD band NUL = Normal Uplink in contrast to SUL. |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-222623: “New WID: High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TR 37.865-01-01: “NR Carrier Aggregation for intra-band (m Down Link (DL) / 1 Up Link (UL) bands) and inter-band (n Down Link (DL) / 1 Up Link (UL) bands)”

[5]	3GPP TR38.717-02-01 V17.0.0: “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 17)”

[6]	3GPP TR38.716-02-00 V16.0.0: “NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 16)”

[7]	R4-2321924 AdHoc report [109][104] NR_Baskets_Part_1, RAN4#109

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN4#97-e meeting, a basket Work Item on “High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)” [2] was approved for Rel-18. The objectives of the core part are as follows:

Specify the band-combination specific RF requirements for all listed band combinations as defined in attached excel file of this WI.

The requirements that need to analyse and specify include

-	Maximum output power.

-	Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

-	Other additional impact to the requirements due to the high power on UL, if necessary

The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

## 5.x	CA_nX-nY

### 5.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY | nX8,9, nY8,9, CA_nX-nY8 | nX |  |  |
|  |  | nY |  |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

<Editor’s note: In table 5.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

#### 5.x.3.1	Power class 2 case a

#### 5.x.3.2	Power class 2 case b

#### 5.x.3.3	Power class 2 case c

#### 5.x.3.4	Power class 1.5 for single uplink nX

#### 5.x.3.5	Power class 1.5 for single uplink nY

### 5.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 5.1	CA_n25-n41C

### 5.1.1	Configurations

Table 5.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25(2A)-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n25A-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.1.2	Maximum output power

Table 5.1.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.1.3	REFSENS requirements

#### 5.1.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n25:

Table 5.1.3-1 PC3 MSD

| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.1.3-2:

Table 5.1.3-2 PC2 MSD

| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

### 5.1.4	∆TIB and ∆RIB values

Void

## 5.2	CA_n41C-n66

### 5.2.1	Configurations

Table 5.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n66A | n418, 9 n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A) | n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.2.2	Maximum output power

Table 5.2.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.2.3	REFSENS requirements

#### 5.2.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n66:

Table 5.2.3-1 PC3 MSD

| CA_n41-n66 | CA_n41C | n4112 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  |  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.2.3-2:

Table 5.2.3-2 PC2 MSD

| CA_n41-n66 | CA_n41C | n4112 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |
|  |  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD |

### 5.2.4	∆TIB and ∆RIB values

Void

## 5.3	CA_n41C-n71

### 5.3.1	Configurations

Table 5.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n71A | n418, 9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | n418,9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | n418,9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

Note: DL CA_n41C-n71(2A) with UL CA_n41C is not in 38.101-1 17.7.0, but it was in agreed CR R4-2214650 From RAN4#104-e.

### 5.3.2	Maximum output power

Table 5.3.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.3.3	REFSENS requirements

#### 5.3.3.1	Power class 2 case a, b, c, d

For  PC3 CA_n41C there is no MSD into n71.

For  PC2 CA_n41C there should be no MSD into n71.

### 5.3.4	∆TIB and ∆RIB values

Void

## 5.4	CA_n41C-n77A

### 5.4.1	Configurations

Table 5.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n77A | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.4.2	Maximum output power

Table 5.4.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.4.3	REFSENS requirements

#### 5.4.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n77:

Table 5.4.3-1 PC3 MSD

| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | FDD | IMD9 |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.4.3-2.

Table 5.4.3-2 PC2 MSD

| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | FDD | IMD9 |

### 5.4.4	∆TIB and ∆RIB values

Void

## 5.5	CA_n77-n79

### 5.5.1	Configurations

The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. These are shown in Table 5.2A.2.1-1 of TS 38.101-1 [3].

Table 5.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | n77A8,9, n79A8,9 CA_n77A-n79A8 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.5.2	Maximum output power

Table 5.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.5.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01 [4]. For PC2 CA_n77-n79 with UL CA_n77-n79, the co-existence study is provided in TR38.717-02-01[5]. For PC2 and PC1.5 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01[4]. Analysis is based on these studies.

#### 5.5.3.0	Power class 2 case a, b, c and d

Considering both n77 and n79 are TDD bands and synchronous operation for CA_n77-n79 is assumed, no MSD is needed for dual UL of CA_n77-n79.

#### 5.5.3.1	Power class 2 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.3	Power class 1.5 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.4	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

### 5.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.6	CA_n78-n79

### 5.6.1	Configurations

For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. These are shown in Table 5.2A.2.1-1 of TS 38.101-1 [3].

Table 5.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A-n79A | n78A8,9, n79A8,9 n78A-n79A8 | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.6.2	Maximum output power

Table 5.6.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n78-n79 | CA power class | Carrier n78 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.6.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n78-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01 [4]. In addition, MSD due to cross band isolation is needed for UEs supporting the inter-band carrier aggregation with simultaneous Rx/Tx capability.

#### 5.6.3.0	Power class 2 case a, b, c and d

#### Considering both n78 and n79 are TDD bands, no MSD is needed for dual UL of CA_n78-n79.5.6.3.1	Power class 2 for single uplink n78

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-1 below.

Table 5.6.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 5 | >ACLR2 |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n78.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-2 below.

Table 5.6.3-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 5.6 | >ACLR2 |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 5.6 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.3	Power class 1.5 for single uplink n78

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

For MSD due to cross band isolation, MSD value of PC1.5 case will be 6dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-3 below.

Table 5.6.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 8 | >ACLR2 |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.4	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n78.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-2 below.

Table 5.6.3-4: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 8.6 | >ACLR2 |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 8.6 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

### 5.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.7	CA_n3-n41

### 5.7.1	Configurations

Table 5.7.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418,9 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.7.2	Maximum output power

This band combination does not support high power UL CA, so this section is omitted.

### 5.7.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.7.3.1	Power class 1.5 for single uplink n41

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		 the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n3.

-		MSD due to cross band isolation should be defined, PC2 	and PC3 of this band combination also suffers this kind of interference.

Table 5.7.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 3.9 | >ACLR2 |

### 5.7.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.8	CA_n3-n77

### 5.8.1	Configurations

Table 5.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77(2A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n77(3A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.8.2	Maximum output power

Table 5.8.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.8.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.8.3.1	Power class 2 case a, b

For UL CA_n3-n77, based on calculation, IMD 2, 4, 5 of dual UL falls into n3 DL, the MSD exception is defined as below, the values are reused from CA_n3-n78.

Additionally, IMD 5, 6, 7 and 8 of CA_n77(2A) UL falls into n3 DL. And for this combination PC3 with CA_n77(2A) UL is specified, with MSD defined for IMD7. Possible problematic 2UL IMD cases are:

-  IMD5: There is no need to specify the MSD because for IMD5 to fall on n3 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD6:  There is no need to specify the MSD because of the same reason.

-  IMD7: The MSD can be evaluated using the same test configuration as for PC3

-  IMD8. Because test points for IMD2 and IMD4 exist already, hence it is not recommended to add third test point for even order IMD.

As such, we recommend to specify IMD7 MSD using the same test configuration as was used for PC3. The MSD exception is defined as in Table below.

Table 5.8.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n774 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 |  |  |  |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.8.3.2	Power class 2 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n3 DL should be defined, the value is reused from CA_n3-n78.

Table 5.8.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |

#### 5.8.3.3	Power class 1.5 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n3 DL should be defined.

Table 5.8.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

### 5.8.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.9	CA_n8-n78

### 5.9.1	Configurations

Table 5.9.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n78A | n788 CA_n8A-n78A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.9.2	Maximum output power

Table 5.9.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n8-n78 | CA power class | Carrier n8 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n8-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.9.3	REFSENS requirements

For PC2 CA_n8-n78, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n78, where:

4th harmonic frequency of band 8 UL may fall into band n78 DL Rx

4th IMD frequencies (i.e. 3*f8-fn78) may fall into band 8 DL Rx

4th Harmonic mixing issue, i.e. Band n78 UL may fall into 4th receiver harmonic mixing frequency of band n8

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n78 configuration in TS38.101-1 specification.

#### 5.9.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n78A, no additional MSD are expected for this PC2 CA_n8A-n78A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n78A, no additional MSD are expected for this PC2 CA_n8A-n78A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1, it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD4 MSD for PC2 2UL/2DL CA_n8A-n78A should be defined additionally.

The IMD4 MSD for PC2 2UL/2DL CA_n8A-n78A is proposed to be defined in table 5.9.3.1-1, by reusing the same IMD4 MSD of PC2 2UL/2DL DC_8A_n78A.

Table 5.9.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |

#### 5.9.3.2	Power class 2 case b

For PC2 case b, similar with case a, no additional harmonic MSD are expected for this PC2 CA_n8A-n78A with 1 uplink carrier since the power of band n8 is kept as 23dBm.

For PC2 case b, the harmonic mixing issue, comparing with PC3 CA_n8A-n78A, additional MSD are expected for this PC2 CA_n8A-n78A since the power of band n78 is increased to 26dBm. The harmonic mixing MSD for PC2 CA_n8A-n78A is defined in table 5.9.3.2-1 by referring to the same MSD of PC2 CA_n5A-n77A.

Table 5.9.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD4 MSD value defined in table 5.9.3.1-1 can be applied.

### 5.9.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n78A requirements are applied for PC2 CA_n8A-n78A.

## 5.10	CA_n40-n77

### 5.10.1	Configurations

Table 5.10.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n77A | n778 | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.10.2	Maximum output power

This band combination does not support high power UL CA, so this section is omitted.

### 5.10.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.10.3.1	Power class 2 for single uplink n77

-		The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n40.

-		 The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n40.

-		n40 DL suffers UL2/DL3 harmonic mixing from n77.

-		 Cross band isolation of n77 UL falls into n40 DL.

Therefore, MSD issue for cross band isolation due to higher power on single n77 UL should be defined.

Table 5.10.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 6.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 6.5 | >ACLR2 |

Table 5.10.3.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |

### 5.10.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.11	CA_n28-n77

### 5.11.1	Configurations

Table 5.11.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A | n778,9 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77(2A) | n778,9 CA_n77(2A)8 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n77(3A) | n778,9 CA_n77(2A) CA_n28A-n77A8 | n28 | 5, 10 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.11.2	Maximum output power

Table 5.11.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.11.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.11.3.1	Power class 2 case a, b

For UL CA_n28A-n77A, based on calculation, IMD5 of dual UL falls into n28 DL, the MSD exception is defined as below.

For UL CA_n77(2A), based on calculation, IMD 2, 4, 6, 7 and 8 of dual n77 UL falls into n28 DL. Possible problematic cases are

-  IMD2: For IMD2 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD4

-  IMD6: Considered to be covered by lower even order case of IMD4. A note should be added to clarify that IMD6 is present but the corresponding MSD does not need to be specified (e.g. NOTE X as in Table below).

-  IMD7: For IMD7 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD8: Considered to be covered by lower even order case of IMD4

The MSD exception is defined as below.

Table 5.11.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD4X |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.11.3.2	Power class 2 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n28 DL should be defined, the value is reused from CA_n28-n78.

Table 5.11.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |

#### 5.11.3.3	Power class 1.5 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n28 DL should be defined,.

Table 5.11.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |

### 5.11.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.12	CA_n1-n41

### 5.12.1	Configurations

Table 5.12.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A | n418 CA_n1A-n41A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.12.2	Maximum output power

Table 5.12.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.12.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.12.3.1	Power class 2 case a, b

Based on calculation, no IMD of dual uplink falls into n1 DL, the MSD exception is not needed.

#### 5.12.3.2	Power class 2 for single uplink n41

-    the harmonic mixing interference of n41 does not fall into Rx frequencies of n1.

-    Cross band isolation interference of PC2 n41 falls into n1

The PC2 MSD due to cross band isolation is specified in following table.

Table 5.12.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 20.8 | >ACLR2 |

### 5.12.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.13	CA_n1-n77

### 5.13.1	Configurations

Table 5.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A | n778,9 CA_n1A-n77A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77(2A) | n778,9 CA_n1A-n77A8 CA_n77(2A)8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.13.2	Maximum output power

Table 5.13.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.13.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

(The analysis for PC3 is addressed in R4-2313802)

#### 5.13.3.1	Power class 2 case a, b

For UL CA_n1A-n77A) based on calculation, IMD2/4/5 of dual UL falls into n1 DL, the MSD exception is defined as below, which is reused from DC_1_n77.

For UL CA_n77(2A), based on calculation, IMD 5, 6 and 7 of dual intra-band n77 UL falls into n1 DL. Possible problematic cases are:

- IMD5

-  IMD6: For IMD6 to fall on n1 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD7: This is not an issue either since it can be covered by IMD5, the lowest odd order in this case. A note should be added to clarify that IMD7 is present but the corresponding MSD does not need to be specified, e.g. NOTE X as in Table below.

Table 5.13.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 | 5 | 25 | 2140 | 35.8 | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 31 | FDD | IMD5X |
|  | n7712 | 3310 | 10 | 1 RBSTART=25 | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.13.3.2      Power class 2 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.13.3.3	Power class 1.5 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.14	CA_n25-n78

### 5.14.1	Configurations

Table 5.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n78A | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n78(2A) | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.14.2	Maximum output power

Table 5.14.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.14.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.14.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD5 of dual UL falls into n25 DL. PC2 MSD value is reused from CA_n2-n77.

Table 5.14.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.14.3.2	Power class 2 for single uplink n78

Table 5.14.3.2-1 lists up to 7th harmonics for n25A-n78A which shows that there are 2nd harmonics issues from UL n25 into DL n78.

Table 5.14.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n78 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 | 19800 | 22800 | 23100 | 26600 |

Table 5.14.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.14.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

No MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined

#### 5.14.3.3	Power class 1.5 for single uplink n78

No MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined

### 5.14.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.15	CA_n66-n78

### 5.15.1	Configurations

Table 5.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n78A | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n78(2A) | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.15.2	Maximum output power

Table 5.15.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.15.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.15.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL falls into n66 DL. PC2 MSD value is reused from CA_n66-n77.

Table 5.15.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n78 | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

#### 5.15.3.2	Power class 2 for single uplink n78

Table 5.15.3.2-1 lists up to 7th harmonics for n66A-n78A which shows that there are 2nd harmonics issues from UL n66 into DL n78.

Table 5.15.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 | 10260 | 10680 | 11970 | 12460 |
| n78 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 | 19800 | 22800 | 23100 | 26600 |

Table 5.15.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.15.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

No MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined

#### 5.15.3.3	Power class 1.5 for single uplink n78

No MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined

### 5.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.16	CA_n7-n77

### 5.16.1	Configurations

Table 5.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n77A | n778, 9 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n77A | n778, 9 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n77(2A) | n778, 9 CA_n77(2A) 8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n77(2A) | n778, 9 CA_n77(2A) 8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n77(3A) | n778, 9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7(2A)-n77(3A) | n778, 9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.16.2	Maximum output power

Table 5.16.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.16.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.16.3.1	Power class 2 case a, b

Based on co-existence calculations, IMD4 of dual UL and IMD5 of non-contiguous UL falls into n7 DL.

The PC3 IMD4 MSD value for CA_n7-n77 is 7.1

The PC3 IMD5 MSD value from non-contiguous UL CA_n7-n77 is 15.

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 interference power increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed IMD4 and IMD5 MSD values for PC2 based on above calculations can be found in Table 5.16.3-1.

Table 5.16.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 18.2 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 29.9 | FDD | IMD5x |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.16.3.2	Power class 2 for single uplink n77

Table 5.16.3.2-1 lists up to 7th harmonics for n7A-n77A which shows that there are no harmonics issues.

Table 5.16.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 | 15000 | 15420 | 17500 | 17990 |
| n77 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 | 19800 | 25200 | 23100 | 29400 |

Table 5.16.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.16.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

No MSD issue due to harmonic or harmonic mixing from PC2 n77 UL need to be defined.

The cross band isolation needed is defined in PC2 table below. Values from CA_n7-n78 is reused.

Table 5.16.3.2-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 6.5 | >ACLR2 |

#### 5.16.3.3	Power class 1.5 for single uplink n77

No MSD issue due to harmonic or harmonic mixing from PC1.5 n77 UL need to be defined.

The cross band isolation needed is defined in PC1.5 table below. Values from CA_n41-n77 is reused.

Table 5.16.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 9.0 | >ACLR2 |

### 5.16.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.17	CA_n1-n79

### 5.17.1	Configurations

Table 5.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n79A | n798, 9 CA_n1A-n79A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.17.2	Maximum output power

Table 5.17.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.17.3	REFSENS requirements

### Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.17.3.1	Power class 2 case a, b

For PC2 CA_n1-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n1-n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-1, there is no harmonic issue for the band combination of n1 and n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-2, there is no harmonic mixing issue for the band combination of n1 and n79.

#### 5.17.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

#### Therefore, there is no MSD issue for this CA configuration.

#### 5.17.3.3	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

### 5.17.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n1A-n79A requirements are applied for PC2 CA_n1A-n79A and PC1.5 CA_n1A-n79A.

## 5.18	CA_n8-n79

### 5.18.1	Configurations

Table 5.18.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n79A | n798, 9 CA_n8A-n798 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.18.2	Maximum output power

Table 5.18.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n8-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.18.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

For PC2 and PC1.5 CA_n8-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n79.

According to the study results in TR 37.865-01-01[4] Table 8.14.2-1,

-	5th order IMD may fall into Rx frequencies of band n8.

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n79 configuration in TS38.101-1[3] Table 7.3A.5-1.

#### 5.18.3.1	Power class 2 case a, b

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1, it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD5 MSD for PC2 2UL/2DL CA_n8A-n79A should be defined additionally.

Table 5.18.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

|  | NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| CA_n8-n79 |  | n8 | 897.5 | 5 | 25 | 942.5 | 21.5 | FDD | IMD5 |
|  |  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD5 MSD value defined in table 5.18.3.1-1 can be applied.

#### 5.18.3.2	Power class 2 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.18.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.18.3.2	Power class 1.5 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.18.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

### 5.18.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n79A requirements are applied for PC2 CA_n8A-n79A and PC1.5 CA_n8A-n79A.

## 5.19	CA_n13-n77

### 5.19.1	Configurations

Table 5.19.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n77(2A) | CA_n77(2A)8 | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.19.2	Maximum output power

Table 5.19.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.19.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.19.3.1	Power class 2 case a, b

PC3 DL_n13A-n77(2A)_UL_n77(2A) is specified while IMD4/6 are missing. We submit a draft CR R4-2315469 to correct the error. For PC2 case, IMD 4/6 of dual n77 UL falls into n13 DL, which is the same as PC3. The MSD exception is defined as below.

Table 5.19.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n77 | n13 | 781 | 5 | 25 | 750 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 RBSTART=25 | 3885 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.19.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.20	CA_n5-n77

### 5.20.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n5A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.20.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n77(2A) | n778, 9 CA_n5A-n77A8 CA_n77(2A) 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n77(3A) | n778, 9 CA_n77(2A) 8 CA_n5A-n77A 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.20.2	Maximum output power

Table 5.20.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.20.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.20.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n5A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n5A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n28-n77.

Table 5.20.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n774 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 RBSTART=25 | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 RBSTART=25 | 3850 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.20.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n5A-n77(2A) DL is already specified.

#### 5.20.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n5A-n77(2A) DL is already specified.

## 5.21	CA_n66-n77

### 5.21.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n66A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.21.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A) 8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66A-n77(3A) | n778,9 CA_n77(2A) 8 CA_n66A-n77A 8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.21.2	Maximum output power

Table 5.21.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.21.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.21.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n66A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n66A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n1-n77.

Table 5.21.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n774 | n66 | N/A | 5 | N/A | 2197.5 | 31 | FDD | IMD5x |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.21.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n66A-n77(2A) DL is already specified.

#### 5.21.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n66A-n77(2A) DL is already specified.

## 5.22	CA_n25-n77

### 5.22.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n25A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.22.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n77(2A) | n778,9 CA_n77(2A) 8 CA_n25A-n77A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n77(3A) | n778,9 CA_n77(2A) 8 CA_n25A-n77A 8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.22.2	Maximum output power

Table 5.22.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.22.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.22.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n25A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n25A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n3-n77.

Table 5.22.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n774 | n25 | N/A | 5 | N/A | 1987.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 RBSTART=0 | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.22.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n25A-n77(2A) DL is already specified.

#### 5.22.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n25A-n77(2A) DL is already specified.

## 5.23	CA_n71-n78

### 5.23.1	Configurations

Table 5.23.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n71A-n78A | n788,9 CA_n71A-n78A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n78(2A) | n788,9 CA_n71A-n78A8 | n71 | 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.23.2	Maximum output power

Table 5.23.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.23.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.23.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n71-n78 falls into n71, the MSD values are defined as below.

Table 5.23.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

#### 5.23.3.2	Power class 2 for single uplink n78

Based on calculation, there is no harmonic issues for Rx of n71.

#### Therefore, no MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined.

#### 5.23.3.3	Power class 1.5 for single uplink n78

Based on calculation, there is no harmonic issues for Rx of n71.

Therefore, no MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined.

### 5.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.24	CA_n5-n78

### 5.24.1	Configurations

To enable CA_n78(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

Table 5.24.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n78(2A) | n788, 9 CA_n5A-n78A8 CA_n78(2A) 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

### 5.24.2	Maximum output power

Table 5.24.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.24.3	REFSENS reqsuirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n78(2A) UL for PC2.

#### 5.24.3.1	Power class 2 case a, b

CA_n78(2A) UL for CA_n5A-n78(2A) DL is already defined for PC3. PC2 is also defined for CA_n5A-n78(2A) DL but not yet for CA_n78(2A) UL.

PC2 test points for the now added CA_n78(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n28-n78.

Table 5.24.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 RBSTART=25 | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 RBSTART=25 | 3780 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.24.3.2      Power class 2 for single uplink n78

This section is omitted since PC2 single uplink for CA_n5A-n78(2A) DL is already specified.

## 5.25	DL CA_n1-n78, UL n78A

### 5.25.1	Configurations

Table 5.25.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | n788,9 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.25.2	Maximum output power

Table 5.25.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n1-n78 | CA power class | Carrier n1 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.25.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.25.3.1	Power class 2 case a

This PC2 band combination of CA_n1A-n78A is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.2	Power class 2 case b

This PC2 band combination of CA_n1A-n78A is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.4	Power class 1.5 for single uplink n78

For PC3 CA_n1-n78 with single uplink, the co-existence study is provided in TR 38.716-02-00 [6].

Based on above, there is no harmonic issue for the band combination of n1 and n78.

Based on above, there is no harmonic mixing issue for the band combination of n1 and n78.

For PC1.5 CA_n1-n78 with single uplink, there is no harmonic and harmonic mixing issue.

### 5.25.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.26	DL CA_n3-n78, UL n78A

### 5.26.1	Configurations

Table 5.26.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | n788,9 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.26.2	Maximum output power

Table 5.26.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n78 | CA power class | Carrier n3 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.26.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.26.3.1	Power class 2 case a

This PC2 band combination of CA_n3A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.2	Power class 2 case b

This PC2 band combination of CA_n3A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.4	Power class 1.5 for single uplink n78

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-		the 2nd order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC1.5 n78 UL falling into n3 DL should be defined, the value is reused from CA_n3-n77.

Table 5.26.3.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

### 5.26.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.27	DL CA_n28-n78, UL n78A

### 5.27.1	Configurations

Table 5.27.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A | n788,9 CA_n28A-n78A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.27.2	Maximum output power

Table 5.27.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n78 | CA power class | Carrier n28 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.27.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.27.3.1	Power class 2 case a

This PC2 band combination of CA_n28A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.2	Power class 2 case b

This PC2 band combination of CA_n28A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.4	Power class 1.5 for single uplink n78

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-		the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC1.5 n78 UL falling into n28 DL should be defined, the value is reused from CA_n28-n77.

Table 5.27.3.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 30 | 15 | 160 (Rbstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |

### 5.27.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.28	CA_n8-n77

### 5.28.1	Configurations

Table 5.28.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n77A | n778,9 CA_n8A-n77A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.28.2	Maximum output power

Table 5.28.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n8-n77 | CA power class | Carrier n8 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n8A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.9.3	REFSENS requirements

For PC2 CA_n8-n77, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n77, where:

4th harmonic frequency of band 8 UL may fall into band n77 DL Rx

4th IMD frequencies (i.e. 3*f8-fn77) may fall into band 8 DL Rx

4th Harmonic mixing issue, i.e. Band n77 UL may fall into 4th receiver harmonic mixing frequency of band n8

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n77 configuration in TS38.101-1[3] specification.

#### 5.28.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n77A, no additional MSD are expected for this PC2 CA_n8A-n77A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n77A, no additional MSD are expected for this PC2 CA_n8A-n77A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1[3], it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD4 MSD for PC2 2UL/2DL CA_n8A-n77A should be defined additionally.

The IMD4 MSD for PC2 2UL/2DL CA_n8A-n77A is proposed to be defined in table 5.28.3.1-1, by reusing the same IMD4 MSD of PC2 2UL/2DL CA_n8A_n78A.

Table 5.28.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | N/A |

#### 5.28.3.2	Power class 2 case b

For PC2 case b, similar with case a, no additional harmonic MSD are expected for this PC2 CA_n8A-n77A with 1 uplink carrier since the power of band n8 is kept as 23dBm.

For PC2 case b, the harmonic mixing issue, comparing with PC3 CA_n8A-n77A, additional MSD are expected for this PC2 CA_n8A-n77A since the power of band n77 is increased to 26dBm. The harmonic mixing MSD for PC2 CA_n8A-n77A is defined in table 5.28.3.2-1 by referring to the same MSD of PC2 CA_n8A-n78A.

Table 5.28.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n8 | 10 | 15 | 25 (Rbstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 20 | 15 | 20 (Rbstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD4 MSD value defined in table 5.28.3.1-1 can be applied.

### 5.28.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n77A requirements are applied for PC2 CA_n8A-n77A.

## 5.29	CA_n3-n79

### 5.29.1	Configurations

Table 5.29.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | N798,9 CA_n3A-n79A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.29.2	Maximum output power

This band combination does not support high power UL CA, so this section is omitted.

### 5.29.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.29.3.1	Power class 2 case a, b

This band combination does not support high power UL CA, so this section is omitted.

#### 5.29.3.2	Power class 2 for single uplink n79

This band combination does not support high power UL CA, so this section is omitted.

#### 5.29.3.3	Power class 1.5 for single uplink n79

Based on MSD analysis for PC3, there’re no harmonic/harmonic mixing/cross-band isolation issues. Therefore, no MSD exception needs to be defined.

### 5.29.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.30	CA_n28-n79

### 5.30.1	Configurations

Table 5.30.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | n798, 9 CA_n28A-n79A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.30.2	Maximum output power

Table 5.30.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.30.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.30.3.1	Power class 2 case a, b

For PC2 CA_n28-n79, the co-existence studies on the harmonic issue and intermodulation issues have already been defined in TS38.101-1[3] specification.

#### 5.30.3.2	Power class 2 for single uplink n79

For PC2 CA_n28-n79 for single uplink n79, the co-existence studies on the harmonic issue and intermodulation issues have already been defined in TS38.101-1[3] specification.

#### 5.30.3.3	Power class 1.5 for single uplink n79

For PC1.5 CA_n28-n79 for single uplink n79,

- the 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n28.

- the 2nd, 3rd, 4th and 5th order harmonic mixing do not fall into Rx frequencies of n28.

Therefore, no MSD exception needs to be defined.

### 5.30.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.31	CA_n41-n79

### 5.31.1	Configurations

Table 5.31.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | n418,9 n798,9 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90,100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.31.2	Maximum output power

Table 5.31.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n41-n79 | CA power class | Carrier n41 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.31.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.31.3.1	Power class 2 case a, b, c, d

Based on TS38.101-1[3] Table 7.3A.5-1 and Table 7.3A.5-1b, there is no harmonic issue for this band combination of n41 and n79.

Based on the same tables, there is no harmonic mixing issue for this band combination of n41 and n79.

#### 5.31.3.2	Power class 2 for single uplink n79

For PC2 CA_n41-n79 for single uplink n79 is already defined in TS38.101-1[3].

So, this section is omitted.

#### 5.31.3.3	Power class 1.5 for single uplink n79

Based on above,

- No harmonic mixing issue.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.31.3.4	Power class 2 for single uplink n41

For PC2 CA_n41-n79 for single uplink n41 is already defined in TS38.101-1[3].

So, this section is omitted.

#### 5.31.3.5	Power class 1.5 for single uplink n41

Based on above,

- No harmonic mixing issue.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

### 5.31.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.32	CA_n18-n41

### 5.32.1	Configurations

Table 5.32.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | n41A 8 CA_n18A-n41A 8 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.32.2	Maximum output power

Table 5.32.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.32.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink inter-band CA.

- the 3rd order harmonic mixing may fall into Rx frequencies of n18.

- IMD3 of inter-band UL CA, CA_n18A-n41A, may fall into Rx frequencies of n18.

- No crossband isolation interference.

#### 5.32.3.1	Power class 2 case a, b

MSD due to IMD3 of inter-band UL CA to downlink n18 is defined as below.

Table 5.32.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 24.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |

#### 5.32.3.2	Power class 2 for single uplink n41

MSD due to harmonic mixing is defined as below.

Table 5.32.3-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n18X | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE Y | UL1/DL3 |
| n41 | n18X | 10 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE Y | UL1/DL3 |
| NOTE X: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE Y: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### 5.32.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

## 5.33	CA_n18-n77

### 5.33.1	Configurations

Table 5.33.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77A | CA_n18A-n77A 8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.33.2	Maximum output power

Table 5.33.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

Unaffected parts are omitted.

### 5.33.3	REFSENS requirements

Comparing the 3 results, for both IMD4 and IMD5, the biggest difference among the corresponding MSD values are 2.3dB for IMD4 and 2.5dB for IMD5, hence the data from 3 companies are quite consistent. The test points are the same.  During RAN4#109 meeting in Chicago, experts from the 3 Companies agree to average the corresponding results to the value for PC2 [7] as shown in Table 5.33.3.1

Table 5.33.3-1. MSD values for PC2 HPUE CA_n18-n77

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 17.5 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.5 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.x	CA_nX-nY-nZ

### 6.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY-nZ | nX7,9, nY7,9, CA_nX-nY7 | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

<Editor’s note: In table 6.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.x.3.1	Power class 2 case a

### 6.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 6.1 	DL CA_n25-n41-n66, UL CA_n25A-n41A

### 6.1.1	Configurations

Table 6.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.1.2	Maximum output power

Table 6.1.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.1.3	REFSENS requirements

#### 6.1.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n25A-n41A into CA_n25-n41-n66, none is proposed for PC2.

#### 6.1.3.2	Power class 2 case b

Since there is no MSD for PC3 UL CA_n25A-n41A into CA_n25-n41-n66, none is proposed for PC2.

### 6.1.4	Void

## 6.2	DL CA_n25-n41-n66, UL CA_n41A-n66A

### 6.2.1	Configurations

Table 6.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.2.2	Maximum output power

Table 6.2.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.2.3	REFSENS requirements

#### 6.2.3.1	Power class 2 case a

There is 11.0 dB of MSD for PC3 CA_n41A-n66A into n25:

Table 6.2.3-1 PC3 MSD

| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 interference power increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.x.3-2.

Table 6.x.3-2 PC2 MSD

| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 22.7 | FDD | IMD4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

#### 6.2.3.2	Power class 2 case c

The same MSD applies for Power class 2 case c as power class 2 case a.

### 6.2.4	Void

## 6.3	DL CA_n25-n41-n71, UL CA_n25A-n41A

### 6.3.1	Configurations

Table 6.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.3.2	Maximum output power

Table 6.3.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.3.3	REFSENS requirements

#### 6.3.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n25A-n41A into n71, none is proposed for PC2.

#### 6.3.3.2	Power class 2 case b

Since there is no MSD for PC3 UL CA_n25A-n41A into -n71, none is proposed for PC2.

### 6.3.4	Void

## 6.4	DL CA_n25-n41-n71, UL CA_n41A-n71A

### 6.4.1	Configurations

Table 6.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.4.2	Maximum output power

Table 6.4.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.4.3	REFSENS requirements

#### 6.4.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n25-n41-n71, none is proposed for PC2.

#### 6.4.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n25-n41-n71, none is proposed for PC2.

### 6.4.4	Void

## 6.5	DL CA_n25-n41-n71, UL CA_n41C

### 6.5.1	Configurations

Table 6.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C-n71A | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.5.2	Maximum output power

Table 6.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.5.3	REFSENS requirements

#### 6.5.3.1	Power class 2 case a, case b, case c, case c

Any MSD for UL CA_n41C PC2 is handled in the 2 band DL fallbacks.

### 6.5.4	Void

## 6.6	DL CA_n25-n41-n77, UL CA_n25A-n41A

### 6.6.1	Configurations

Table 6.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A7 CA_n25A-n77A CA_n41A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n77A | CA_n25A-n41A7 CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.6.2	Maximum output power

Table 6.6.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.6.3	REFSENS requirements

#### 6.6.3.1	Power class 2 case a

Power class 3 CA for UL CA_n25A-n41A:

Table 6.6.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n41A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.6.3.1-1.

Table 6.6.3.1-2 Proposed Power class 2 MSD for UL CA_n25-n41-n71

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 23.7 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 17.2 | TDD | IMD5 |

#### 6.6.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.6.4	∆TIB and ∆RIB values

Void.

## 6.7	DL CA_n25-n41-n77, UL CA_n25A-n77A

### 6.7.1	Configurations

Table 6.7.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A7 CA_n41A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  | CA_n25A-n41A CA_n25A-n77A7 CA_n41A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
| CA_n25(2A)-n41A-n77A |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.7.2	Maximum output power

Table 6.7.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.7.3	REFSENS requirements

#### 6.7.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n25A-n77A:

Table 6.7.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5ZZ |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.7.3.1-2.

Table 6.7.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2640 | 5 | 25 | 2640 | 18.8 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.7.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.7.4	∆TIB and ∆RIB values

Void

## 6.8	DL CA_n25-n41-n77, UL CA_n41A-n77A

### 6.8.1	Configurations

Table 6.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
| CA_n25(2A)-n41A-n77A |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.8.2	Maximum output power

Table 6.8.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.8.3	REFSENS requirements

#### 6.8.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.8.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3ZZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2675 | 5 | 25 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.8.3.1-2.

Table 6.8.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n41A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | 26.5 | FDD | IMD35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2675 | 5 | 25 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 20.0 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.8.4	∆TIB and ∆RIB values

Void

## 6.9	DL CA_n25-n66-n77, UL CA_n25A-n77A

### 6.9.1	Configurations

Table 6.9.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.9.2	Maximum output power

Table 6.9.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.9.3	REFSENS requirements

#### 6.9.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n25A-n77A:

Table 6.9.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | 29.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*4=12 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.9.3.1-2.

Table 6.9.3.1-2 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | 35.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 22.0 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 16.9 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |

#### 6.9.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.9.4	∆TIB and ∆RIB values

Void.

## 6.10	DL CA_n25-n66-n77, UL CA_n66A-n77A

### 6.10.1	Configurations

Table 6.10.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.10.2	Maximum output power

Table 6.10.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.10.3	REFSENS requirements

#### 6.10.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.10.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4ZZ |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5ZZ |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*4=12 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.10.3.1-2.

Table 6.10.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n25-n66-n77 | n25 | 1880 | 5 | 25 | 1960 | 38.1 | FDD | IMD2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 20.6 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 13.2 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.10.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.10.4	∆TIB and ∆RIB values

Void

## 6.11	DL CA_n25-n71-n77, UL CA_n25A-n77A

### 6.11.1	Configurations

Table 6.11.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.11.2	Maximum output power

Table 6.11.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.11.3	REFSENS requirements

#### 6.11.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n25A-n77A, so there should be none for PC2.

#### 6.11.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.11.4	∆TIB and ∆RIB values

Void.

## 6.12	DL CA_n25-n71-n77, UL CA_n71A-n77A

### 6.12.1	Configurations

Table 6.12.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.12.2	Maximum output power

Table 6.12.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.12.3	REFSENS requirements

#### 6.12.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.12.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.12.3.1-2.

Table 6.12.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 25.4 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.12.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.12.4	∆TIB and ∆RIB values

Void

## 6.13	DL CA_n41-n66-n77, UL CA_n41A-n66A

### 6.13.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.13.2	Maximum output power

<Editor’s note: In table 6.13.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.13.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.13.3	REFSENS requirements

#### 6.13.3.1	Power class 2 case a

Power class 3 CA for UL CA_n25A-n41A:

Table 6.13.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n66A

| CA_n41-n66-n77 | n41 | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 16.1 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.13.3.1-1.

Table 6.13.3.1-2 Proposed Power class 2 MSD for UL CA_n41-n66

| CA_n41-n66-n77 | n41 | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 25.0 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.13.3.2	Power class 2 case b

The same MSD for case a applies to casec.

### 6.13.4	∆TIB and ∆RIB values

Void.

## 6.14	DL CA_n41-n66-n77, UL CA_n41A-n77A

### 6.14.1	Configurations

Table 6.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A7 CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A7 CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.14.2	Maximum output power

Table 6.14.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.14.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.14.3.1	Power class 2 case a, case b, case c, case d

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.14.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n41-n66-n77 | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.14.3.1-2.

Table 6.14.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_41A-n77A

| CA_n41-n66-n77 | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | 20.5 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

### 6.14.4	∆TIB and ∆RIB values

Void

## 6.15	DL CA_n41-n66-n77, UL CA_n66A-n77A

### 6.15.1	Configurations

Table 6.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.15.2	Maximum output power

Table 6.15.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.15.3	REFSENS requirements

#### 6.15.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.15.3.1-1 Power class 3 MSD for 2 bands UL CA_n66A-n77A

| CA_n41-n66-n77 | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.15.3.1-2.

Table 6.15.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n41-n66-n77 | n41 | 2670 | 5 | 25 | 2670 | 18.7 | TDD | IMD55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.15.4	∆TIB and ∆RIB values

Void

## 6.16	DL CA_n41-n71-n77, UL CA_n41A-n71A

### 6.16.1	Configurations

Table 6.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.16.2	Maximum output power

Table 6.16.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.16.3	REFSENS requirements

#### 6.16.3.1	Power class 2 case a

Power class 3 CA for UL CA_n41A-n71A:

Table 6.16.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n71A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 | 25 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.16.3.1-1.

Table 6.16.3.1-2 Proposed Power class 2 MSD for UL CA_n41-n71

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 35.1 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 | 25 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | 25.2 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 21.9 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.16.3.2	Power class 2 case c

The same MSD for case a applies to case c.

### 6.16.4	∆TIB and ∆RIB values

Void.

## 6.17	DL CA_n41-n71-n77, UL CA_n41A-n77A

### 6.17.1	Configurations

Table 6.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.17.2	Maximum output power

Table 6.17.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.17.3	REFSENS requirements

#### 6.17.3.1	Power class 2 case a, case b, case c, case d

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.17.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n41-n71-n77 | 41 | 2680 | 5 | 25 | 2680 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 30.8 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.17.3.1-2.

Table 6.17.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_41A-n77A

| CA_n41-n71-n77 | 41 | 2680 | 5 | 25 | 2680 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 36.8 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.17.4	∆TIB and ∆RIB values

Void

## 6.18	DL CA_n41-n71-n77, UL CA_n71A-n77A

### 6.18.1	Configurations

Table 6.18.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.18.2	Maximum output power

Table 6.18.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.18.3	REFSENS requirements

#### 6.18.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.18.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2564 | 5 | 25 | 2564 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.18.3.1-2.

Table 6.18.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | 34.7 | TDD | IMD25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2564 | 5 | 25 | 2564 | 24.4 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.18.4	∆TIB and ∆RIB values

Void

## 6.19	DL CA_n66-n71-n77, UL CA_n66A-n77A

### 6.19.1	Configurations

Table 6.19.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.19.2	Maximum output power

Table 6.19.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.19.3	REFSENS requirements

#### 6.19.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.19.3.1-1 Power class 3 MSD for 2 bands UL CA_n66A-n77A

| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.19.3.1-2.

Table 6.19.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 24.2 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.19.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.19.4	∆TIB and ∆RIB values

Void.

## 6.20	DL CA_n66-n71-n77, UL CA_n71A-n77A

### 6.20.1	Configurations

Table 6.20.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.20.2	Maximum output power

Table 6.20.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.20.3	REFSENS requirements

#### 6.20.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.20.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n66-n71-n77 | n66 | 1750 | 5 | 25 | 2150 | 15.5 | FDD | IMD32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.20.3.1-2.

Table 6.20.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n66-n71-n77 | n66 | 1750 | 5 | 25 | 2150 | 24.4 | FDD | IMD32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.20.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.20.4	∆TIB and ∆RIB values

Void

## 6.21	DL CA_n41-n66-n71, UL CA_n41A-n71A

### 6.21.1	Configurations

Table 6.21.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.21.2	Maximum output power

Table 6.21.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.21.3	REFSENS requirements

#### 6.21.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n41-n66-n71, none is proposed for PC2.

#### 6.21.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n41-n66-n71, none is proposed for PC2.

### 6.21.4	Void

## 6.22	DL CA_n41-n66-n71, UL CA_n41A-n71A

### 6.22.1	Configurations

Table 6.22.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.22.2	Maximum output power

Table 6.22.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.22.3	REFSENS requirements

#### 6.22.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n66A into CA_n41-n66-n71, none is proposed for PC2.

#### 6.22.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n66A into CA_n41-n66-n71, none is proposed for PC2.

### 6.22.4	Void

## 6.23	CA_n3-n28-n41

### 6.23.1	Configurations

Table 6.23.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | CA_n3A-n28A CA_n3A-n41A7 CA_n28A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.23.2	Maximum output power

Table 6.23.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.23.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.23.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD3 of dual UL CA_n3-n41 fall into n28 DL, IMD2 of dual UL CA_n28-n41 falls into n3 DL.

Table 6.23.3-13 DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n41 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 32 | FDD | IMD24 |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 32 | FDD | IMD2 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.24	CA_n3-n41-n77

### 6.24.1	Configurations

Table 6.24.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A-n77A | CA_n3A-n41A7 CA_n3A-n77A7 CA_n41A-n77A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.24.2	Maximum output power

Table 6.24.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.24.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.24.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD3/5 of dual UL CA_n3-n41 fall into n77 DL; IMD3/4 of dual UL CA_n41-n77 falls into n3 DL; IMD5 of dual UL CA_n3-n77 falls into n41 DL.

Table 6.24.3-13 DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | 25.6 | TDD | IMD31 |
|  | n3 | 1745 | 5 | 25 | 1840 | 25.1 | FDD | IMD32 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 13 | TDD | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.24.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.25	CA_n3-n28-n77

### 6.25.1	Configurations

Table 6.25.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n77A | n777 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | n777 CA_n77(2A)7 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.25.2	Maximum output power

Table 6.25.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.25.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.25.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n3-n77 fall into n28 DL, IMD3/4 of dual UL CA_n28-n77 falls into n3 DL.

For UL CA_n77(2A) with DL CA_n3A-n77(2A), based on calculation, IMD 2, 4, 5 of dual UL falls into n3 DL, the MSD exception are also already defined in Table 7.3A.5-1a of the current TS 38.101-1 [2]. For UL CA_n77(2A), with DL CA_n28A-n77(2A), IMD 2, 4, 6, 7 and 8 of dual n77 UL falls into n28 DL. The MSD exception have also been defined in Table 7.3A.5-1a of the current TS 38.101-1 [2].Table 6.25.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n77 | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 24.2 | FDD | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 25.8 | FDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.25.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.26	CA_n28-n41-n77

### 6.26.1	Configurations

Table 6.26.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n77A | n417 n777 CA_n28A-n41A7 CA_n28A-n77A7 CA_n41A-n77A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | n417 n777 CA_n28A-n41A7 CA_n28A-n77A7 CA_n41A-n77A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.26.2	Maximum output power

Table 6.26.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.26.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.26.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD2/3/4 of dual UL CA_n28-n41 fall into n77 DL; IMD2/3/5 of dual UL CA_n41-n77 falls into n28 DL; IMD2/3 of dual UL CA_n28-n77 falls into n41 DL.

Table 6.26.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n77 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 34.2 | TDD | IMD22,4 |
|  | n28 | 743 | 5 | 25 | 798 | 36.8 | FDD | IMD21,4 |
|  | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 35.5 | TDD | IMD24 |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.26.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.27	CA_n2-n48-n77

### 6.27.1	Configurations

Table 6.27.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n48A-n77A | n777, 9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.27.2	Maximum output power

Table 6.27.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.27.3	REFSENS requirements

Analysis performed for the PC3 CA_n2A-n48A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-	Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence, no MSD is needed for IMD4 and IMD5 products produced by Band n2 and n48 that might fall in Rx of band n77.and IMD4 and IMD5 products produced by Band n2 and n77 that might fall in Rx of band n48.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.27.3.1	Power class 2 case a, b

No additional MSD is needed, since uplink configuration CA_n2A-n77A PC2 will still not affect band n48, since n48 and n77 are not simultaneous RX/TX.

### 6.27.4	Void

## 6.28	CA_n5-n48-n77

### 6.28.1	Configurations

Table 6.28.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A-n77A | n777, 9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.28.2	Maximum output power

Table 6.28.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.28.3	REFSENS requirements

Analysis performed for the PC3 CA_n5A-n48A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-    Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.28.3.1	Power class 2 case a, b

No additional MSD is needed

### 6.28.4	Void

## 6.29	CA_n48-n66-n77

### 6.29.1	Configurations

Table 6.29.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n66A-n77A | n777, 9 CA_n48A-n66A CA_n66A-n77A7 | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.29.2	Maximum output power

Table 6.29.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n48A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.29.3	REFSENS requirements

Analysis performed for the PC3 CA_n48A-n66A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-	Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence, no MSD is needed for IMD4 and IMD5 products produced by Band n48 and n66 that might fall in Rx of band n77 and IMD4 and IMD5 products produced by Band n66 and n77 that might fall in Rx of band n48.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.29.3.1	Power class 2 case a, b

No additional MSD is needed, since uplink configuration CA_n66A-n77A PC2 will still not affect band n48, since n48 and n77 are not simultaneous RX/TX.

### 6.29.4	Void

## 6.30	DL CA_n2-n5-n77, UL_ CA_n2A-n77A

### 6.30.1	Configurations

Table 6.30.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A-n77C | CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.30.2	Maximum output power

Table 6.30.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n2-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.30.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements had been studied in R4-2117251 for both case a and b, and the requirements were specified in 38.101-1 due to higher power uplink. Thus, there is no more additional requirement.

### 6.30.4	∆TIB and ∆RIB values

The values had been specified in 38.101-1. There is no change by comparing to the values, so this section is omitted.

## 6.31	DL CA_n2-n5-n77, UL_CA_n5A-n77A

### 6.31.1	Configurations

Table 6.31.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A-n77C | CA_n5A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.31.2	Maximum output power

Table 6.31.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.31.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements had been studied in R4-2117251 for both case a and b, and the requirements were specified in 38.101-1 due to higher power uplink. Thus, there is no more additional requirement.

### 6.31.4	∆TIB and ∆RIB values

The values had been specified in 38.101-1. There is no change by comparing to the values, so this section is omitted.

## 6.32	CA_n1-n3-n41

### 6.32.1	Configurations

Table 6.32.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n41A | CA_n1A-n41A7 CA_n3A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.32.2	Maximum output power

Table 6.32.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.32.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.32.3.1	Power class 2 case a, b

Based on calculation, no MSD issue for dual UL CA_n1-n41 falls into n3 DL or dual UL CA_n3-n41 falls into n1 DL.

### 6.32.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.33	CA_n7-n66-n78

### 6.33.1	Configurations

Table 6.33.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n66A-n78A | n787, 9 CA_n7A-n78A7 CA_n66A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.33.2	Maximum output power

Table 6.33.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.33.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 6.33.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n78 fall into n66 DL, MSD values reused from CA_n41-n66-n77.

Table 6.33.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n66-n78 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 20.5 | FDD | IMD4 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.33.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.34	CA_n25-n66-n78

### 6.34.1	Configurations

Table 6.34.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n78A | n787, 9 CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | n787, 9 CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.34.2	Maximum output power

Table 6.34.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.34.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 6.34.3.1	Power class 2 case a, b

IMD4 of dual UL CA_n25-n78 falls into n66 DL. MSD value n66 is reused from PC2 DC_2A-66A_n77A.

IMD2/4/5 of dual UL CA_n66-n78 falls into n25 DL, MSD value n25 is reused from PC2 CA_n2-n66-n77.

Table 6.34.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | M/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 21.1 | FDD | IMD4 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 37.6 | FDD | IMD21,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.34.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.35	CA_n5-n7-n77

### 6.35.1	Configurations

Table 6.35.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n7A-n77A | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n7A-n77(2A) | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n5A-n7A-n77(3A) | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.35.2	Maximum output power

Table 6.35.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.35.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.35.3.1	Power class 2 case a, b

Based on calculation, IMD2 of dual UL CA_n5-n77 falls into n7 DL and IMD2/3/5 of dual UL CA_n7-n77 falls into n5 DL respectively, the MSD values are defined as below.

Table 6.35.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n7-n77 | n5 | N/A | 5 | N/A | 879 | 35.2 | FDD | IMD21,4 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 35.1 | FDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.35.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.36	CA_n7-n25-n77

### 6.36.1	Configurations

Table 6.36.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n25A-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n77(3A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.36.2	Maximum output power

Table 6.36.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.36.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.36.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n77 falls into n25 DL and IMD5 of dual UL CA_n25-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.36.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n25-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 20.0 | FDD | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |

### 6.36.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.37	CA_n5-n25-n77

### 6.37.1	Configurations

Table 6.37.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n25A-n77A | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n77(2A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n77(3A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.37.2	Maximum output power

Table 6.37.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.37.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.37.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL n5-n77 fall into n25 DL, MSD values reused from CA_n2-n5-n77.

Based on calculation, IMD5 of dual UL n25-n77 fall into n5 DL, MSD values reused from CA_n2-n5-n77.

Table 6.37.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n25-n77 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD55 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.37.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.38	CA_n5-n25-n78

### 6.38.1	Configurations

Table 6.38.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n25A-n78A | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.38.2	Maximum output power

Table 6.38.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.38.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.38.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL n5-n78 fall into n25 DL, MSD values reused from CA_n2-n5-n77.

Based on calculation, IMD5 of dual UL n25-n78 fall into n5 DL, MSD values reused from CA_n2-n5-n77.

Table 6.38.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n25-n78 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |

### 6.38.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.39	CA_n7-n66-n77

### 6.39.1	Configurations

Table 6.39.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n66A-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n77(3A) | n77A7,9 CA_n77(2A)7 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.39.2	Maximum output power

Table 6.39.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.39.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.39.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n77 falls into n66 DL and IMD5 of dual UL CA_n66-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.39.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n66-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 20.5 | FDD | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |

### 6.39.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.40	CA_n7-n71-n77

### 6.40.1	Configurations

Table 6.40.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n71A-n77A | n77A7,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n71A-n77(2A) | n77A7,9 CA_n77(2A) CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n7A-n71A-n77(3A) | n77A7,9 CA_n77(2A) CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n7A-n71A-n77A | n77A7,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.40.2	Maximum output power

Table 6.40.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n71-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.40.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.40.3.1	Power class 2 case a, b

Based on calculation, IMD2 of dual UL CA_n71-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.40.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n71-n77 | n7 | N/A | 5 | N/A | 2670 | 34.6 | FDD | IMD2 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |

### 6.40.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.41	CA_n13-n25-n77

### 6.41.1	Configurations

Table 6.41.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n25A-n77A | n77A7,9 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n77(2A) | n77A7,9 CA_n77(2A) CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.41.2	Maximum output power

Table 6.41.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n13-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.41.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.41.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n13-n77 falls into n25 DL respectively. The MSD values are defined as below.

Table 6.41.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n25-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 24.0 | FDD | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | TDD | N/A |

### 6.41.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.42	CA_n1-n3-n77

### 6.42.1	Configurations

Table 6.42.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n77A | CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.42.2	Maximum output power

Table 6.42.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.42.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.42.3.1	Power class 2 case a, b

Based on calculation, IMD2/4/5 of dual UL CA_n1A-n77A falls into n3 DL and IMD2/5 of dual UL CA_n3A-n77A falls into n1 DL respectively, the MSD exceptions are defined as below, which are reused from DC_1-3_n77.

Table 6.42.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 37.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 37.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.42.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.43	DL CA_n3-n77-n79, UL CA_n3A-n77A

### 6.43.1	Configurations

Table 6.43.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A7 CA_n3A-n79A CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A7 CA_n3A-n79A CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.43.2	Maximum output power

Table 6.43.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n77 | CA power class | Carrier n3 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.43.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.43.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n3A-n77A into CA n3-n77-n79, none is proposed for PC2.

#### 6.43.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n3A-n77A into CA n3-n77-n79, none is proposed for PC2.

### 6.43.4	Void

## 6.44	DL CA_n3-n77-n79, UL CA_n3A-n79A

### 6.44.1	Configurations

Table 6.44.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A7 CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A7 CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.44.2	Maximum output power

Table 6.44.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n79 | CA power class | Carrier n3 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.44.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.44.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n3A-n79A into CA n3-n77-n79, none is proposed for PC2.

#### 6.44.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n3A-n79A into CA n3-n77-n79, none is proposed for PC2.

### 6.44.4	Void

## 6.45	DL CA_n3-n77-n79, UL CA_n77A-n79A

### 6.45.1	Configurations

Table 6.45.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.45.2	Maximum output power

Table 6.45.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n77-n79 | CA power class | Carrier n77 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.45.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.45.3.1	Power class 2 case a, b, c and d

The MSD is same for case a, b ,c and d.

Power class 3 MSD for UL CA_n77A-n79A:

Table 6.45.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD31, 2 ¦2*fBn77-fBn79¦ |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD3 increases by 3*3=9dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.45.3.1-2.

Table 6.45.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | FDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 24.2 | TDD | IMD31, 2 ¦2*fBn77-fBn79¦ |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.45.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.46	DL CA_n28-n77-n79, UL CA_n28A-n77A

### 6.46.1	Configurations

Table 6.46.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A7 CA_n28A-n79A CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A7 CA_n28A-n79A CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.46.2	Maximum output power

Table 6.46.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n77 | CA power class | Carrier n28 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.46.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.46.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n28A-n77A into CA n28-n77-n79, none is proposed for PC2.

#### 6.46.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n28A-n77A into CA n28-n77-n79, none is proposed for PC2.

### 6.46.4	Void

## 6.47	DL CA_n28-n77-n79, UL CA_n28A-n79A

### 6.47.1	Configurations

Table 6.47.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A7 CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A7 CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.47.2	Maximum output power

Table 6.47.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.47.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.47.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n28A-n79A into CA n28-n77-n79, none is proposed for PC2.

#### 6.47.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n28A-n79A into CA n28-n77-n79, none is proposed for PC2.

### 6.47.4	Void

## 6.48	DL CA_n28-n77-n79, UL CA_n77A-n79A

### 6.48.1	Configurations

Table 6.48.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A7 | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A7 | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.48.2	Maximum output power

Table 6.48.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n77-n79 | CA power class | Carrier n77 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.48.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.48.3.1	Power class 2 case a, b, c and d

The MSD is same for case a, b ,c and d.

Power class 3 MSD for UL CA_n77A-n79A:

Table 6.48.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD21, 2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD2 increases by 3*2=6dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.48.3.1-2.

Table 6.48.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 22.7 | FDD | IMD21, 2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.48.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.49	DL CA_n3-n28-n79, UL CA_n3A-n79A

### 6.49.1	Configurations

Table 6.49.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n79A | CA_n3A-n79A7 CA_n28A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n28 | n28 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.49.2	Maximum output power

Table 6.49.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n79 | CA power class | Carrier n3 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.49.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.49.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n3A-n79A:

Table 6.49.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | FDD | IMD4 ¦3*fBn3-fBn79¦ |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD4 increases by 3*4=12dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.49.3.1-2.

Table 6.49.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 21.5 | FDD | IMD4 ¦3*fBn3-fBn79¦ |

### 6.49.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.50	DL CA_n3-n28-n79, UL CA_n28A-n79A

### 6.50.1	Configurations

Table 6.50.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n79A | CA_n3A-n79A CA_n28A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n28 | n28 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.50.2	Maximum output power

Table 6.50.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.50.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.50.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n28A-n79A:

Table 6.50.3.1-1 Power class 3 MSD for 2 bands UL CA_n28A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 ¦4*fBn28-fBn79¦ |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD5 increases by 3*5=15dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.50.3.1-2.

Table 6.50.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1870 | 20.5 | FDD | IMD5 ¦4*fBn28-fBn79¦ |

### 6.50.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.51	CA_n1-n28-n41

### 6.51.1	Configurations

Table 6.51.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n41A | CA_n1A-n41A7 CA_n28A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.51.2	Maximum output power

Table 6.51.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.51.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.51.3.1	Power class 2 case a, b

Based on calculation, IMD2/5 of dual UL CA_n1A-n41A falls into n28 DL and no IMD issue for dual UL CA_n28-n41 falls into n1 DL, the MSD exceptions are defined as below

Table 6.51.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n41 | n1 | 1923 | 5 | 25 | 2113 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 762 | 36.6 | FDD | IMD21 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.51.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.52	CA_n18-n28-n77

### 6.52.1	Configurations

Table 6.52.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n28A-n77A | n77A7 CA_n18A-n77A7 CA_n28A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.52.2	Maximum output power

Table 6.52.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.52.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.52.3.1	Power class 2 case a, b

Based on calculation, IMD5 and IMD6 of dual UL CA_n18A-n77A fall into n18 DL and IMD5 and IMD7 of dual UL CA_n28A-n77A falls into n28 DL, the same as the two band cases. For UL CA_n18A-n77A MSD exceptions are defined [4-5] and are reflected as Table 6.52.3-1. For DL and UL CA_n28A-n77A, and victim n28, the MSD exceptions are already captured in the specification [1]. Hence the necessary MSD values for the two bands fallback cases are available, only 3 bands case need to be investigated.

Based on calculation, IMD5 of dual UL CA_n18A-n77A falls into n28 DL and IMD5 and IMD7 of dual UL CA_n28A-n77A falls into n18 DL, the MSD exceptions are defined as in yellow in Table 6.52.3-1. The MSD exception for UL CA_n18A-n77A falls into n28 DL is borrowed from Table 5.60.3-1 of 3GPP TR 38.898 V0.6.0 (2023-10)[6].

Table 6.52.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n28-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 17.5 | FDD | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 12.1 | FDD | IMD5 |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n77 | 3757 | 10 | 52 | 3757 | N/A | TDD | N/A |

### 6.52.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.53	CA_n3-n18-n77

### 6.53.1	Configurations

Table 6.53.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n18A-n77 | n77A7 CA_n3A-n77A7 CA_n18A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77(2A) | n77A7 CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.53.2	Maximum output power

Table 6.53.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.53.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

The MSD exceptions to n3 downlink and n18 downlink due to single UL n77 need not to defined because it drops no IMD and no harmonic mixing into the n3 or n18 downlink.

#### 6.53.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n18A-n77A falls into n3 DL and IMD6 and 7 of dual UL CA_n3A-n77A falls into n18 DL respectively, IMD6 and IMD7 interferences are lower than REFESENS, so we don’t need to define the MSD for IMD6 and IMD7. The MSD exceptions due to CA_n18A-n77A falls into n3 DL are defined as below.

Table 6.53.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n18-n77 | n18 | 820 | 5 | 25 | 865 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1865 | 24.2 | FDD | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.53.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA

# 7	Power class 2 CA with SUL

## 7.x	CA_nW-nX_SUL_nY-nZ

### 7.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5C-4 unless additional clarification>

Table 7.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink configuration | NR Band | Channel Bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nW-nX_SUL_nY-nZ | SUL_nY-nZ | nW |  |  |
|  |  | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |

### 7.x.2	Maximum output power

### 7.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

### 7.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-11 | RAN4 #105 | R4-2219272 |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4#105 | R4-2220844 |  |  |  | The following TPs are approved: R4-2220470 TP for HPUE CA_n77-n79 with 1UL for TR 38.899, NTT DOCOMO Inc R4-2220471 TP for HPUE CA_n78-n79 with 1UL for TR 38.899, NTT DOCOMO Inc R4-2220472 TP for TR38.899 for DL CA_n25-n41-n66 with PC2 2BUL, T-Mobile USA R4-2220004 TP for TR38.899 for DL CA_n25-n41-n71 with PC2 2BUL, T-Mobile USA R4-2220475 TP for TR38.899 for DL CA_n25-n41-n77 with PC2 2BUL, T-Mobile USA R4-2220476 TP for TR38.899 for DL CA_n25-n66-n77 with PC2 2BUL, T-Mobile USA R4-2220477 TP for TR38.899 for DL CA_n25-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220008 TP for TR38.899 for DL CA_n41-n66-n71 with PC2 2BUL, T-Mobile USA R4-2220478 TP for TR38.899 for DL CA_n41-n66-n77 with PC2 2BUL, T-Mobile USA R4-2220479 TP for TR38.899 for DL CA_n41-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220480 TP for TR38.899 for DL CA_n66-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220012 TP for TR38.899 for DL CA_n25-n41C with PC2 UL CA_n41C, T-Mobile USA R4-2220013 TP for TR38.899 for DL CA_n41C-n66 with PC2 UL CA_n41C, T-Mobile USA R4-2220014 TP for TR38.899 for DL CA_n41C-n71 with PC2 UL CA_n41C, T-Mobile USA R4-2220015 TP for TR38.899 for DL CA_n41C-n77 with PC2 UL CA_n41C, T-Mobile USA | 0.1.0 |
| 2023-03 | RAN4#106 | R4-2300720 |  |  |  | The following TPs are approved: R4-2303453 TP for HPUE CA_n3-n41 with 1UL for TR 38.899, Samsung, KDDI R4-2303456 TP for HPUE CA_n3-n77 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2303528 TP for TR38.899 PC2 CA_n8A-n78A, ZTE Corporation, China Unicom, CHTTL R4-2303454 TP for HPUE CA_n40-n77 with 1UL for TR 38.899, Samsung, KDDI R4-2301130 TP for HPUE CA_n3-n28-n41 with 2UL for TR 38.899, Samsung, KDDI R4-2301132 TP for HPUE CA_n3-n41-n77 with 2UL for TR 38.899, Samsung, KDDI | 0.2.0 |
| 2023-04 | RAN4#106bis-e | R4-2304522 |  |  |  | The following TPs are approved: R4-2305020 TP for HPUE CA_n28-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2305018 TP for HPUE CA_n3-n28-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2305019 TP for HPUE CA_n28-n41-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm | 0.3.0 |
| 2023-05 | RAN4#107 | R4-2307920 |  |  |  | The following TPs are approved: R4-2308074 TP to TR 38.899 Addition of CA_n2-n48-n77, Nokia, Verizon R4-2308075 TP to TR 38.899 Addition of CA_n5-n48-n77, Nokia, Verizon R4-2308076 TP to TR 38.899 Addition of CA_n48-n66-n77, Nokia, Verizon R4-2308139 TP for HPUE CA_n1-n41 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2308140 TP for HPUE CA_n1-n77 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2309379 TP for HPUE CA_n2A-n5A-n77C with 2UL for TR 38.899, Samsung, Verizon, Nokia | 0.4.0 |
| 2023-08 | RAN4#108 | R4-2313487 |  |  |  | The following TPs are approved: R4-2311967 TP for HPUE CA_n1-n3-n41 with 2UL for TR 38.899, Samsung, KDDI R4-2313322 TP for 38.899 adding HPUE for CA_n25-n78, Ericsson, Bell Mobility R4-2313330 TP for 38.899 adding HPUE for CA_n66-n78, Ericsson, Bell Mobility R4-2313324 TP for 38.899 adding HPUE for CA_n7-n66-n78, Ericsson, Bell Mobility R4-2314637 TP for 38.899 adding HPUE for CA_n7-n77, Ericsson, Bell Mobility R4-2314638 TP for 38.899 adding HPUE for CA_n25-n66-n78, Ericsson, Bell Mobility R4-2314639 TP for adding UL CA_n77(2A) to DL CA_n1A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314640 TP for adding UL CA_n77(2A) to DL CA_n3A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314689 TP for adding UL CA_n77(2A) to DL CA_n28A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314901 TP for TR38.899: CA_n77-n79, CA_n3-n77, CA_n28-n77, CA_n1-n77, CA_n1-n79, CA_n8-n79, SoftBank, LG Electronics | 0.5.0 |
| 2023-10 | RAN4#108bis | R4-2316479 |  |  |  | The following TPs are approved: R4-2315470 TP for TR 38.899 to include HPUE CA_n13-n77 Samsung, TELUS, Bell Mobility R4-2316418 TP for 38.899 adding non-contiguous UL to CA_n7-n77 Ericsson, Bell Mobility, Huawei, HiSiicon R4-2316419 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n5-n77 Ericsson, Bell Mobility R4-2316420 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n66-n77 Ericsson, Bell Mobility R4-2316421 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n25-n77 Ericsson, Bell Mobility R4-2317581 TP for TR 38.899 to include HPUE CA_n5-n7-n77 Samsung, TELUS, Bell Mobility R4-2317582 TP for TR 38.899 to include HPUE CA_n7-n25-n77 Samsung, TELUS, Bell Mobility R4-2317584 TP for 38.899 adding HPUE CA_n5-n25-n77 Ericsson, Bell Mobility R4-2317585 TP for 38.899 adding HPUE CA_n5-n25-n78 Ericsson, Bell Mobility R4-2317676 TP for 38.899 to include HPUE CA_n7-n66-n77 Nokia, TELUS, Bell Mobility R4-2317677 TP for 38.899 to include HPUE CA_n7-n71-n77 Nokia, TELUS, Bell Mobility R4-2317678 TP for 38.899 to include HPUE CA_n13-n25-n77 Nokia, TELUS, Bell Mobility | 0.6.0 |
| 2023-11 | RAN#109 | R4-2320676 |  |  |  | The following TPs are approved: R4-2319769 TP for TR 38.899 to include HPUE CA_n71-n78 Samsung, TELUS, Bell Mobility R4-2319771 TP for HPUE CA_n1-n3-n77 with 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2320313 TP for 38.899 adding PC2 non-contiguous UL to CA_n5-n78 Ericsson, Bell Mobility R4-2321704 TP for TR38.899 PC2 NRCA for FR1 2DL 1or2UL and PC1.5 NRCA for FR1 2DL and 1UL SoftBank, LG Electronics R4-2321705 TP for HPUE CA_n1-n28-n41 with 2UL for TR 38.899 Samsung, KDDI, LGE R4-2321707 TP for adding UL CA_n77(2A) to DL CA_n3A-n28A-n77(2A) for PC2 HPUE in TR 38.899 Samsung, KDDI, Qualcomm R4-2321708 TP for PC2 HPUE CA_n18-n41 with 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2321709 TP for PC2 HPUE CA_n18-n77 with 2UL in TR 38.899 KDDI, Samsung, LGE, Murata, Skyworks R4-2321710 TP for PC2 HPUE CA_n18-n28-n77 with 2UL in TR 38.899 KDDI, Samsung, Murata R4-2321978 TP for HPUE CA_n3-n18-n77 with 2UL in TR 38.899 KDDI, Samsung, LGE | 0.7.0 |