3GPP TSG RAN WG4 Meeting #109	R4-2320685

Chicago, USA, November 13-17, 2023

Agenda Item:	8.33.4

Source: 	Ericsson

Title:	Analysis of performance requirements for mIAB-MT

Document for:	Discussion

# Introduction

The RRM core requirements for the mIAB-MT were completed in the last meeting in terms of the formal Big CR [1].

RAN4 needs to analyze and identify necessary performance requirements for mIAB-MT, which will include the side conditions and necessary RRM.

In this contribution we provide an initial analysis for defining the necessary RRM performance requirements for mIAB-MT.

# RRM test cases for mIAB-MT

## Summary of core requirements

The following set of core requirements are defined for mIAB-MT as captured in the endorsed Big CR [1]:

- RRC connection re-establishment for intra-frequency cell (clause 12.1.1.1A)

- Handover for mIAB-MT (clause 12.1.2A)

- Radio Link Monitoring (clause 12.3A.1)

- Link Recovery Procedure (clause 12.3A.2)

- Active TCI state switching delay (clause 12.3A.3)

- NR intra-frequency measurements (clause 12.4A.2)

- L1-RSRP measurements for Reporting (clause 12.4A.3)

## Testing of core requirements

The existing test cases in annex G.2 of TS 38.174 are defined to verify the following RRM requirements for the legacy IAB-MT i.e. fixed IAB:

- RRC connection re-establishment for intra-frequency cell

- Radio Link Monitoring

- Link Recovery Procedure

The above legacy test cases are performed in AWGN. In our view for mIAB-MT the tests can also be performed in AWGN. Therefore, the existing test cases for the above RRM requirements can be made applicable to also verify the corresponding mIAB-MT RRM core requirements namely 1), 3) and 4).

The new test cases in FR1 and FR2 are needed to verify the remaining mIAB-MT core requirements namely 2), 5), 6) and 7). The new mIAB-MT test cases in FR1 and FR2 can be based on the following existing test cases defined for the UE in annex A of TS 38.133:

- The mIAB-MT handover tests are based on the following UE tests in TS 38.133:

- A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell

- A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell

- A.7.3.1.2	Intra-frequency handover from FR2 to FR2; known target cell

- A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell

- The mIAB-MT active TCI switch test cases are based on the following UE tests in TS 38.133:

- A.7.5.8.1	MAC-CE based active TCI state switch

- A.7.5.8.2	RRC based active TCI state switch

- The mIAB-MT intra-frequency measurement test cases are based on the following UE tests in TS 38.133:

  - A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX

  - A.7.6.1.1	SA event triggered reporting test without gap under non-DRX

- The mIAB-MT intra-frequency measurement test cases are based on the following UE tests in TS 38.133:

  - A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used

  - A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used

# Side conditions for mIAB-MT

The side conditions related to the RRM requirements are defined in annex H of TS 38.174 for the legacy IAB-MT classes i.e. local area (LA) IAB-MT and wide area (WA) IAB-MT.

The conditions in terms of SSB Ês/Iot and minimum SSB_RP and CSI-RS Ês/Iot and minimum CSI-RS_RP (for CSI-RS based L1-RSRP) for the mIAB-MT for the following RRM core requirements need to be defined in annex H of TS 38.174:

-	mIAB-MT RRC connection re-establishment requirements to intra-frequency cell.

-	mIAB-MT measurements on intra-frequency cells.

-	SSB based L1-RSRP measurement reporting by mIAB-MT.

- CSI-RS based L1-RSRP measurement reporting by mIAB-MT.

According to the RF session agreement in their WF [2], the LA IAB-MT requirements also apply to mIAB-MT:

- Add a new section in TS 38.174 to state that requirements for mIAB-MT, unless otherwise stated, follows Local area IAB-MT requirements.

Therefore, based on the above RF agreements, the conditions for the mIAB-MT can be based on those defined for the local area (LA) IAB-MT. More specifically, the conditions can be defined as follows:

- The mIAB-MT RRC connection re-establishment requirements for intra-frequency cell are applicable down to SSB Ês/Iot = -6 dB. Therefore, the existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can be made applicable to mIAB-MT connection re-establishment requirements for intra-frequency cell for all the three IAB classes: 1-H, 1-O and 2-O classes.

- The mIAB-MT measurements for intra-frequency cells are also applicable down to SSB Ês/Iot = -6 dB. Therefore, the existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can also be made applicable to mIAB-MT measurements for intra-frequency cells for all the three IAB classes: 1-H, 1-O and 2-O classes.

- SSB/CSI-RS based L1-RSRP measurement reporting by mIAB-MT also applicable down to SSB Ês/Iot = -3 dB or CSI-RS Ês/Iot = -3 dB. Therefore, the existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can be adapted by increasing the minimum SSB_RP level by 3 dB for all the three IAB classes: 1-H, 1-O and 2-O classes.

# Summary

In this paper we have analyzed the RRM performance requirements including RRM test cases and side conditions for mIAB-MT RRM requirements. The main observation and proposals are as follows:

NCR-MT test case scenarios:

- Proposal 1: The existing test cases defined in annex G.2.1.1.1.2, G.2.3.1 and G.2.3.2 of TS 38.174 for the legacy IAB-MT are made applicable to verify the following RRM core requirements for the mIAB-MT, respectively:

- RRC connection re-establishment for intra-frequency cell (clause 12.1.1.1A)

- Radio Link Monitoring (clause 12.3A.1)

- Link Recovery Procedure (clause 12.3A.2)

- Proposal 2: The new test cases are defined to verify the following RRM core requirements for the mIAB-MT:

- Handover for mIAB-MT (clause 12.1.2A)

- Active TCI state switching delay (clause 12.3A.3)

- NR intra-frequency measurements (clause 12.4A.2)

- L1-RSRP measurements for Reporting (clause 12.4A.3)

- Proposal 3: The new mIAB-MT test cases in FR1 and FR2 in proposal 2 are based on the following existing test cases defined for the UE in annex A of TS 38.133:

- The mIAB-MT handover tests are based on the following UE tests in TS 38.133:

- A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell

- A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell

- A.7.3.1.2	Intra-frequency handover from FR2 to FR2; known target cell

- A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell

- The mIAB-MT active TCI switch test cases are based on the following UE tests in TS 38.133:

- A.7.5.8.1	MAC-CE based active TCI state switch

- A.7.5.8.2	RRC based active TCI state switch

- The mIAB-MT intra-frequency measurement test cases are based on the following UE tests in TS 38.133:

  - A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX

  - A.7.6.1.1	SA event triggered reporting test without gap under non-DRX

- The mIAB-MT intra-frequency measurement test cases are based on the following UE tests in TS 38.133:

  - A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used

  - A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used

mIAB-MT side conditions:

- Observation 1: Side conditions in terms of SSB Ês/Iot and minimum SSB_RP and CSI-RS Ês/Iot and minimum CSI-RS_RP (for CSI-RS based L1-RSRP) for the mIAB-MT for the following RRM core requirements need to be defined in annex H of TS 38.174:

-	mIAB-MT RRC connection re-establishment requirements to intra-frequency cell.

-	mIAB-MT measurements on intra-frequency cells.

-	SSB based L1-RSRP measurement reporting by mIAB-MT.

- CSI-RS based L1-RSRP measurement reporting by mIAB-MT.

- Observation 2: According to the RF session agreement the LA IAB-MT requirements also apply to mIAB-MT unless otherwise stated.

- Observation 3: Applicable SSB Ês/Iot for intra-frequency and inter-frequency RRC connection re-establisment requirements for NCR-MT are -6 dB and -4 dB respectively.

- Observation 4: SSB/CSI-RS based L1-RSRP measurement reporting by mIAB-MT also applicable down to SSB Ês/Iot = -3 dB or CSI-RS Ês/Iot = -3 dB..

- Proposal 4: The existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can be made applicable to mIAB-MT connection re-establishment requirements for intra-frequency cell for all the three IAB classes: 1-H, 1-O and 2-O classes.

- Proposal 5: The existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can also be made applicable to mIAB-MT measurements for intra-frequency cells for all the three IAB classes: 1-H, 1-O and 2-O classes.

- Proposal 6: The existing conditions for RRC connection re-establishment for intra-frequency cell for LA IAB-MT can be adapted by increasing the minimum SSB_RP level by 3 dB for all the three IAB classes: 1-H, 1-O and 2-O classes.

A draft CR defining side conditions for mIAB-MT RRM requirements is provided in [3].

# References

- R4-2317373, Draft Big CR to TS 38.174 on RRM core requirements for NR Mobile IAB, Qualcomm

- R4-2316911, WF for mobile IAB RF requirements and co-existence study, Qualcomm.

- R4-2320686, Draft CR on conditions for mIAB-MT in Annex H of TS 38.174, Ericsson.