3GPP TSG-RAN4 Meeting #109		R4-2320691

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on interruption requirements for multipath relay operation under SL-DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define requirements for interruption in multi-path relay scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are defined for the UE supporting SL relay operation as agreed in the approved WF in R4-2310059: - Requirement on interruption caused by the remote UE on direct path (PCell) due to SL-DRX operation on indirect path (U2N relay) in multi-path relay scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of the remote UE in in multi-path relay scenario cannot be guaranteed. The remote may cause unpredictable interruption during SL-DRX operation wrt the relay UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.x1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 12.7.x1	Interruptions to WAN by remote UE due to SL-DRX transitions in multi-path relay scenario

This clause contains the requirements related to the interruptions on the PCell/serving cell on the direct path caused by a remote UE in a multi-path relay scenario due to transitions between active and non-active times during SL-DRX in multi-path relay scenario [2].

Interruption on PCell/serving cell due to transitions between active and non-active times during SL-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured SL-DRX cycle is 640 ms or longer. When multiple SL-DRX cycles are configured, the shortest SL-DRX cycle is applied. Each interruption shall not exceed X slot as defined in table 12.7.x1-1.

Table 12.7.x1-1: Interruption length X at transition between active and non-active during SL-DRX in multi-path relay scenario

|  | NR Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

For SL-DRX active to inactive state transition, when the remote UE is in non-DRX or DRX on the PCell/serving cell of the direct path and the remote UE is in sidelink resource allocation mode 2 with the relay UE on the indirect path, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

In addition, for SL-DRX active to inactive state transition, when the UE is in non-DRX or DRX on the PCell/serving cell of the direct path and the remote UE is in sidelink resource allocation mode 2 with the relay UE on the indirect path and SL DRX cycle is less than 320 ms, the interruptions in this clause shall not apply when one of the following conditions is met:

-	T310 timer is running for RLF on PCell

-	performing candidate beam detection on PCell/serving cell as specfied in section 8.5.5. and 8.5.6.

----------------------END OF CHANGE----------------------------