3GPP TSG-RAN4 Meeting #109		R4-2320697

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3877 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_pos_enh] TA validation requirements for positioning in RRC inactive state in Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define TA validation requirements for SRS transmission configured for positioning measurements involving SRS transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR defines requirements for the UE to validate the TA before each SRS transmission when configured for positioning measurements (e.g. UE Rx-Tx time difference) in RRC inactive state. The TA validation procedure is defined in the following RAN2 specifications since Rel-17 but the corresponding RRM requirements are missing in TS 38.133: - TS 38.321: - 5.26.2 TA validation for SRS transmission in RRC_INACTIVE - TS 38.331: - 5.7.17 Derivation of pathloss reference for TA validation of SRS for Positioning transmission and CG-SDT in RRC_INACTIVE. The TA validation procedure is similar to the TA validation procedure used for CG-SDT transmission in SDT. Therefore, the TA validation requirements for SRS transmission are based on the existing TA validation requirements defined for CG-SDT in clause 5.5.3. - NOTE: Rel-18 cat-A CR is not provided since all the Rel-17 and Rel-18 TA validation requirements are covered by another draft CR under Rel-18 WI (NR_pos_enh2-Core). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning measurement accuracy involving SRS transmission is degraded and SRS transmission if the TA is invalid will cause interference at the BS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1B, 5.6.1B.1, 5.6.1B.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 5.6.1B	TA validation requirements for positioning measurements

#### 5.6.1B.1	Introduction

The requirements in clause 5.6.1B shall apply when the UE is configured with the SRS transmission on the PCell and with inactivePosSRS-RSRP-ChangeThreshold for the time alignment (TA) validation before the SRS transmission [2].

#### 5.6.1B.2	TA validation requirements

When inactivePosSRS-RSRP-ChangeThreshold [2] is configured for TA validation based on the RSRP change criterion according to clause 5.26.2 in [7], the UE is allowed to transmit the SRS using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that the following conditions are met:

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	TA for the SRS transmission is valid according to the TA validation criteria defned in clause 5.26.2 in [7].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.6.1B.2-1 and Table 5.6.1B.2-2 are met for FR1 and FR2-1 respectively.

Table 5.6.1B.2-1 Valid measurement for FR1

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

Table 5.6.1B.2-2 Valid measurement for FR2-1

| Measurement | FR2-1 |
| --- | --- |
| RSRP1 | (T1 – max(480ms, 8*SMTC periodicity)) ≤ T1’ ≤ (T1 + max(480ms, 8*SMTC periodicity)) |
| RSRP2 | (T2 – max(480ms, 8*SMTC periodicity)) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is invalid based on the above conditions, then the UE shall not validate the SRS transmission using RSRP1 and RSRP2 and shall not transmit the SRS. The UE shall not transmit in an SRS resource that occurs more than 640 ms after T2.

Where:

-	T1 is the time when:

-	RRCRelease with SRS-PosRRC-InactiveConfig [2] is received by the UE, or

-	the latest TA is received by the UE while in RRC_INACTIVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.26.2 in [7] for the SRS transmission.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in clause 4.2.2.2.

----------------------END OF CHANGE----------------------------