3GPP TSG-RAN WG4 Meeting #109	R4-2320742

Chicago, USA, November 13 – November 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on NB-IoT Neighbor Cell Measurements in RRC_Connected (36.133) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implementing the section for connected mode measurements for eMTC in NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature will not be captured in specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14A.5 (new) AND 8.14A.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Change #1>

### 8.14A.5	Reserved

### 8.14A.6	NB-IoT neighbour cell measurements

#### 8.14A.6.1	Introduction

This clause contains requirements for the neighbour cell measurements performed by the UE category NB1 in RRC_CONNECTED state. The UE supporting connModeMeasIntraFreq-r17  and/or connModeMeasInterFreq-r17 [31] shall measure neighbour cells when:

the UE supports time-based measurement initiation and the serving cell is configured with t-service. In this case, the UE shall start neighbor cell measurements before t-service; or

the UE supports location-based measurement initiation and the distance between the UE and the serving cell reference location - – referenceLocation ¬– is larger than distanceThresh. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m.

[Editor’s note: Further study whether to introduce Radio criteria for start monitoring of neighbor cells]

If t-serviceStartNeigh is configured for the neighbor cells in a given frequency layer, the UE is not required to initiate measurements in this frequency layer or in neighbor cells associated to this satellite until t-serviceStartNeigh is reached.

The requirement is applicable for both in intra-satellite and inter-satellite neighbor cells, when the UE is provided with satellite assistance information.

The measurement quantities are defined in [4], the measurement model is defined in [22].

[Editor’s Note FFS on whether proper requirements will be defined:

The requirements for intra-frequency neighbour cell measurement when the target carrier is same as serving carrier is defined in clause8.14A.6.2 and are applicable for UEs supporting connModeMeasIntraFreq-r17.

The requirements for inter-frequency neighbour cell measurement when the target carrier is different from serving carrier is defined in clause8.14A.6.3 and are applicable for UEs supporting connModeMeasInterFreq-r17.]