3GPP TSG RAN WG4 Meeting #109	R4-2320763

Chicago, USA, 13 Nov. 2023 – 17 Nov. 2023

Agenda Item:	8.7.2.6

Source: 	Ericsson

Title:	On extending GBBR L1-RSRP to RTD>CP

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remaining open issues in TCI state switching delay requirements for the multi-rx.

# Discussion

In previous meeting following WF [2] was agreed.

  - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI.

    - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

Though we agreed that RAN4 will not consider RTD greater than CP in this release we take a closer look at what really needs to be done.

As part of the multi-Rx WI, RAN4 defined following requirements.

- L1-RSRP requirements for group-based beam reporting

- Scheduling restriction enhancements

- Measurement restrictions enhancements

- TCI state switching requirements

In last meeting RAN4 agreed on not to consider RTD > CP for muti-RX WI as there could be more work required for defining scheduling restriction and measurement restrictions.

However, for L1-RSRP measurement requirements, we agreed that UE uses single panel to derive the group-based beam reporting requirements. Whether it is RTD less than CP or greater than CP, it does not matter as UE do not make simultaneous measurements when making L1-RSRP GBBR measurement. If we do not have L1-RSRP requirements, though UE is capable of RTD>CP, NW cannot schedule data to the UE as L1-RSRP requirements are not specified for RTD >CP. If a Rel-18 UE is in a NW and supports RTD >CP, if NW can configure GBBR for that UE, NW can utilise the capability of the UE.

Even for TCI state switching requirements, NW do not need to always switch two TCI states at the same time.

Since there is no additional work is needed for RTD >CP, L1-RSRP GBBR measurement requirements should be applicable to RTD >CP as single panel is assumed for GBBR computation.

- RAN4 to agree that L1-RSRP GBBR requirements are applicable for RTD >CP

# Summary

The following have been observed and proposed in this contribution.

- RAN4 to agree that L1-RSRP GBBR requirements are applicable for RTD >CP

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2314283, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2), Ericsson.