3GPP TSG RAN WG4 Meeting #109																R4-2320766

Chicago, USA, 13 Nov. 2023 – 17 Nov. 2023

Agenda Item:	8.8.2.2

Source: 	Ericsson

Title:	Discussion on general aspects of the FR2 SCell activation enhancement

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the other enhancements, multiple SCell activation case in R18.

# Discussion

In last meeting UE capabilities were agreed and the following WF is agreed.

Issue 3-1-1a: if option 1 is adopted in issue 3-1-1, discuss the following issues

- FFS:

  - Proposal 1 (CMCC):

    - If it is agreed that the L3 measurement report after SCell activation command, and using SSB periodicity instead of SMTC periodicity are applicable for FR1 SCell activation delay reduction, it is proposed that the related UE capabilities are applied for both FR1 and FR2.

  - Proposal 2 (HW, Apple):

    - For L3 measurement reporting triggered by SCell activation, introduce per UE capability indication.

  - Proposal 3 (OPPO):

    - Two options to define UE capability for FR2 unknown SCell activation enhancement:

      - Option 1: introduce UE capability for support the feature of FR2 unknown SCell activation enhancement at least including new L3 report, enhanced SCell activation delay and faster beam sweeping as one whole package.

      - Option 2: include FR2 unknown SCell activation enhancement as part of R18 RRM enhancement in UE feature list.

  - Proposal 4 (ZTE):

    - such UE capability is common for the single SCell activation, single PUCCH SCell activation and multiple SCell activation.

In last meeting it was agreed that L3 report based SCell activation enhancement can be applied to FR1 also. Hence the UE capability of sending L3 report after SCell activation command can be applied for FR1 and FR2. Since this is applied for FR1 and FR2, we suggest having this capability as per UE.

- UE capability of sending L3 report after SCell activation command to be introduced as per-UE

Other open issue after last meeting was

Issue 3-1-2a: if option 1 is adopted in issue 3-1-2, discuss the following issues

- FFS

  - Proposal 1:

    - Option 1(MTK, CMCC, CTC, ZTE, Apple): Two beam sweeping factor reduction capability for L3 and L1, respectively, to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

    - Option 2(Ericsson, vivo): Beam sweeping factor reduction to be a single capability and if UE supports this capability, UE should report X1 and X2.

  - Proposal 2 (HW, Apple):

    - For X1 and X2, introduce per band capability indication

Since it is agreed to have X1 and X2 in the same feature group, main open issue is what should be the UE capability type. Since beam sweeping factor reduction is only for FR2, we think it can be introduced as  per-band,

- For X1 and X2, introduce per band capability indication.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction and made following proposals:

- UE capability of sending L3 report after SCell activation command to be introduced as per-UE

- For X1 and X2, introduce per band capability indication.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo

- R4-2317350