3GPP TSG RAN WG4 Meeting #109	R4-2320772

Chicago, USA, 13 Nov. 2023 – 17 Nov. 2023

Agenda Item:	8.14.4

Source: 	Ericsson

Title:	Simulation results for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we provide simulation results for OOS and IS evaluation of RLM for NR in less than 5 MHz bandwidth.

# Discussion

In last meeting, following PDCCH parameters are agreed for RLM or BFD.

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [4] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

Following table for Insync evaluation.

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [2] | [4] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

We have performed simulation based on the above assumptions and the simulation parameters agreed during the core part of the discussion. We present results in the following table.

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8.66 |  | 7.8 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -0.855 |  |

Simulation Assumption:

PDCCH BW: 12 PRBs, Number of COREEST symbols: 2; Carrier freq.: 900 MHz; CORESET bandwidth 12 PRBs and non-punctured. Antenna config: 2*2, low.

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -6.94 |  | 10.04 |
| IS | 4 | TDL-C 300ns 100 Hz |  | 3.1 |  |

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R4-2306244

- R4-2306346

- R4-2306400