3GPP TSG-RAN WG4 Meeting # 109	R4-2320791

Chicago, US, November 13 – 17, 2023

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for UE PDSCH Demodulation for ATG

Agenda item:	8.13.8.2

Document for:	Approval

- Introduction

In this paper, we share SNR simulation results for alignment based on the Agreed Simulation assumptions.

# General

## SNR Results for PDSCH ATG UE Demod Requirements

Observation 1: Alignment SNR considers Tx EVM 3% for MCS Table 2, Tx EVM = 6% for MCS Table 1;

Table 2-1

| Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | Correlation matrix and antenna configuration | Modulation format and code rate | MCS Table | Fraction of maximum throughput (%) | Tx EVM Assumption [%] | Alignment SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 / 15 | na | 2x2 | 13 | 1 | 70 | 6 | 4 |
|  |  |  | 22 | 1 | 70 | 6 | 11.6 |
|  |  |  | 24 | 2 | 70 | 3 | 20.7 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1.1 |
|  |  |  | 22 | 1 | 70 | 6 | 8.7 |
|  |  |  | 24 | 2 | 70 | 3 | 18.2 |
| 40 / 30 | 7D1S2U | 2x2 | 13 | 1 | 70 | 6 | 4 |
|  |  |  | 22 | 1 | 70 | 6 | 11.4 |
|  |  |  | 24 | 2 | 70 | 3 | 20.8 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1 |
|  |  |  | 22 | 1 | 70 | 6 | 8.8 |
|  |  |  | 24 | 2 | 70 | 3 | 18.1 |
|  | 30D4S6U | 2x2 | 13 | 1 | 70 | 6 | 4 |
|  |  |  | 22 | 1 | 70 | 6 | 11.2 |
|  |  |  | 24 | 2 | 70 | 3 | 20.6 |
|  |  | 2x4 | 13 | 1 | 70 | 6 | 1 |
|  |  |  | 22 | 1 | 70 | 6 | 8.8 |
|  |  |  | 24 | 2 | 70 | 3 | 17.8 |

- Conclusions

Observation 1: Alignment SNR considers Tx EVM 3% for MCS Table 2, Tx EVM = 6% for MCS Table 1;

- References