3GPP TSG-RAN WG4 Meeting #109	R4-2320805

# Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Case 1 requirements

Agenda item:	8.9.2.2

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on some open issues for Case 1 requirements as discussed at RAN4 #108bis [2], i.e. the combination of pre-configured MG and concurrent MG and lists corresponding proposals.

Discussion

Collision handling

Collision handling was discussed for following 4 scenarios [2]:

- Scenario 1: the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Scenario 2: pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Scenario 3: pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG.

- Scenario 4: One pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision (This scenario is for Pre-MG + Pre-MG).

At RAN4 #108bis, collision handling was agreed for scenarios 1 to 3 [2]. In particular, if there is a collision between Pre-MG activation / de-activation and concurrent MG occasion, then the gap with the configured lower priority will be dropped. However, there was no consensus for Scenario 4, which treats the case of Pre-MG activation and colliding Pre-MG deactivation. This is treated below.

Overlapping of pre-configured MG de-activation procedure with pre-configured MG activation procedure

The status is summarized in [2][3]:

Issue 2-1-4: [Case 1] - [Scenario 4] When one pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision Moderator’s note: this issue is a mix between an existing issue of fully overlapping activation/deactivation Pre-MG with collision a Pre-MG gap in the concurrent gap with Pre-MG. - Background: - Agreements from fully overlap with activation/deactivation [R4-2310175]: - For Case 1 (Pre-configured MG and multiple concurrent MGs), under the assumption that the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated, extend the delay by T1 ms for fully overlapped simultaneous activation/deactivation for Pre-MG + Pre-MG - T1 = 2ms. - FFS if this activation delay collide with existing gaps - An illustration example is captured below [R4-2306330]: - Agreements from dynamic collision: - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG [and UE should continue the measurement within the MG#2] - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used - The collision scenario in this issue is depicted in the figure below: - Proposals - if fully overlapped simultaneous activation/deactivation for Pre-MG + Pre-MG collides with activated Pre-MG: - Option 1: Apple, Nokia - regarding scenario 4 (when one pre-configured MG deactivation procedure is fully overlapped with another pre-configured MG activation procedure triggered by single BWP switching during the dynamic collision), apply same agreement as in scenario 1 when the Type-2 MG is replaced by an activated pre-MG - Option 2: OPPO - For case a and case b when Pre-MG2 (activate à deactivate) is overlapped with (de)activation procedure, support option 2 to Follow the same agreement as scenario 1; - For case c and case d when Pre-MG1 (deactivate à activate) is overlapped with (de)activation procedure, support option 5 since no gap collision will happen. - Option 3: HW - For scenario 4, follow the same agreement as in scenario 1 and 2 when the Type-2 MG is replaced by an activated pre-MG. Alternatively, RAN4 not to define UE behaviour and requirements for scenario 4. - Option 4: QC, HW, MTK, ZTE - No need to discuss the case when two pre-configured MGs activation procedures are overlapped during the dynamic collision. - Option 5: E///, CATT - When one pre-configured MG(Pre-MG #1) deactivation procedure fully overlaps with another pre-configured MG(Pre-MG #2) activation procedure, no gap dropping rule shall be applied and UE shall perform measurement within each activated Pre-MG. Data scheduling is not expected within the deactivated Pre-MG occasions before and after the Pre-MG activation/deactivation procedure. - Way Forward - Companies to further check whether this scenario can be captured reusing the agreements from Scenarios 1,2, and 3.

In our view, the same proceeding can be applied in scenario 4 as for scenarios 1 to 3, i.e. based on configured priorities for the gaps. This means, in case of collision of Pre-MG activation and Pre-MG deactivation, the gap is activated or deactivated which is assigned the higher priority and the activation/deactivation of the other Pre-MG is postponed by 5ms.

For scenario 4, in case of collision of Pre-MG activation and Pre-MG deactivation, the gap is activated or deactivated which is assigned the higher priority and the activation/deactivation of the other Pre-MG is postponed by 5ms.

UE capability for dynamic collisions

RAN4 #108bis did not get further progress on the issue whether to define a new UE capability for dynamic collisions.

Issue 2-1-5: [Case 1] Whether to define a new UE capability for dynamic collisions? - Way Forward - Option 1: - Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions. - Option 2: - No additional capability is needed to handle the dynamic collision.

Due to the RAN4 #107 agreement to consider only collisions between an activated Pre-MG and a concurrent MG, we do not see a justification to introduce a new UE capability for such dynamic collisions, where Pre-MG status changes between activated and de-activated, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation and collision rules for Pre-MG (de-)activation with concurrent MG are handled in previous issues.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation.

Other requirements

Pre-MG association

RAN4 #108bis did not get further progress on the issue whether the Pre-MG association needs to be clarified.

Issue 2-2-1: [Case 1] Pre-MG association clarification - Way Forward - Option 1: - When NW configures a Pre-MG1 and a Pre-MG2/Type-2 MG in ConMGs, the MO associated with Pre-MG1 will be measured within activated Pre-MG2/Type-2 MG if Pre-MG1 is deactivated and the MO is fully overlapping with activated Pre-MG2/Type-2 MG. - Option 1a: HW - FFS: whether it need to be captured in spec - Option 2: - RAN4 to discuss options related to UE behaviour, for UE supporting Case 1 requirements, in case of deactivated Pre-MG, i.e. require the UE to perform measurements for MO’s assigned to Pre-MG outside any other MG, or define a priority for deactivated Pre-MG to be compared against priority of any other overlapping MG, or define a Pre-MG association rule by transferring MO’s assigned to Pre-MG to any other active MG (Pre-MG or Type-2 MG) as long as Pre-MG is deactivated

We do not support option 1 as it stands, as it just transfers MOs from Pre-MG1 to other Pre-MG2 or Concurrent MO, i.e. it impacts the CSSF within gap in that more measurement objects have to be performed by Pre-MG2 and hence measurement delay increases. In addition, it is unclear how the configured priorities for all MGs are taken into account in such case.

Thus, in case of

- no overlap or partial overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 outside the activated Pre-MG2 (e.g. search for SMTCs outside the MG).

- full overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 in Pre-MG2 and drop all measurements assigned to Pre-MG2, if Pre-MG1 is configured with higher priority than Pre-MG2, or UE is required to perform all measurements assigned to Pre-MG2 in Pre-MG2 and drop all measurements assigned to Pre-MG1, if Pre-MG1 is configured with lower priority than Pre-MG2.

For Pre-MG association with deactivated Pre-MG1 and activated Pre-MG2, RAN4 to distinguish the cases:

- no overlap or partial overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 outside the activated Pre-MG2 (e.g. search for SMTCs outside the MG).

- full overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 in Pre-MG2 and drop all measurements assigned to Pre-MG2, if Pre-MG1 is configured with higher priority than Pre-MG2, or UE is required to perform all measurements assigned to Pre-MG2 in Pre-MG2 and drop all measurements assigned to Pre-MG1, if Pre-MG1 is configured with lower priority than Pre-MG2.

Conclusion

This paper has presented Nokia’s views on Case 1 requirements as discussed at RAN4 #108bis [2], i.e. the combination of pre-configured MG and concurrent MG.

Resulting from this discussion, we make following proposals:

- For scenario 4, in case of collision of Pre-MG activation and Pre-MG deactivation, the gap is activated or deactivated which is assigned the higher priority and the activation/deactivation of the other Pre-MG is postponed by 5ms.

No new UE capability for dynamic collisions is needed for UEs supporting Case 1 requirements, as Rel-17 collision rules based on configured priorities are valid in case of Pre-MG instances after Pre-MG activation.

For Pre-MG association with deactivated Pre-MG1 and activated Pre-MG2, RAN4 to distinguish the cases:

- no overlap or partial overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 outside the activated Pre-MG2 (e.g. search for SMTCs outside the MG).

- full overlap of Pre-MG1 and Pre-MG2: UE is required to perform all measurements assigned to Pre-MG1 in Pre-MG2 and drop all measurements assigned to Pre-MG2, if Pre-MG1 is configured with higher priority than Pre-MG2, or UE is required to perform all measurements assigned to Pre-MG2 in Pre-MG2 and drop all measurements assigned to Pre-MG1, if Pre-MG1 is configured with lower priority than Pre-MG2.

- References

- RP-231477, Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, Intel, MediaTek

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek Inc.

- R4-2317198, Topic summary for [108-bis][206] NR_MG_enh2_part1, Moderator (MediaTek Inc.)