3GPP TSG-RAN WG4 Meeting #109	R4-2320807

# Chicago, USA, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM performance requirements for Pre-MG, concurrent MG and NCSG

Agenda item:	8.9.4

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on RRM performance requirements for Case 1 and Case 2 requirements, i.e., the gap combinations i) concurrent MG with Pre-MG and Pre-MG+Pre-MG and ii) concurrent MG with NCSG and NCSG+NCSG.

Discussion

RAN4 #108bis has discussed test cases for Case 1 and Case 2 requirements and has listed open issues in [2]. These are discussed in the following.

General configuration for Case 1 and Case 2

RAN4 discussed the general configuration for test cases.

Issue 4-1-1: Which general configuration shall be defined for the test cases? - Way forward - Option 1: - Only inter-frequency measurement - Only non-DRX - without SSB time index detection - Option 2: - Only define test case in NR SA in both FR1 and FR2 - Do not introduce the test for L1 impact - Do not introduce test cases for intra-freq measurement without gap - Define a minimum set of test cases for SSB-based measurement - Only define test case under non-DRX - Define test case without SBI reporting. - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations - Do not define test cases with simultaneously FR1 and FR2 gaps configured. - Test cases are limited to single serving carrier - Only use mandatory gap patterns to define test cases - Focus on only fully non-overlp and partially partial overlap in the test case design - Verify gap dropping behaviour without introducing additional test cases

In our view, there are commonalities between both options, however option 2 is more detailed and hence can serve as a baseline.

The following proposal is made.

RAN4 to proceed with option 2 of issue 4-1-1 as baseline for the general configuration.

Topics for test cases related to Case 1

RAN4#108bis discussed topics for test cases related to Case 1:

Issue 4-2-1: [Case 1] Which topics shall RAN4 RRM define test cases for Pre-MG and concurrent MG (Case 1) - Way forward - Option 1: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG and Pre-MG + Type-1 MG. - Option 2: - Pre-MG + Pre-MG, Pre-MG + Type-2 MG. - Option 3: - Concurrent MG and 1 Pre-MG activation / deactivation in same FR - Pre-MGs with activation / deactivation in the same FR. - Option 4: - Pre-MG (de)activation - Pre-MG (de)activation is based on the associated MO or the associated pre-configured status - In non-simultaneous case, the (de)activation delay is same as in Rel-17 - In simultaneous case, the (de)activation delay is extended by 2ms - Collision handling - Deactivated pre-MG is not considered in collision - Dynamic collision including at least scenario 1 and 2

In our view, it is not necessary to test Pre-MG with Type-1 MG, being proposed in Option 1, as it is assumed that both MGs are fully non-overlapping and this is ensured by proper network configuration.

Option 2 and Option 3 are similar. Option 4 distinguishes between simultaneous and non-simultaneous activation/deactivation of the Pre-MGs and in addition proposes to limit the collision handling to scenarios 1 and 2, where Pre-MG has higher priority than concurrent MG. In our view RAN4 should also develop test cases for scenarios 3 and 4. With this modification we can use Option 4 as baseline for the topics.

The following proposal is made.

RAN4 to proceed with option 4 of issue 4-2-1 as baseline for the topics related to Case 1 test cases and specify tests for collision handling for scenarios 1 to 4.

Test case combinations for Case 1

RAN4#108bis discussed test case combinations related to Case 1:

Issue 4-2-2: [Case 1] Test cases combination for Case 1 - Way forward - Option 1: - RAN4 to define Pre-MG and Con-MGs test cases with the following combinations: - Pre-MG Combination 1: Pre-MG+MG without gap collision - Pre-MG Combination 2: Pre-MG+MG with gap collision - Pre-MG Combination 3: (Pre-MG1, Pre-MG2): (Deactivation, Deactivation) <-> (Activation, Activation) without gap collision - Pre-MG Combination 4: (Pre-MG1, Pre-MG2): (Deactivation, Deactivation) <-> (Activation, Activation) with gap collision - Pre-MG Combination 5: (Pre-MG1, Pre-MG2): (Activation, Deactivation) <-> (Deactivation, Activation) without gap collision - RAN4 not to define the test cases to verify the collision during the transition period between Pre-MG activation/deactivation and MG occasion - Option 2: - TC1: pre-MG + pre-MG, autonomous non-simultaneous (de)activation, FR1 - TC2: pre-MG + pre-MG, NW controlled non-simultaneous (de)activation, FR1 - TC3: pre-MG + pre-MG, autonomous simultaneous (de)activation, FR1 - TC4: pre-MG + pre-MG, NW controlled simultaneous (de)activation, FR1 - TC5: pre-MG + pre-MG, autonomous non-simultaneous (de)activation, FR2 - TC6: pre-MG + pre-MG, NW controlled non-simultaneous (de)activation, FR2 - TC7: pre-MG + pre-MG, autonomous simultaneous (de)activation, FR2 - TC8: pre-MG + pre-MG, NW controlled simultaneous (de)activation, FR2 - TC9: pre-MG + type-2 MG, dynamic collision scenario 1, autonomous (de)activation, FR1 - TC10: pre-MG + type-2 MG, dynamic collision scenario 1, NW controlled (de)activation, FR1 - TC11: pre-MG + type-2 MG, dynamic collision scenario 2 autonomous (de)activation, FR1 - TC12: pre-MG + type-2 MG, dynamic collision scenario 2, NW controlled (de)activation, FR1 - TC13: pre-MG + type-2 MG, dynamic collision scenario 1, autonomous (de)activation, FR2 - TC14: pre-MG + type-2 MG, dynamic collision scenario 1, NW controlled (de)activation, FR2 - TC15: pre-MG + type-2 MG, dynamic collision scenario 2 autonomous (de)activation, FR2 - TC16: pre-MG + type-2 MG, dynamic collision scenario 2, NW controlled (de)activation, FR2 - Option 3: - for gap combination, it is propsed to define test for the combination with 2 gaps for UE which does not support per-FR gap, and define test for the combination with 3 gaps cross all FRs for UE which supports per-FR gap - it is proposed to define test to verify the gap collision handling for concurrent gap with Pre-MG and concurrent gap with NCSG - it is proposed to define test for dynamic collision with Pre-MG - Option 4: - 1-1-1 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG for - 1-1-2 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and network-controlled activation/deactivation of Pre-MG for - 1-1-3 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and autonomous activation/deactivation of Pre-MG - 1-1-3 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG

In our view options 2 and 4 are most detailed and hence can form a baseline for Case 1 test cases.

The following proposal is made.

RAN4 to proceed with options 2 and 4 of issue 4-2-2 as baseline for Case 1 test cases.

Rules for Pre-MG + Pre-MG test cases

RAN4#108bis also discussed  Pre-MG + Pre-MG test cases:

Issue 4-2-3: [Case 1] Which general rules shall be defined for the Pre-MG + Pre-MG test cases ? - Way forward - Option 1: - Different test cases for UE autonomous and network-controlled Pre-MG (de)activation mechanisms separately. - Combine the test cases for the measurement reporting with multiple Pre-MG activation/deactivation delay. - The trigger events of Pre-MG activation will be only DCI-based BWP switching

We support option 1.

The following proposal is made.

RAN4 to proceed with option 1 of issue 4-2-3 as baseline for rules for Pre-MG + Pre-MG test cases.

Topics for test cases related to Case 2

RAN4#108bis discussed topics for test cases related to Case 2:

Issue 4-3-1: [Case 2] Which topics shall RAN4 RRM define test cases for NCSG and concurrent MG (Case 2) - Way forward - Option 1: - NCSG + NCSG, NCSG + Type-2 MG and NCSG + Type-1 MG. - Option 2: - NCSG + NCSG, NCSG + Type-2 MG. - Option 3: - 1 Concurrent MG and 1 NCSG in the same FR - 2 NCSGs (non-overlapping) in the same FR - Measurement for deactivated SCells with 1 Concurrent MG (w/o gap or with NCSG, as being discussed for Rel-17 maintenance). - Option 4: - Collision definition - Total NCSG duration, including both the VILs and the ML are considered in the proximity - [Deactivated SCell measurement - Deactivated SCell is measured in NCSG regardless of the gap association]

In our view, it is not necessary to test NCSG with Type-1 MG, being proposed in Option 1, as it is assumed that both MGs are fully non-overlapping and this is ensured by proper network configuration.

Option 2 and Option 3 are again similar. Option 3 mentions in addition the deactivated SCell measurement, which is also considered in Option 4. Thus we consider Option 3 as baseline for the topics.

The following proposal is made.

RAN4 to proceed with option 3 of issue 4-3-1 as baseline for the topics related to Case 2 test cases.

Test case combinations for Case 2

RAN4#108bis discussed test case combinations related to Case 2:

Issue 4-3-2: [Case 2] Test cases combination for Case 2 - Way forward - Option 1: - TC1: Measurement with NCSG on deactivated SCC without collision(NCSG+MG) - TC2: Measurement with NCSG on deactivated SCC with collision(NCSG+MG) - TC3: Inter-frequency measurement with NCSG without collision (NCSG+NCSG) - TC4: Inter-frequency measurement with NCSG with collision (NCSG+NCSG) - TC5: Intra-frequency measurement with NCSG without collision(NCSG+NCSG) - TC6: Intra-frequency measurement with NCSG with collision(NCSG+NCSG) - Option 2: - TC1: NCSG + type-2 MG, with collision, FR1 - TC2: NCSG + NCSG, with collision, FR1 - TC3: NCSG + type-2 MG, with collision, FR2 - TC4: NCSG + NCSG, with collision, FR2 - [TC5: NCSG + type-2 MG, for deactivated SCC measurement, FR1] - [TC6: NCSG + type-2 MG, for deactivated SCC measurement, FR2] - Option 3: - 1-1-1 Event triggered reporting test on intra-frequency in FR1 with concurrent gap and NCSG - 1-1-2 Event triggered reporting test on intra-frequency in FR2 with concurrent gap and NCSG - Option 4: - for gap combination, it is propsed to define test for the combination with 2 gaps for UE which does not support per-FR gap, and define test for the combination with 3 gaps cross all FRs for UE which supports per-FR gap - it is proposed to define test to verify the gap collision handling for concurrent gap with Pre-MG and concurrent gap with NCSG

In our view options 1 and 2 are similar, with option 1 also specifying tests without collision, whilst option 3 addresses the reporting test. Option 4 is more generic. Thus option 1 and option 3 can form a baseline for Case 2 test cases.

The following proposal is made.

RAN4 to proceed with options 1 and 3 of issue 4-3-2 as baseline for Case 2 test cases.

Rules for NCSG + NCSG test cases

RAN4#108bis also discussed NCSG + NCSG test cases:

Issue 4-3-3: [Case 2] Which general rules shall be defined for the NCSG + NCSG test cases ? - Way forward - Option 1: - RAN4 to follow the Rel-17 NCSG test configurations to define NCSG+Con-MGs test. - If a UE supports per-FR NCSG, it only needs to pass test case with per-FR NCSG, otherwise, UE only needs to pass test case with per-UE NCSG. - For FR1 NCSG #0 per-UE gap and NCSG #2 per-FR gap are both tested in inter-frequency sub test cases. For FR2 we test with per-FR NCSG pattern #13. - In NCSG tests for intra-frequency, inter-frequency and inter-RAT, the serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency. Option 2:

We support option 1.

The following proposal is made.

RAN4 to proceed with option 1 of issue 4-3-3 as baseline for rules for NCSG + NCSG test cases.

Conclusion

In the paper, Nokia’s views on open issues related to RRM performance requirements for Case 1 and Case 2 requirements [2], i.e., the gap combinations i) concurrent MG with Pre-MG and Pre-MG+Pre-MG and ii) concurrent MG with NCSG and NCSG+NCSG, are presented. The following proposals are made.

- RAN4 to proceed with option 2 of issue 4-1-1 as baseline for the general configuration.

RAN4 to proceed with option 4 of issue 4-2-1 as baseline for the topics related to Case 1 test cases and specify tests for collision handling for scenarios 1 to 4.

RAN4 to proceed with options 2 and 4 of issue 4-2-2 as baseline for Case 1 test cases.

RAN4 to proceed with option 1 of issue 4-2-3 as baseline for rules for Pre-MG + Pre-MG test cases.

RAN4 to proceed with option 3 of issue 4-3-1 as baseline for the topics related to Case 2 test cases.

RAN4 to proceed with options 1 and 3 of issue 4-3-2 as baseline for Case 2 test cases.

RAN4 to proceed with option 1 of issue 4-3-3 as baseline for rules for NCSG + NCSG test cases.

- References

- RP-231477, Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, Intel, MediaTek

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek Inc.