3GPP TSG-RAN WG4 Meeting # 109						R4-2320836

Chicago, US, November 13 – 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on UAV operating bands

Agenda Item:	9.7.1

Document for:	Discussion

# Introduction

In this contribution we provide discussion on UAV operating bands in relation to the content of CRs for E-UTRA and NR UE specifications.

# Discussion

The NR_UAV and LTE_UAV_enh WIs in [1, 2] were triggered by the ECC Decision (22)07 [3] to address (among others) additional UAV requirements, including OOBE emission limits. The referred ECC decision was treating on the harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710-1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570-2620 MHz harmonised for MFCN.

During drafting phase of related CRs to the E-UTRA specification, it was found that information on the operating bands for UAV operation would be necessary.

Even if not explicitly captured in work item descriptions [1, 2], the ECC (22)07 [3] has clearly stated that ECC Decision provides harmonised technical conditions for the usage of aerial UE for communications based on E-UTRA and NR in the following MFCN harmonised bands:

- 703-733 MHz, i.e. Band 28/n28; NOTE: Band 28/n28 is defined in 703 – 748MHz.

- 832-862 MHz, i.e. Band 20/n20,

- 880-915 MHz, i.e. Band 8/n8,

- 1710-1785 MHz, i.e. Band 3/n3,

- 1920-1980 MHz, i.e. Band 1/n1,

- 2500-2570 MHz, i.e. Band 7/n7,

- 2570-2620 MHz, i.e. Band 38/n38.

Therefore, the following is proposed:

Proposal 1: Capture in TS 36.101 and TS 38.101-1 that the UAV operation is allowed in the following MFCN harmonised bands in ECC countries:

- Band 1/n1,

- Band 3/n3,

- Band 7/n7,

- Band 8/n8,

- Band 20/n20,

- Band 28/n28, restricted to 703-733 MHz operation.

- Band 38/n38.

# Conclusions

Based on analysis, the following proposals were formulated:

Proposal 1: Capture in TS 36.101 and TS 38.101-1 that the UAV operation is allowed in the following MFCN harmonised bands in ECC countries:

- Band 1/n1,

- Band 3/n3,

- Band 7/n7,

- Band 8/n8,

- Band 20/n20,

- Band 28/n28, restricted to 703-733 MHz.

- Band 38/n38.

The above proposal applies to NR_UAV, as well as to the LTE_UAV_enh WI.

# References

[1]	RP-230782	NR support for UAV (Uncrewed Aerial Vehicles), NR_UAV, WID

[2]	RP-230783	Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles), LTE_UAV_enh, WID

[3]	ECC Decision (22)07 	Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710-1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570-2620 MHz harmonised for MFCN, ECC