3GPP TSG-RAN WG4 Meeting # 109									          R4-2320868

Chicago, USA, November 13 – 17, 2023

Title: 	Improved table template for 1UL/CC and 2UL/CC MSD studies

Source: 	Skyworks Solutions Inc.

Agenda Item:	8.1.3	Simplification of specification and reduction of test burden [FS_SimBC]

Document for:	Approval

# 1	Background

In RAN4#108b, a new table template covering all 1UL/CC and 2UL/CC MSD studies was addressed in [1], it also proposed a MSD investigation report table. In this contribution, we provide additional details and a way forward to introduce these tables in the TP to TR templates for inter-band NRCA and ENDC.

# Discussion

## 2.1 Proposed template covering MSDs for 1UL(1CC) and IMDs for 2UL(1CC per band)

Based on the discussion and questions raised about which cases should be investigate for harmonic mixing and cross band isolation, we submit an update of tabular calculations used for MSD studies covering UL harmonics, harmonic mixing and cross-band isolation MSDs for 1UL(1CC) and IMDs for 2UL(1CC per band). This update can be added to the already well-used IMD calculation table in TP to TR for 2 and 3 band DL with 2 band UL and 1CC per UL band. For the two band DL and 1UL/1CC case, additional rows are added to provide calculations for harmonic ranges, harmonic mixing ranges, and cross-band ACLR ranges.

A preliminary version of such update is presented in Table 2 with different sections across rows with, in top-to-bottom order:

- The input frequencies and maximum UL channel BW section provide:

  - The two band names and types

  - The two bands UL frequencies

  - The two bands DL frequencies

  - The duplex distance (0 for TDD) and the maximum UL channel bandwidth per band

  - A preliminary input on near miss ranges. This input still requires additional work but uses the maximum UL channel bandwidth per band.

- The UL harmonics frequencies section is to be used for the 2DL band TP only for 1UL harmonic calculations. This provides the H2 to H5 harmonic ranges

- The DL harmonics frequencies section is to be used for 2DL band TP only for 1UL harmonic mixing calculations. This provides the H2 to H5 harmonic ranges

- The cross band ACLR range section is to be used for 2DL band TP only for cross band calculations. This provides the ACLR1 to 4, based on the maximum UL channel bandwidth per band. Note that this approach may also be applicable to intra-band CA cases, and subsequently, a specific template developed.

- The typical IMD frequency range calculations section is to be used for two and three DL bands with 2UL bands with 1CC per band TPs.

- Additionally, a notes section is provided for guidance on the cases that should be investigated, along with other pertinent details. This section requires additional work to integrate as many guidance and rules as deemed necessary.

- This table is populated with the CA_n18-n77 case as an example.

Table 2: Template for 1UL(1CC) and 2UL(1CC per band) frequency analysis

| UE carriers |  | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- | --- |
| Band number / type |  | n18 | FDD | n77 | TDD |
| Input frequencies2 | UL | 815 | 830 | 3400 | 4200 |
|  | DL | 860 | 875 | 3400 | 4200 |
|  | near miss | 760 | 975 | 3385 | 4215 |
| Duplex distance / Maximum UL CBW |  | 45 | 15 | 0 | 100 |
| UL harmonics frequencies | H2 | 1630 | 1660 | 6800 | 8400 |
|  | H3 | 2445 | 2490 | 10200 | 12600 |
|  | H4 | 3260 | 3320 | 13600 | 16800 |
|  | H5 | 4075 | 4150 | 17000 | 21000 |
| DL harmonics frequencies | H23 | 1720 | 1750 | 6800 | 8400 |
|  | H34 | 2580 | 2625 | 10200 | 12600 |
|  | H45 | 3440 | 3500 | 13600 | 16800 |
|  | H56 | 4300 | 4375 | 17000 | 21000 |
| Cross band ACLR range | ACLR1 | 845 | 890 | 3300 | 4300 |
|  | ACLR2 | 830 | 905 | 3200 | 4400 |
|  | ACLR3 | 815 | 920 | 3100 | 4500 |
|  | ACLR47 | 800 | 935 | 3000 | 4600 |
| 2nd order IMD8 | products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
|  | frequency | 2570 | 3385 | 4215 | 5030 |
| 3rd order IMD8 | products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
|  | frequency | 1740 | 2570 | 5970 | 7585 |
|  | products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
|  | frequency | 5030 | 5860 | 7615 | 9230 |
| 4th order IMD8 | products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
|  | frequency | 910 | 1755 | 9370 | 11785 |
|  | products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
|  | frequency | 5845 | 6690 | 11015 | 13430 |
|  | products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
|  | frequency | 5140 | 6770 | 8430 | 10060 |
| 5th order IMD8 | products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
|  | frequency | 12770 | 15985 | 80 | 940 |
|  | products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
|  | frequency | 14415 | 17630 | 6660 | 7520 |
|  | products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
|  | frequency | 8540 | 10970 | 4310 | 5955 |
|  | products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
|  | frequency | 11830 | 14260 | 9245 | 10890 |
| Note 1: All frequencies are in MHz |  |  |  |  |  |
| Note 2: Region or country-based frequency restriction can be applied if the band combination is identifiable to a single region or country |  |  |  |  |  |
| Note 3: UL1/DL2 harmonic mixing collisions should be investigated for DL bands > 1.5GHz, UL3/DL2 may be considered for DL bands >3GHz |  |  |  |  |  |
| Note 4: UL1or2or3/DL3 harmonic mixing collisions should be investigated |  |  |  |  |  |
| Note 5: UL1/DL4 harmonic mixing collisions should be investigated for DL bands > 0.5GHz and UL2or3/DL4 for bands > 1.5GHz. |  |  |  |  |  |
| Note 6: UL1or2/DL5 harmonic mixing collisions should be investigated |  |  |  |  |  |
| Note 7: In cases where ACLR3 range does not overlap with the other band DL range, there may still be a residual MSD due to transmitter noise floor for low filter rejection |  |  |  |  |  |
| Note 8: The IMD section is needed for 2UL case only, only the lowest even and odd order overlapping a DL band should be specified |  |  |  |  |  |

To ease a systematic review of the potential MSD cases, another table is added to the TP wherein each MSD source is discussed, and the possible need for a specification is reported. Table 3 is a preliminary version populated with the CA_n18-n77 MSD sources analysis.

Table 3: Template for 1UL(1CC) and 2UL(1CC per band) MSD analysis

| 1UL harmonics | UL4 and UL5 of band n18 fall into DL1 of band n77 |
| --- | --- |
| 1UL Harmonic mixing | DL4 of n18 overlaps with UL1 of n77, MSD should be investigated as it a DL4 of a band <1GHz |
| 1UL cross band | no cross band range overlaps with the other DL band and the distance is large so no noise floor related MSD should be investigated |
| 2UL IMDs | IMD2 falls in band n77 DL but can be ignored as it is a TDD band IMD4 falls in band n18 DL and should be specified IMD5 falls in band n18 DL and should be specified |
| Note 1: For MSD analysis, any harmonic or IMD product overlapping a DL band or DL harmonic overlapping an UL band or its harmonics should be investigated and commented in this Table. For cases where there is no overlap but the product falls within the maximum UL CBW distance of a DL band, near miss MSD should be considered for low order products. |  |

Proposal:

- The two band and three band DL TP to TR templates for NRCA and ENDC are updated to add the 1UL harmonic, harmonic mixing, and cross-band isolation calculations tables with notes provided for guidance.

- An associated MSD analysis reporting table is also introduced in the TP to TR templates.

- Example tables provided in the contribution can be used as a starting point

- Harmonic mixing rules for investigation can be revised based on agreements.

- The cross band section can be reused for intra-band cases as up to IMD9 cases are evaluated.

- The objective is to introduce new templates for use in Release 19.

# Conclusion

In this contribution, we proposed a new template for the MSD studies for 1UL/CC and 2UL/CC inter-band combinations and a analysis table to promote and acquire more consistent reporting of the MSD investigations.

Proposal:

- The two band and three band DL TP to TR templates for NRCA and ENDC are updated to add the 1UL harmonic, harmonic mixing, and cross-band isolation calculations tables with notes provided for guidance.

- An associated MSD analysis reporting table is also introduced in the TP to TR templates.

- Example tables provided in the contribution can be used as a starting point

- Harmonic mixing rules for investigation can be revised based on agreements.

- The cross band section can be reused for intra-band cases as up to IMD9 cases are evaluated.

- The objective is to introduce new templates for use in Release 19.

# References

[1] R4-2316859 On UL1-DL4 harmonic mixing and table templated for 1UL-CC and 2UL-CC MSD studies, Skyworks Solutions Inc., RAN4#108b