# 3GPP TSG-RAN WG4 Meeting #109	R4-2320872

# Chicago, USA, 13– 17 Nov, 2023

Source: 	T-Mobile USA

Title:	TP for TR 38.880: Input for CA_n41A-n66A

Agenda Item:	7.29.2

Document for:	Approval

- Introduction

This contribution provides a text proposal for CA_n41A-n66A 3Tx with 2Tx on n41 for TR 38.880.

# Text Proposal

<< Beginning of Text Proposal >>

## 5.6	CA_n41A-n66A

### 5.6.1	Operating bands for CA

Table 5.6.1-1:  CA band combination of band n41+n66

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n41-n66 | n41, n66 |

### 5.6.2	Maximum output power for inter-band CA

Table 5.6.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n66A |  |  | 264 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.6.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

Editor's note: The targeted max Tx power configuration shall be within the scope of basket WI.

### 5.6.3	MSD scenario studies

Table 5.6.3-1 and 5.6.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n41-n66.

Table 5.6.3-1: MSD scenarios for CA_n41-n66 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Cross band leakage | n41Y UL | n66 DL | Yes |

Table 5.6.3-2: MSD scenarios for CA_n41-n66 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Cross band leakage | n41Y UL | n66 DL | Yes |

### 5.6.4	REFSENS requirements

#### 5.6.4.1	REFSENS requirements for total power class 2

The PC2 MSD due to cross band isolation can be re-used from PC2 with 2Tx and is therefore not shown here.

#### 5.6.4.2	REFSENS requirements for total power class 1.5

The PC1.5 MSD due to cross band isolation can be re-used from PC1.5 with 2Tx and is therefore not shown here.

<< End of Text Proposal >